ELSKY IPC100U Industrial Computer Specifications

―――――――――――――――――――――――――――――――――――――――――――――――――――――――

PC size diagram

―――――――――――――――――――――――――――――――――――――――――――――――――――――――

Front Interface Diagram

―――――――――――――――――――――――――――――――――――――――――――――――――――――――

Rear interface diagram with network port in the middle and small audio port.

―――――――――――――――――――――――――――――――――――――――――――――――――――――――

Rear interface side view of network port + large audio port diagram

―――――――――――――――――――――――――――――――――――――――――――――――――――――――

Rear interface diagram DP optional

―――――――――――――――――――――――――――――――――――――――――――――――――――――――

Rear I/O panel: Dual HDMI® optional

―――――――――――――――――――――――――――――――――――――――――――――――――――――――

Rear interface diagram, dual network optional

―――――――――――――――――――――――――――――――――――――――――――――――――――――――

Rear Interface Diagram 2 * COM Optional

―――――――――――――――――――――――――――――――――――――――――――――――――――――――

―――――――――――――――――――――――――――――――――――――――――――――――――――――――

PC size diagram

―――――――――――――――――――――――――――――――――――――――――――――――――――――――

The pictures are for reference only. Please refer to the actual product.

※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※

| HDMI®​ | VGA |  | LAN |  | USB 2.0 | USB 3.1 | COM | SSD | Memory | Power |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 4K 30Hz | 2K 60Hz |  | 1 |  | 6 | 2 | 2/6 | 1*M.2 1*SATA Optional 1*MSATA | 2*NB-DDR4 MAX 64G | 12V/19V |
|  |  |  | 2 |  | 4 |  |  |  |  |  |
| PC model |  | IPC100U-Feiteng ET2000 |  |  |  |  |  |  |  |  |
| PC size |  | Dimensions with handle: 430mm(L)*301mm(W)*44mm(H) Dimensions with handle: 480mm(L)*301mm(W)*44mm(H) |  |  |  |  |  |  |  |  |
| Cooling system |  | With ventilation holes and silent fan cooling |  |  |  |  |  |  |  |  |
| Color |  | black |  |  |  |  |  |  |  |  |
| Compatible motherboard models |  | ET2000 |  |  |  |  |  |  |  |  |
| Motherboard |  | CPU |  | Domestic Phytium Tengrui D2000, octa-core, 2.3GHz; Domestic Phytium Tengrui FT2000 Quad-core 2.6GHz; |  |  |  |  |  |  |
|  |  | graphics card |  | Supports onboard X100 4G/8G discrete graphics card, supporting single display, dual display duplication, and dual display expansion. |  |  |  |  |  |  |
|  |  | Memory |  | 2*NB-DDR4 memory supports 2133/2400/2666MHz, with a maximum capacity of 64GB. |  |  |  |  |  |  |
| Front panel interface |  | POWER |  | 1* Switch button |  |  |  |  |  |  |
|  |  | RESET |  | 1* Restart Switch |  |  |  |  |  |  |
|  |  | led |  | 1 power indicator, 1 hard drive indicator |  |  |  |  |  |  |
|  |  | USB |  | 2 USB 2.0 ports |  |  |  |  |  |  |
| Rear panel interface |  | Power interface |  | 1 power interface, built-in DC power adapter, supports DC 12V/19V, recommended power supply is 12V 7A or 19V 5A or higher. |  |  |  |  |  |  |
|  |  | Display Interface |  | 1 VGA interface (1920*1080@60Hz) 1*HDMI® 1.4 port (4096/3840*2160@30HZ) |  |  |  |  |  |  |
|  |  | LAN |  | Single gigabit network card: YT6801 or Realtek 8111H are optional; Dual Gigabit Ethernet Adapters: YT6801 + YT8521SC or Realtek 8111H + YT8521SC (optional) Supports Wake-on-LAN functionality; |  |  |  |  |  |  |
|  |  | USB |  | 2 USB 3.1 Gen1 ports 4 USB 2.0 ports (2 USB 2.0 ports for dual-network PC) |  |  |  |  |  |  |
|  |  | COM |  | 6 COM interfaces, supporting RS232 by default; COM1 can optionally support RS232/RS422/RS485. |  |  |  |  |  |  |
|  |  | audio |  | 1*LINE_OUT interface, audio output 1*MIC_IN interface, microphone input |  |  |  |  |  |  |
|  |  | WiFi |  | 2 WiFi antenna interfaces |  |  |  |  |  |  |
| Storage |  | M.2 SSD |  | 1 M.2 Key-M slot, supporting M.2 2280 NVMe PCIe 3.0 x4 / SATA dual-protocol adaptive standard SSDs. |  |  |  |  |  |  |
|  |  | SATA |  | One SATA 3.0 hard drive interface, supporting 2.5-inch SSDs/HDDs or 3.5-inch HDDs. |  |  |  |  |  |  |
|  |  | Hard drive bays in the chassis |  | Two hard drive bays (one 2.5-inch SSD/HDD bay and one 3.5-inch HDD bay; this only indicates the number of hard drives the case can accommodate). |  |  |  |  |  |  |
|  |  | mSATA |  | Optional 1 mSATA slot, defaults to WiFi module, can be modified to support mSATA drives. |  |  |  |  |  |  |
| Extend |  | Mini-PCIe |  | One Mini-PCIe slot, supporting PCIe 3.0 x1 & USB signals, and WiFi/Bluetooth/4G modules; can be modified to support mSATA drives. |  |  |  |  |  |  |
| Operating environment |  | Operating temperature |  | -10℃ ~ 60℃ |  |  |  |  |  |  |
|  |  | Storage temperature |  | -20℃ ~ 70℃ |  |  |  |  |  |  |
|  |  | relative humidity |  | 5%-95% relative humidity, non-condensing |  |  |  |  |  |  |
|  |  | Working vibration |  | 0.5grms/5-500HZ/random/operating |  |  |  |  |  |  |
| BIOS |  | Supports power-on upon startup, scheduled power-on, and remote power-on/off. |  |  |  |  |  |  |  |  |
| Watch Dog |  | Watchdog programming with support for hardware reset function (256 levels, 0~255 seconds). |  |  |  |  |  |  |  |  |
| operating system |  | Supports domestic operating systems such as Kylin-Desktop-V10-SP1-Release-2503-ARM64 and Linux (must support X100 version). |  |  |  |  |  |  |  |  |

※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※

※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※

| HDMI®​ | VGA |  | LAN |  | USB 2.0 | USB 3.0 | COM | SSD | Memory | Power |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2K 30Hz | 1024*768@60Hz |  | 1 |  | 6 | 2 | 2/6 | 1*M.2 1*SATA | 1*NB-DDR4 MAX 32G | 12V/19V |
|  |  |  | 2 |  | 4 |  |  |  |  |  |
| PC model |  | IPC100U-Feiteng Q T2000 |  |  |  |  |  |  |  |  |
| PC size |  | Dimensions with handle: 430mm(L)*301mm(W)*44mm(H) Dimensions with handle: 480mm(L)*301mm(W)*44mm(H) |  |  |  |  |  |  |  |  |
| Cooling system |  | With ventilation holes and silent fan cooling |  |  |  |  |  |  |  |  |
| Color |  | black |  |  |  |  |  |  |  |  |
| Compatible motherboard models |  | QT2000 |  |  |  |  |  |  |  |  |
| Motherboard |  | CPU |  | Domestic Phytium Tenlong E2000Q Quad-core 1.5GHz; |  |  |  |  |  |  |
|  |  | graphics card |  | Supports single display, dual display duplication, and dual display expansion. |  |  |  |  |  |  |
|  |  | Memory |  | 1*NB-DDR4 supports 2133/2400/2666MHz memory, with a maximum support of 32GB. |  |  |  |  |  |  |
| Front panel interface |  | POWER |  | 1* Switch button |  |  |  |  |  |  |
|  |  | RESET |  | 1* Restart Switch |  |  |  |  |  |  |
|  |  | led |  | 1 power indicator, 1 hard drive indicator |  |  |  |  |  |  |
|  |  | USB |  | 2 USB 2.0 ports |  |  |  |  |  |  |
| Rear panel interface |  | Power interface |  | 1 power interface, built-in DC power adapter, supports DC 12V/19V, recommended power supply is 12V 4A or 19V 3A or higher. |  |  |  |  |  |  |
|  |  | Display Interface |  | 1 VGA interface (1024*768@60Hz) 1*HDMI® 1.4 port (1920*1080@30Hz) |  |  |  |  |  |  |
|  |  | LAN |  | Single Gigabit network card: Supports Yutai Micro YT8521SC; Dual Gigabit Ethernet Adapters: Supports YT8521SC + YT8521SC; Supports Wake-on-LAN functionality; |  |  |  |  |  |  |
|  |  | USB |  | 2 USB 3.0 ports 4 USB 2.0 ports (2 USB 2.0 ports for dual-network PC) |  |  |  |  |  |  |
|  |  | COM |  | 6 COM interfaces, supporting RS232 by default; COM1 can optionally support RS232/RS422/RS485. |  |  |  |  |  |  |
|  |  | audio |  | 1*LINE_OUT interface, audio output 1*MIC_IN interface, microphone input |  |  |  |  |  |  |
|  |  | WiFi |  | 2 WiFi antenna interfaces |  |  |  |  |  |  |
| Storage |  | M.2 SSD |  | 1 M.2 Key-M slot, supporting M.2 2280 NVMe PCIe 3.0 x4 / SATA dual-protocol adaptive standard SSDs. |  |  |  |  |  |  |
|  |  | SATA |  | One SATA 3.0 hard drive interface, supporting 2.5-inch SSDs/HDDs or 3.5-inch HDDs. |  |  |  |  |  |  |
|  |  | Hard drive bays in the chassis |  | Two hard drive bays (one 2.5-inch SSD/HDD bay and one 3.5-inch HDD bay; this only indicates the number of hard drives the case can accommodate). |  |  |  |  |  |  |
| Extend |  | M2_WIFI |  | One M.2 Wi-Fi interface, supporting Key-E interface M.2 2230 PCIe 3.0 x1 & USB signals, and supporting Wi-Fi and Bluetooth modules. |  |  |  |  |  |  |
| Operating environment |  | Operating temperature |  | -10℃ ~ 60℃ |  |  |  |  |  |  |
|  |  | Storage temperature |  | -20℃ ~ 70℃ |  |  |  |  |  |  |
|  |  | relative humidity |  | 5%-95% relative humidity, non-condensing |  |  |  |  |  |  |
|  |  | Working vibration |  | 0.5grms/5-500HZ/random/operating |  |  |  |  |  |  |
| BIOS |  | Supports power-on upon startup, scheduled power-on, and remote power-on/off. |  |  |  |  |  |  |  |  |
| Watch Dog |  | Watchdog programming with support for hardware reset function (256 levels, 0~255 seconds). |  |  |  |  |  |  |  |  |
| Operating system |  | Supports Kylin Embedded System (V10-SP1-General-Release-Base-2309-ARM64), Tongxin and other domestic systems. |  |  |  |  |  |  |  |  |

※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※

※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※

| HDMI®​ | VGA |  | LAN |  | USB 2.0 | USB 3.1 | COM | SSD | Memory | Power |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 4K 30Hz | 2K 60Hz |  | 1 |  | 4 | 4 | 2/6 | 1*M.2 1*SATA Optional 1*MSATA | 2*NB-DDR4 MAX 64G | 12V/19V |
|  |  |  | 2 |  | 4 | 2 |  |  |  |  |
| PC model |  | IPC100U-Feiteng E T 3 000 |  |  |  |  |  |  |  |  |
| PC size |  | Dimensions with handle: 430mm(L)*301mm(W)*44mm(H) Dimensions with handle: 480mm(L)*301mm(W)*44mm(H) |  |  |  |  |  |  |  |  |
| Cooling system |  | With ventilation holes and silent fan cooling |  |  |  |  |  |  |  |  |
| color |  | black |  |  |  |  |  |  |  |  |
| Compatible motherboard models |  | ET3000 |  |  |  |  |  |  |  |  |
| motherboard |  | CPU |  | Domestic Phytium Tengrui D3000, octa-core 2.5GHz; |  |  |  |  |  |  |
|  |  | graphics card |  | Supports onboard X100 4G/8G dedicated graphics card, supporting single display, dual display duplication, and dual display expansion. |  |  |  |  |  |  |
|  |  | Memory |  | 2*NB-DDR4 memory modules support 2133/2400/2666MHz, with a maximum capacity of 64GB. |  |  |  |  |  |  |
| Front panel interface |  | POWER |  | 1* Switch button |  |  |  |  |  |  |
|  |  | RESET |  | 1* Restart Switch |  |  |  |  |  |  |
|  |  | led |  | 1 power indicator, 1 hard drive indicator |  |  |  |  |  |  |
|  |  | USB |  | 2 USB 2.0 ports |  |  |  |  |  |  |
| Rear panel interface |  | Power interface |  | 1 power interface, built-in DC power adapter, supports DC 12V/19V, recommended power supply is 12V 7A or 19V 5A or higher. |  |  |  |  |  |  |
|  |  | Display Interface |  | 1 VGA interface (1920*1080@60Hz) 1*HDMI® 1.4 port (4096/3840*2160@30Hz) |  |  |  |  |  |  |
|  |  | LAN |  | Single gigabit network card: YT6801 or Realtek 8111H are optional; Dual Gigabit Ethernet Adapters: YT6801 + YT8521SC or Realtek 8111H + YT8521SC (optional) Supports Wake-on-LAN functionality; |  |  |  |  |  |  |
|  |  | USB |  | 4 USB 3.1 Gen1 ports (2 USB 3.1 Gen1 ports for dual-network PCs) 2 USB 2.0 ports |  |  |  |  |  |  |
|  |  | COM |  | 6 COM interfaces, supporting RS232 by default; COM1 can optionally support RS232/RS422/RS485. |  |  |  |  |  |  |
|  |  | audio |  | 1*LINE_OUT interface, audio output 1*MIC_IN interface, microphone input |  |  |  |  |  |  |
|  |  | WiFi |  | 2 WiFi antenna interfaces |  |  |  |  |  |  |
| storage |  | M.2 SSD |  | 1 M.2 Key-M slot, supporting M.2 2280 NVME PCIe 4.0 x4 / SATA dual-protocol adaptive standard SSD; Optional M.2 2242 hard drive; |  |  |  |  |  |  |
|  |  | SATA |  | One SATA 3.0 hard drive interface, supporting 2.5-inch SSD/HDD or 3.5-inch HDD. |  |  |  |  |  |  |
|  |  | Hard drive bays in the chassis |  | Two hard drive bays (one 2.5-inch SSD/HDD bay and one 3.5-inch HDD bay; this only indicates the number of hard drives the case can accommodate). |  |  |  |  |  |  |
|  |  | mSATA |  | Optional 1 mSATA slot, defaults to WiFi module, can be modified to support mSATA drives. |  |  |  |  |  |  |
| Extend |  | Mini-PCIe |  | One Mini-PCIe slot, supporting PCIe 3.0 x1 & USB signals, and supporting WIFI, Bluetooth/4G modules; can be modified to support mSATA drives. |  |  |  |  |  |  |
| Operating environment |  | Operating temperature |  | -10℃ ~ 60℃ |  |  |  |  |  |  |
|  |  | Storage temperature |  | -20℃ ~ 70℃ |  |  |  |  |  |  |
|  |  | relative humidity |  | 5%-95% relative humidity, non-condensing |  |  |  |  |  |  |
|  |  | Working vibration |  | 0.5grms/5-500HZ/random/operating |  |  |  |  |  |  |
| BIOS |  | Supports power-on upon startup, scheduled power-on, and remote power-on/off. |  |  |  |  |  |  |  |  |
| Watch Dog |  | Watchdog programming with support for hardware reset function (256 levels, 0~255 seconds). |  |  |  |  |  |  |  |  |
| operating system |  | Supports domestic operating systems such as Kylin-Desktop-V10-SP1-Release-2503-ARM64 and Linux (must support X100 version). |  |  |  |  |  |  |  |  |

※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※

※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※

| HDMI®​ | VGA |  | LAN |  | USB 2.0 | USB 3.0 | COM | SSD | Memory | Power |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 4K 30Hz | 2K 60Hz |  | 1 |  | 6 | 2 | 2/6 | 1*MSATA 1*SATA | 2*NB-DDR4 MAX 64G | 12V/19V |
|  |  |  | 2 |  | 4 |  |  |  |  |  |
| PC model |  | IPC100U-Zhaoxin KC6000 |  |  |  |  |  |  |  |  |
| PC size |  | Dimensions with handle: 430mm(L)*301mm(W)*44mm(H) Dimensions with handle: 480mm(L)*301mm(W)*44mm(H) |  |  |  |  |  |  |  |  |
| Cooling system |  | With ventilation holes and silent fan cooling |  |  |  |  |  |  |  |  |
| color |  | black |  |  |  |  |  |  |  |  |
| Compatible motherboard models |  | KC6000 |  |  |  |  |  |  |  |  |
| motherboard |  | CPU |  | Supports the entire series of domestic Zhaoxin KX6000 16NM surface-mount CPUs. KX 6640MA Quad-core 2.2GHz, Turbo Boost 2.6GHz; KX 6640A Quad-core 2.6GHz; KX U6580 Octa-core 2.5GHz; KX U6780A Octa-core 2.7GHz; KX U6780 Octa-core 2.7GHz |  |  |  |  |  |  |
|  |  | graphics card |  | Integrated ZX C960 DX11 super graphics card, supporting single display, dual display duplication, and dual display expansion. |  |  |  |  |  |  |
|  |  | Memory |  | 2*NB-DDR4 memory supports 2133/2400/2666MHz, with a maximum capacity of 64GB. |  |  |  |  |  |  |
| Front panel interface |  | POWER |  | 1* Switch button |  |  |  |  |  |  |
|  |  | RESET |  | 1* Restart Switch |  |  |  |  |  |  |
|  |  | led |  | 1 power indicator, 1 hard drive indicator |  |  |  |  |  |  |
|  |  | USB |  | 2 USB 2.0 ports |  |  |  |  |  |  |
| Rear panel interface |  | Power interface |  | 1 power interface, built-in DC power adapter, supports DC 12V/19V, recommended power supply is 12V 10A or 19V 5A or higher. |  |  |  |  |  |  |
|  |  | Display Interface |  | 1 VGA interface (1920*1080@60Hz) 1*HDMI® 1.4 port (4096/3840*2160@30HZ) |  |  |  |  |  |  |
|  |  | LAN |  | Single gigabit network card: Realtek 8111H or Yutaiwei YT6801 are optional; Dual Gigabit network cards: Realtek 8111H + Realtek 8111H or Yutai Micro YT6801 + Yutai Micro YT6801 optional; Supports Wake-on-LAN and PXE network functionality; |  |  |  |  |  |  |
|  |  | USB |  | 2 USB 3.0 ports 4 USB 2.0 ports (2 USB 2.0 ports for dual-network PC) |  |  |  |  |  |  |
|  |  | COM |  | 6 COM interfaces, supporting RS232 by default; COM1 can optionally support RS232/RS422/RS485. |  |  |  |  |  |  |
|  |  | audio |  | 1*LINE_OUT interface, audio output 1*MIC_IN interface, microphone input |  |  |  |  |  |  |
|  |  | WiFi |  | 2 WiFi antenna interfaces |  |  |  |  |  |  |
| storage |  | mSATA |  | 1 mSATA slot, supports mSATA drives |  |  |  |  |  |  |
|  |  | SATA |  | One SATA 3.0 hard drive interface, supporting 2.5-inch SSD/HDD or 3.5-inch HDD. |  |  |  |  |  |  |
|  |  | Hard drive bays in the chassis |  | Two hard drive bays (one 2.5-inch SSD/HDD bay and one 3.5-inch HDD bay; this only indicates the number of hard drives the case can accommodate). |  |  |  |  |  |  |
| Extend |  | Mini-PCIe |  | One Mini-PCIe slot, supporting PCIe 3.0 x1 signal and WiFi module; can be modified to support 4G/Bluetooth module. |  |  |  |  |  |  |
| Operating environment |  | Operating temperature |  | -10℃ ~ 60℃ |  |  |  |  |  |  |
|  |  | Storage temperature |  | -20℃ ~ 70℃ |  |  |  |  |  |  |
|  |  | relative humidity |  | 5%-95% relative humidity, non-condensing |  |  |  |  |  |  |
|  |  | Working vibration |  | 0.5grms/5-500HZ/random/operating |  |  |  |  |  |  |
| BIOS |  | Supports power-on upon startup, scheduled power-on, and remote power-on/off. |  |  |  |  |  |  |  |  |
| Watch Dog |  | Watchdog programming with support for hardware reset function (256 levels, 0~255 seconds). |  |  |  |  |  |  |  |  |
| operating system |  | Windows 10, Windows 7, Linux, etc. |  |  |  |  |  |  |  |  |

※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※

※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※

| HDMI1®​ | VGA/ HDMI2® |  | LAN |  | USB 2.0 | USB 3.0 | COM | SSD | Memory | Power |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 4K 60Hz | VGA: 2K 60Hz HDMI2® : 4K 60Hz |  | 1 |  | 6 | 2 | 2/6 | 1*M.2 1*SATA | 2*NB-DDR4 MAX 64G | 12V/19V |
|  |  |  | 2 |  | 4 |  |  |  |  |  |
| PC model |  | IPC100U-Zhaoxin KC 7 000 |  |  |  |  |  |  |  |  |
| PC size |  | Dimensions with handle: 430mm(L)*301mm(W)*44mm(H) Dimensions with handle: 480mm(L)*301mm(W)*44mm(H) |  |  |  |  |  |  |  |  |
| Cooling system |  | With ventilation holes and silent fan cooling |  |  |  |  |  |  |  |  |
| color |  | black |  |  |  |  |  |  |  |  |
| Compatible motherboard models |  | KC7000 |  |  |  |  |  |  |  |  |
| motherboard |  | CPU |  | Supports the entire series of domestic Zhaoxin KX7000 surface mount CPUs Zhaoxin KX 7000 Octa-core 3.5GHz; |  |  |  |  |  |  |
|  |  | graphics card |  | Integrated ZX C1199 graphics card, supporting single display, dual display duplication, and dual display expansion. |  |  |  |  |  |  |
|  |  | Memory |  | 2*NB-DDR4 memory modules support 2133/2400/2666/3200MHz, with a maximum capacity of 64GB. |  |  |  |  |  |  |
| Front panel interface |  | POWER |  | 1* Switch button |  |  |  |  |  |  |
|  |  | RESET |  | 1* Restart Switch |  |  |  |  |  |  |
|  |  | led |  | 1 power indicator, 1 hard drive indicator |  |  |  |  |  |  |
|  |  | USB |  | 2 USB 2.0 ports |  |  |  |  |  |  |
| Rear panel interface |  | Power interface |  | 1 power interface, built-in DC power adapter, supports DC 12V/19V, recommended power supply is 12V 10A or 19V 5A or higher. |  |  |  |  |  |  |
|  |  | Display Interface |  | 1 VGA interface (1920*1080@60Hz) 1*HDMI1® 2.0 port (4096/3840*2160@60Hz) Optional 1*HDMI2® 2.0 interface (4096/3840*2160@60Hz) |  |  |  |  |  |  |
|  |  | LAN |  | Single gigabit network card: YT6801 or Realtek 8111H are optional; Dual Gigabit network cards: YT6801 + YT6801 or Realtek 8111H + Realtek 8111H optional; Supports Wake-on-LAN and PXE network functionality; |  |  |  |  |  |  |
|  |  | USB |  | 2 USB 3.0 ports 4 USB 2.0 ports (2 USB 2.0 ports for dual-network PC) |  |  |  |  |  |  |
|  |  | COM |  | 6 COM interfaces, with default support for RS232; |  |  |  |  |  |  |
|  |  | audio |  | 1*LINE_OUT interface, audio output 1*MIC_IN interface, microphone input |  |  |  |  |  |  |
|  |  | WiFi |  | 2 WiFi antenna interfaces |  |  |  |  |  |  |
| storage |  | M.2 SSD |  | 1 M.2 Key-M slot, supporting M.2 2280 NVMe PCIe 4.0 x2 / SATA dual-protocol adaptive standard SSDs. |  |  |  |  |  |  |
|  |  | SATA |  | One SATA 3.0 hard drive interface, supporting 2.5-inch SSDs/HDDs or 3.5-inch HDDs. |  |  |  |  |  |  |
|  |  | Hard drive bays in the chassis |  | Two hard drive bays (one 2.5-inch SSD/HDD bay and one 3.5-inch HDD bay; this only indicates the number of hard drives the case can accommodate). |  |  |  |  |  |  |
| Extend |  | M2_WIFI |  | One M.2 Wi-Fi interface, supporting Key-E interface M.2 2230 PCIe 4.0 x1 & USB, supporting Wi-Fi and Bluetooth modules. |  |  |  |  |  |  |
|  |  | CN1 |  | One CN1 interface supports Key-B interface M.2 5G (3052/3042) modules, and the hardware can be modified to support M.2 4G (3042) modules. |  |  |  |  |  |  |
| Operating environment |  | Operating temperature |  | -10℃ ~ 60℃ |  |  |  |  |  |  |
|  |  | Storage temperature |  | -20℃ ~ 70℃ |  |  |  |  |  |  |
|  |  | relative humidity |  | 5%-95% relative humidity, non-condensing |  |  |  |  |  |  |
|  |  | Working vibration |  | 0.5grms/5-500HZ/random/operating |  |  |  |  |  |  |
| BIOS |  | Supports power-on upon startup, scheduled power-on, and remote power-on/off. |  |  |  |  |  |  |  |  |
| Watch Dog |  | Watchdog programming with support for hardware reset function (256 levels, 0~255 seconds). |  |  |  |  |  |  |  |  |
| operating system |  | Windows 10, Windows 7, Linux, etc. |  |  |  |  |  |  |  |  |

※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※

※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※

| HDMI® | VGA |  | LAN |  | USB2.0 | USB3.0 | COM | SSD | Memory | Power |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2K 60Hz | 2K 60Hz |  | 1 |  | 7 | 1 | 2/6 | 1*MSATA 1*SATA | 1*NB-DDR3 MAX 8G | 12V |
|  |  |  | 2 |  | 5 |  |  |  |  |  |
| PC model |  | IPC100U-J1900 series |  |  |  |  |  |  |  |  |
| PC size |  | Dimensions with handle: 430mm(L)*301mm(W)*44mm(H) Dimensions with handle: 480mm(L)*301mm(W)*44mm(H) |  |  |  |  |  |  |  |  |
| Cooling system |  | With ventilation holes, fanless or with silent fan cooling |  |  |  |  |  |  |  |  |
| color |  | black |  |  |  |  |  |  |  |  |
| Compatible motherboard models |  | M218F, M219F, M218H, M219H, M284F, M294F, M284H, M294H, QM1800E, QM1800L, QM1900E, QM1900L |  |  |  |  |  |  |  |  |
| motherboard |  | CPU |  | Supports Intel® Bay Trail J and N series CPUs; Intel Celeron J1800 Dual-core 2.41GHz, Turbo Boost 2.58GHz; Intel Celeron J1900 Quad-core 1.99GHz, Turbo Boost 2.4GHz; Intel Pentium J2900 Quad-core 2.41GHz, Turbo Boost 2.66GHz; Intel Celeron N2840 Dual-core 2.16GHz, Turbo Boost 2.58GHz; Intel Celeron N2930 Quad-core 1.83GHz, Turbo Boost 2.16GHz; Intel Celeron N2940 Quad-core 1.83GHz, Turbo Boost 2.25GHz; |  |  |  |  |  |  |
|  |  | graphics card |  | Intel® HD Graphics graphics controller, supporting single display, dual display duplication, and dual display expansion. |  |  |  |  |  |  |
|  |  | Memory |  | 1*NB-DDR3 memory, supports 1.35V, supports 1600MHz memory, maximum support 8GB |  |  |  |  |  |  |
| Front panel interface |  | POWER |  | 1* Switch button |  |  |  |  |  |  |
|  |  | RESET |  | 1* Restart Switch |  |  |  |  |  |  |
|  |  | led |  | 1 power indicator, 1 hard drive indicator |  |  |  |  |  |  |
|  |  | USB |  | 2 USB 2.0 ports |  |  |  |  |  |  |
| Rear panel interface |  | Power interface |  | 1 power interface, built-in DC power adapter, supports DC 12V, 12V 4A or higher power supply recommended. |  |  |  |  |  |  |
|  |  | Display Interface |  | 1*HDMI® 1.4 port (1920*1080@60Hz) 1 VGA interface (1920*1080@60Hz) |  |  |  |  |  |  |
|  |  | LAN |  | Single gigabit network card; default support for Realtek 8111H; Dual Gigabit Ethernet adapters; supports Realtek 8111H + Realtek 8111H; Supports Wake-on-LAN and PXE functionality. |  |  |  |  |  |  |
|  |  | USB |  | 1 USB 3.0 port (the port under the blue group is the USB 3.0 port) 5 USB 2.0 ports (3 USB 2.0 ports for dual-network PCs) |  |  |  |  |  |  |
|  |  | COM |  | 6 COM interfaces, default RS232; optional support for 2 RS232/RS422/RS485, or 1 RS232/RS485. |  |  |  |  |  |  |
|  |  | audio |  | 1*LINE_OUT interface, audio output 1*MIC_IN interface, microphone input |  |  |  |  |  |  |
|  |  | WiFi |  | 2 WiFi antenna interfaces |  |  |  |  |  |  |
| storage |  | mSATA |  | 1 mSATA slot, supports mSATA drives |  |  |  |  |  |  |
|  |  | SATA |  | One SATA 3.0 hard drive interface, supporting 2.5-inch SSDs/HDDs or 3.5-inch HDDs. |  |  |  |  |  |  |
|  |  | Hard drive bays in the chassis |  | Two hard drive bays (one 2.5-inch SSD/HDD bay and one 3.5-inch HDD bay; this only indicates the number of hard drives the case can accommodate). |  |  |  |  |  |  |
| Extend |  | Mini-PCIe |  | 1 Mini-PCIe slot, supports PCIe 2.0 x1 & USB signal, supports WiFi, Bluetooth/4G modules. |  |  |  |  |  |  |
| environment |  | Operating temperature |  | -10℃ ~ 60℃ |  |  |  |  |  |  |
|  |  | Storage temperature |  | -20℃ ~ 70℃ |  |  |  |  |  |  |
|  |  | relative humidity |  | 5%-95% relative humidity, non-condensing |  |  |  |  |  |  |
|  |  | Working vibration |  | 0.5grms/5-500HZ/random/operating |  |  |  |  |  |  |
| BIOS |  | AMI BIOS supports power-on, scheduled power-on, remote power-on, and intelligent device identification. |  |  |  |  |  |  |  |  |
| Watch Dog |  | Watchdog programming with support for hardware reset function (256 levels, 0~255 seconds). |  |  |  |  |  |  |  |  |
| operating system |  | Windows 10, Windows 8, Windows 7, Linux, etc. |  |  |  |  |  |  |  |  |

※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※

※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※

| HDMI®​ | VGA |  | LAN |  | USB 2.0 | USB 3.0 | COM | SSD | Memory | Power |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2K 60Hz | 2K 60Hz |  | 1 |  | 6 | 2 | 2/6 | 1*MSATA 1*SATA | 1*NB-DDR3 MAX 8G | 12V |
|  |  |  | 2 |  | 4 |  |  |  |  |  |
| PC model |  | IPC100U-M316 series |  |  |  |  |  |  |  |  |
| PC size |  | Dimensions with handle: 430mm(L)*301mm(W)*44mm(H) Dimensions with handle: 480mm(L)*301mm(W)*44mm(H) |  |  |  |  |  |  |  |  |
| Cooling system |  | With ventilation holes, fanless or with silent fan cooling |  |  |  |  |  |  |  |  |
| color |  | black |  |  |  |  |  |  |  |  |
| Compatible motherboard models |  | M316F |  |  |  |  |  |  |  |  |
| motherboard |  | CPU |  | Intel Celeron N3150 Quad-core 1.6GHz, Turbo Boost 2.08GHz; Intel Celeron N3160 Quad-core 1.6GHz, Turbo Boost 2.24GHz; Intel Celeron N3700 Quad-core 1.6GHz, Turbo Boost 2.40GHz; |  |  |  |  |  |  |
|  |  | graphics card |  | Intel® HD Graphics graphics controller, supporting single display, dual display duplication, and dual display expansion. |  |  |  |  |  |  |
|  |  | Memory |  | 1*NB-DDR3 memory, supports 1.35V, supports 1600MHz memory, maximum support 8GB |  |  |  |  |  |  |
| Front panel interface |  | POWER |  | 1* Switch button |  |  |  |  |  |  |
|  |  | RESET |  | 1* Restart Switch |  |  |  |  |  |  |
|  |  | led |  | 1 power indicator, 1 hard drive indicator |  |  |  |  |  |  |
|  |  | USB |  | 2 USB 2.0 ports |  |  |  |  |  |  |
| Rear panel interface |  | Power interface |  | 1 power interface, built-in DC power adapter, supports DC 12V, recommended 12V 4A or higher power supply. |  |  |  |  |  |  |
|  |  | Display Interface |  | 1*HDMI® 1.4 port (1920*1080@60Hz) 1 VGA interface (1920*1080@60Hz) |  |  |  |  |  |  |
|  |  | LAN |  | Single gigabit network card; default support for Realtek 8111H; Dual Gigabit Ethernet adapters; supports Realtek 8111H + Realtek 8111H; Supports Wake-on-LAN and PXE functionality. |  |  |  |  |  |  |
|  |  | USB |  | 2 USB 3.0 ports 4 USB 2.0 ports (2 USB 2.0 ports for dual-network PC) |  |  |  |  |  |  |
|  |  | COM |  | 6 COM interfaces, default RS232; COM1 can optionally support RS232/RS422/RS485. |  |  |  |  |  |  |
|  |  | audio |  | 1*LINE_OUT interface, audio output 1*MIC_IN interface, microphone input |  |  |  |  |  |  |
|  |  | WiFi |  | 2 WiFi antenna interfaces |  |  |  |  |  |  |
| storage |  | mSATA |  | 1 mSATA slot, supports mSATA drives |  |  |  |  |  |  |
|  |  | SATA |  | One SATA 3.0 hard drive interface, supporting 2.5-inch SSD/HDD or 3.5-inch HDD. |  |  |  |  |  |  |
|  |  | Hard drive bays in the chassis |  | Two hard drive bays (one 2.5-inch SSD/HDD bay and one 3.5-inch HDD bay; this only indicates the number of hard drives the case can accommodate). |  |  |  |  |  |  |
| Extend |  | Mini-PCIe |  | 1 Mini-PCIe slot, supporting 4G/ WiFi/Bluetooth modules |  |  |  |  |  |  |
| environment |  | Operating temperature |  | -10℃ ~ 60℃ |  |  |  |  |  |  |
|  |  | Storage temperature |  | -20℃ ~ 70℃ |  |  |  |  |  |  |
|  |  | relative humidity |  | 5%-95% relative humidity, non-condensing |  |  |  |  |  |  |
|  |  | Working vibration |  | 0.5grms/5-500HZ/random/operating |  |  |  |  |  |  |
| BIOS |  | AMI BIOS supports power-on, scheduled power-on, remote power-on, and intelligent device identification. |  |  |  |  |  |  |  |  |
| Watch Dog |  | Watchdog programming with support for hardware reset function (256 levels, 0~255 seconds). |  |  |  |  |  |  |  |  |
| operating system |  | Windows 10, Windows 8, Windows 7, Linux, etc. |  |  |  |  |  |  |  |  |

※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※

※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※

| HDMI®​ | VGA |  | LAN |  | USB 2.0 | USB 3.0 | COM | SSD | Memory | Power |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 4K 30Hz | 2K 60Hz |  | 1 |  | 6 | 2 | 2/6 | 1*M.2 1*MSATA 1*SATA | 1*NB-DDR3 MAX 8G | 12V |
|  |  |  | 1 |  | 4 |  |  |  |  |  |
| PC model |  | IPC100U-J3355 series |  |  |  |  |  |  |  |  |
| PC size |  | Dimensions with handle: 430mm(L)*301mm(W)*44mm(H) Dimensions with handle: 480mm(L)*301mm(W)*44mm(H) |  |  |  |  |  |  |  |  |
| Cooling system |  | With ventilation holes, fanless or with silent fan cooling |  |  |  |  |  |  |  |  |
| color |  | black |  |  |  |  |  |  |  |  |
| Compatible motherboard models |  | M335F |  |  |  |  |  |  |  |  |
| motherboard |  | CPU |  | Intel Celeron J3355, dual-core 2.0GHz, turbo boost 2.5GHz; Intel Celeron J3455 Quad-core 1.5GHz, Turbo Boost 2.3GHz; Intel Pentium N4200 Quad-core 1.1GHz, Turbo Boost 2.5GHz; |  |  |  |  |  |  |
|  |  | graphics card |  | Intel® HD Graphics 500/505 graphics controller, supporting single display, dual display duplication, and dual display expansion. |  |  |  |  |  |  |
|  |  | Memory |  | 1*NB-DDR3 memory, supports 1.35V, supports 1600MHz memory, maximum support 8GB |  |  |  |  |  |  |
| Front panel interface |  | POWER |  | 1* Switch button |  |  |  |  |  |  |
|  |  | RESET |  | 1* Restart Switch |  |  |  |  |  |  |
|  |  | led |  | 1 power indicator, 1 hard drive indicator |  |  |  |  |  |  |
|  |  | USB |  | 2 USB 2.0 ports |  |  |  |  |  |  |
| Rear panel interface |  | Power interface |  | 1 power interface, built-in DC power adapter, supports DC 12V, recommended 12V 4A or higher power supply. |  |  |  |  |  |  |
|  |  | Display Interface |  | 1*HDMI® 1.4 port (4096/3840*2160@30Hz) 1 VGA interface (1920*1080@60Hz) |  |  |  |  |  |  |
|  |  | LAN |  | Single gigabit network card; default support for Realtek 8111H; Dual Gigabit Ethernet adapters; supports Realtek 8111H + Realtek 8111H; Supports Wake-on-LAN and PXE functionality. |  |  |  |  |  |  |
|  |  | USB |  | 2 USB 3.0 ports 4 USB 2.0 ports (2 USB 2.0 ports for dual-network PC) |  |  |  |  |  |  |
|  |  | COM |  | 6 COM interfaces, default RS232; optional support for 2 RS232/RS422/RS485 |  |  |  |  |  |  |
|  |  | audio |  | 1*LINE_OUT interface, audio output 1*MIC_IN interface, microphone input |  |  |  |  |  |  |
|  |  | WiFi |  | 2 WiFi antenna interfaces |  |  |  |  |  |  |
| storage |  | M.2 SSD |  | 1 M.2 Key-M slot, supporting M.2 2242 hard drives; SATA and NVMe (PCIe) protocols are optional. |  |  |  |  |  |  |
|  |  | mSATA |  | Optional 1 mSATA slot, defaults to WiFi module, can be modified to support mSATA drives. |  |  |  |  |  |  |
|  |  | SATA |  | One SATA 3.0 hard drive interface, supporting 2.5-inch SSDs/HDDs or 3.5-inch HDDs. |  |  |  |  |  |  |
|  |  | Hard drive bays in the chassis |  | Two hard drive bays (one 2.5-inch SSD/HDD bay and one 3.5-inch HDD bay; this only indicates the number of hard drives the case can accommodate). |  |  |  |  |  |  |
| Extend |  | Mini-PCIe |  | 1 Mini-PCIe slot, supporting 4G/ WiFi/Bluetooth modules |  |  |  |  |  |  |
| environment |  | Operating temperature |  | -10℃ ~ 60℃ |  |  |  |  |  |  |
|  |  | Storage temperature |  | -20℃ ~ 70℃ |  |  |  |  |  |  |
|  |  | relative humidity |  | 5%-95% relative humidity, non-condensing |  |  |  |  |  |  |
|  |  | Working vibration |  | 0.5grms/5-500HZ/random/operating |  |  |  |  |  |  |
| BIOS |  | AMI BIOS supports power-on, scheduled power-on, remote power-on, and intelligent device identification. |  |  |  |  |  |  |  |  |
| Watch Dog |  | Watchdog programming with support for hardware reset function (256 levels, 0~255 seconds). |  |  |  |  |  |  |  |  |
| operating system |  | Windows 10, Windows 8, Windows 7, Linux, etc. |  |  |  |  |  |  |  |  |

※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※

※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※

| HDMI®​ | VGA/DP |  | LAN | USB 2.0 | USB 3.0 | COM | SSD | Memory | Power |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 4K 60Hz | VGA 2K 60Hz Optional DP 4K 60Hz |  | 1 | 6 | 2 | 2/6 | 1*MSATA 1*SATA | 1*NB-DDR4 MAX 16G | 12V |
|  |  |  | 2 | 4 |  |  |  |  |  |
| PC model |  | IPC100U-J4125 series |  |  |  |  |  |  |  |
| PC size |  | Dimensions with handle: 430mm(L)*301mm(W)*44mm(H) Dimensions with handle: 480mm(L)*301mm(W)*44mm(H) |  |  |  |  |  |  |  |
| Cooling system |  | With ventilation holes, fanless or with silent fan cooling |  |  |  |  |  |  |  |
| color |  | black |  |  |  |  |  |  |  |
| Compatible motherboard models |  | M415F |  |  |  |  |  |  |  |
| motherboard |  | CPU | Supports Intel® Gemini Lake Refresh J and N series CPUs; Intel Celeron J4025 Dual-core 2.0GHz, Turbo Boost 2.9GHz; Intel Celeron J4125 Quad-core 2.0GHz, Turbo Boost 2.7GHz; Intel Celeron N4020 Dual-core 1.1GHz, Turbo Boost 2.8GHz; Intel Celeron N4120 Quad-core 1.1GHz, Turbo Boost 2.6GHz; Intel Pentium N5030 Quad-core 1.1GHz, Turbo Boost 3.1GHz; Intel Pentium J5040 Quad-core 2.0GHz, Turbo Boost 3.2GHz; |  |  |  |  |  |  |
|  |  | graphics card | Intel® UHD Graphics 600 graphics controller, supporting single display, dual display duplication, and dual display expansion. |  |  |  |  |  |  |
|  |  | Memory | 1*NB-DDR4 memory, supports 1.2V, supports 2400MHz memory, maximum support 16GB |  |  |  |  |  |  |
| Front panel interface |  | POWER | 1* Switch button |  |  |  |  |  |  |
|  |  | RESET | 1* Restart Switch |  |  |  |  |  |  |
|  |  | led | 1 power indicator, 1 hard drive indicator |  |  |  |  |  |  |
|  |  | USB | 2 USB 2.0 ports |  |  |  |  |  |  |
| Rear panel interface |  | Power interface | 1 power interface, built-in DC power adapter, supports DC 12V, recommended 12V 4A or higher power supply. |  |  |  |  |  |  |
|  |  | Display Interface | 1*HDMI® 2.0 port (4096/3840*2160@60Hz) 1 VGA interface (1920*1080@60Hz) Optional 1*DP 1.4 interface (4096/3840*2160@60Hz) |  |  |  |  |  |  |
|  |  | LAN | Single gigabit network card; default support for Realtek 8111H; Dual Gigabit Ethernet adapters; supports Realtek 8111H + Realtek 8111H; Supports Wake-on-LAN and PXE functionality. |  |  |  |  |  |  |
|  |  | USB | 2 USB 3.0 ports 4 USB 2.0 ports (2 USB 2.0 ports for dual-network PC) |  |  |  |  |  |  |
|  |  | COM | 6 COM interfaces, default RS232; COM1/COM3/COM4 are optional and support RS232/RS422/RS485. |  |  |  |  |  |  |
|  |  | audio | 1*LINE_OUT interface, audio output 1*MIC_IN interface, microphone input |  |  |  |  |  |  |
|  |  | WiFi | 2 WiFi antenna interfaces |  |  |  |  |  |  |
| storage |  | mSATA | 1 mSATA slot, supports mSATA drives |  |  |  |  |  |  |
|  |  | SATA | One SATA 3.0 hard drive interface, supporting 2.5-inch SSD/HDD or 3.5-inch HDD. |  |  |  |  |  |  |
|  |  | Hard drive bays in the chassis | Two hard drive bays (one 2.5-inch SSD/HDD bay and one 3.5-inch HDD bay; this only indicates the number of hard drives the case can accommodate). |  |  |  |  |  |  |
| Extend |  | Mini-PCIe | 1 Mini-PCIe slot, supports PCIe 2.0 x1 & USB signal, supports WiFi, Bluetooth/4G modules. |  |  |  |  |  |  |
| environment |  | Operating temperature | -10℃ ~ 60℃ |  |  |  |  |  |  |
|  |  | Storage temperature | -20℃ ~ 70℃ |  |  |  |  |  |  |
|  |  | relative humidity | 5%-95% relative humidity, non-condensing |  |  |  |  |  |  |
|  |  | Working vibration | 0.5grms/5-500HZ/random/operating |  |  |  |  |  |  |
| BIOS |  | AMI BIOS supports power-on, scheduled power-on, remote power-on, and intelligent device identification. |  |  |  |  |  |  |  |
| Watch Dog |  | Watchdog programming with support for hardware reset function (256 levels, 0~255 seconds). |  |  |  |  |  |  |  |
| operating system |  | Windows 11, Windows 10, Linux, etc. |  |  |  |  |  |  |  |

※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※

※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※

| HDMI® | VGA/DP |  | LAN | USB2.0 | USB3.1 | COM | SSD | Memory | Power |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 4K 60Hz | VGA 2K 60Hz Optional DP 4K 60Hz |  | 1 | 6 | 2 | 2/6 | 1*M.2 1*SATA | 1*NB-DDR4 MAX 16G | 12V |
|  |  |  | 2 | 4 |  |  |  |  |  |
| PC model |  | IPC100U-J6412 series |  |  |  |  |  |  |  |
| PC size |  | Dimensions with handle: 430mm(L)*301mm(W)*44mm(H) Dimensions with handle: 480mm(L)*301mm(W)*44mm(H) |  |  |  |  |  |  |  |
| Cooling system |  | With ventilation holes, fanless or with silent fan cooling |  |  |  |  |  |  |  |
| color |  | black |  |  |  |  |  |  |  |
| Compatible motherboard models |  | M612F |  |  |  |  |  |  |  |
| motherboard |  | CPU | Supports Intel® Elkhart Lake J and N series CPUs; Intel Celeron J6412 Quad-core 2.0GHz, Turbo Boost 2.6GHz; Intel Celeron J6413 Quad-core 1.8GHz, Turbo Boost 3.0GHz; Intel Celeron N6210 dual-core 1.2GHz, turbo boost 2.6GHz; Intel Celeron N6211 dual-core 1.2GHz, turbo boost 3.0GHz; Intel Pentium J6426 Quad-core 2.0GHz, Turbo Boost 3.0GHz; Intel Pentium N6415 Quad-core 1.2GHz, Turbo Boost 3.0GHz; |  |  |  |  |  |  |
|  |  | graphics card | Intel® UHD Graphics controller, supporting single display, dual display duplication, and dual display expansion. |  |  |  |  |  |  |
|  |  | Memory | 1*NB-DDR4 memory, supports 1.2V, supports 2400MHz memory, maximum support 16GB |  |  |  |  |  |  |
| Front panel interface |  | USB | 2 USB 2.0 ports |  |  |  |  |  |  |
|  |  | POWER | 1* Switch button |  |  |  |  |  |  |
|  |  | RESET | 1* Restart Switch |  |  |  |  |  |  |
|  |  | led | 1 power indicator, 1 hard drive indicator |  |  |  |  |  |  |
| Rear panel interface |  | Power interface | 1 power interface, built-in DC power adapter, supports DC 12V, recommended 12V 4A or higher power supply. |  |  |  |  |  |  |
|  |  | Display Interface | 1*HDMI® 2.0 port (4096/3840*2160@60Hz) 1 VGA interface (1920*1080@60Hz) Optional 1*DP 1.4 interface (4096/3840*2160@60Hz) |  |  |  |  |  |  |
|  |  | LAN | Single gigabit network card; default support for Realtek 8111H; Dual Gigabit Ethernet adapters; supports Realtek 8111H + Realtek 8111H; Supports Wake-on-LAN and PXE functionality. |  |  |  |  |  |  |
|  |  | USB | 2 USB 3.1 Gen1 ports 4 USB 2.0 ports (2 USB 2.0 ports for dual-network PC) |  |  |  |  |  |  |
|  |  | COM | 6 COM interfaces, default RS232; COM1/COM3/COM4/COM6 are optional and support RS232/RS485. |  |  |  |  |  |  |
|  |  | audio | 1*LINE_OUT interface, audio output 1*MIC_IN interface, microphone input |  |  |  |  |  |  |
|  |  | WiFi | 2 WiFi antenna interfaces |  |  |  |  |  |  |
| storage |  | M.2 SSD | 1 M.2 Key-M slot, supporting M.2 2280 NVMe PCIe 3.0 x2 / SATA dual-protocol adaptive standard SSDs. |  |  |  |  |  |  |
|  |  | SATA | One SATA 3.0 hard drive interface, supporting 2.5-inch SSD/HDD or 3.5-inch HDD. |  |  |  |  |  |  |
|  |  | Hard drive bays in the chassis | Two hard drive bays (one 2.5-inch SSD/HDD bay and one 3.5-inch HDD bay; this only indicates the number of hard drives the case can accommodate). |  |  |  |  |  |  |
| Extend |  | Mini-PCIe | 1 Mini-PCIe slot, supports PCIe 3.0 x1 & USB signal, supports WiFi, Bluetooth/4G modules. |  |  |  |  |  |  |
| Operating environment |  | Operating temperature | -10℃ ~ 60℃ |  |  |  |  |  |  |
|  |  | Storage temperature | -20℃ ~ 70℃ |  |  |  |  |  |  |
|  |  | relative humidity | 5%-95% relative humidity, non-condensing |  |  |  |  |  |  |
|  |  | Working vibration | 0.5grms/5-500HZ/random/operating |  |  |  |  |  |  |
| BIOS |  | Supports power-on upon startup, scheduled power-on, and remote power-on/off. |  |  |  |  |  |  |  |
| Watch Dog |  | Watchdog programming with support for hardware reset function (256 levels, 0~255 seconds). |  |  |  |  |  |  |  |
| operating system |  | Windows 11, Windows 10, Linux, etc. |  |  |  |  |  |  |  |

※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※

※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※

| HDMI®​ | VGA/DP |  | LAN | USB 2.0 | USB 3.2 | COM | SSD | Memory | Power |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 4K 60Hz | VGA 2K 60Hz Optional DP 4K 60Hz |  | 1 | 6 | 2 | 2/6 | 1*M.2 1*SATA | 1*NB-DDR4 MAX 16G | 12V |
|  |  |  | 2 | 4 |  |  |  |  |  |
| PC model |  | IPC100U-M100F series |  |  |  |  |  |  |  |
| PC size |  | Dimensions with handle: 430mm(L)*301mm(W)*44mm(H) Dimensions with handle: 480mm(L)*301mm(W)*44mm(H) |  |  |  |  |  |  |  |
| Cooling system |  | With ventilation holes, fanless or with silent fan cooling |  |  |  |  |  |  |  |
| color |  | black |  |  |  |  |  |  |  |
| Compatible motherboard models |  | M100F M150F |  |  |  |  |  |  |  |
| motherboard |  | CPU | Supports Intel® Alder Lake-N series CPUs; Intel Core i3-N300: Eight-core, 0.8GHz, turbo boost up to 3.8GHz; Core i3-N305: Eight-core, 1.8GHz, turbo boost up to 3.8GHz; Intel Core i3-N350: Eight-core, 1.6GHz, turbo boost up to 3.9GHz; Core i3-N355: Eight-core, 1.6GHz, turbo boost up to 3.9GHz; Intel N50 has a maximum turbo frequency of 3.4GHz; Intel N95 is a quad-core processor with a 1.7GHz turbo frequency of 3.4GHz. Intel N97 Quad-core 2.0GHz, Turbo Boost 3.6GHz; Intel N100 Quad-core 0.8GHz, Turbo Boost 3.4GHz; Intel N200 Quad-core 1.0GHz, Turbo Boost 3.7GHz; Intel N150 Quad-core 1.0GHz, Turbo Boost 3.6GHz; Intel N250 Quad-core 1.2GHz, Turbo Boost 3.8GHz; |  |  |  |  |  |  |
|  |  | graphics card | Intel® UHD Graphics controller, supporting single display, dual display duplication, and dual display expansion. |  |  |  |  |  |  |
|  |  | Memory | 1*NB-DDR4 memory, supports 1.2V, supports 2400MHz memory, maximum support 16GB |  |  |  |  |  |  |
| Front panel interface |  | USB | 2 USB 2.0 ports |  |  |  |  |  |  |
|  |  | POWER | 1* Switch button |  |  |  |  |  |  |
|  |  | RESET | 1* Restart Switch |  |  |  |  |  |  |
|  |  | led | 1 power indicator, 1 hard drive indicator |  |  |  |  |  |  |
| Rear panel interface |  | Power interface | 1 power interface, built-in DC power adapter, supports DC 12V, recommended 12V 4A or higher power supply. |  |  |  |  |  |  |
|  |  | Display Interface | 1*HDMI® 2.0 port (4096/3840*2160@60Hz) 1 VGA interface (1920*1080@60Hz) Optional 1*DP 1.4 interface (4096/3840*2160@60Hz) |  |  |  |  |  |  |
|  |  | LAN | Single gigabit network card; default support for Realtek 8111H; optional domestic Yutai Micro YT6801 gigabit network card; Dual Gigabit Ethernet cards; supports Realtek 8111H+Realtek 8111H; optional domestic YT6801+YT6801; Supports Wake-on-LAN and PXE functionality. |  |  |  |  |  |  |
|  |  | USB | 2 USB 3.2 Gen1 ports 4 USB 2.0 ports (2 USB 2.0 ports for dual-network PC) |  |  |  |  |  |  |
|  |  | COM | 6 COM interfaces, default RS232; COM2/COM6 can be RS232/RS485; COM3/COM4 can be RS232/RS422/RS485. |  |  |  |  |  |  |
|  |  | audio | 1*LINE_OUT interface, audio output 1*MIC_IN interface, microphone input |  |  |  |  |  |  |
|  |  | WiFi | 2 WiFi antenna interfaces |  |  |  |  |  |  |
| storage |  | M.2 SSD | 1 M.2 Key-M slot, supporting M.2 2280 NVMe PCIe 3.0 x2 / SATA dual-protocol adaptive standard SSDs. |  |  |  |  |  |  |
|  |  | SATA | One SATA 3.0 hard drive interface, supporting 2.5-inch SSD/HDD or 3.5-inch HDD. |  |  |  |  |  |  |
|  |  | Hard drive bays in the chassis | Two hard drive bays (one 2.5-inch SSD/HDD bay and one 3.5-inch HDD bay; this only indicates the number of hard drives the case can accommodate). |  |  |  |  |  |  |
| Extend |  | M.2 WiFi | One M.2 Key-E slot, supporting M.2 2230 PCIe 3.0 x1 & USB signals, and supporting WiFi and Bluetooth modules. |  |  |  |  |  |  |
| Operating environment |  | Operating temperature | -10℃ ~ 60℃ |  |  |  |  |  |  |
|  |  | Storage temperature | -20℃ ~ 70℃ |  |  |  |  |  |  |
|  |  | relative humidity | 5%-95% relative humidity, non-condensing |  |  |  |  |  |  |
|  |  | Working vibration | 0.5grms/5-500HZ/random/operating |  |  |  |  |  |  |
| BIOS |  | Supports power-on upon startup, scheduled power-on, and remote power-on/off. |  |  |  |  |  |  |  |
| Watch Dog |  | Watchdog programming with support for hardware reset function (256 levels, 0~255 seconds). |  |  |  |  |  |  |  |
| operating system |  | Windows 11, Windows 10, Linux, etc. |  |  |  |  |  |  |  |

※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※

| HDMI®​ | VGA |  | LAN |  | USB 2.0 | USB 3.0 | COM | SSD | Memory | Power |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2K 60Hz | 2K 60Hz |  | 1 |  | 6 | 2 | 2/6 | 1*MSATA 2*SATA | 1*NB-DDR3 MAX 8G | 12V |
|  |  |  | 2 |  | 4 |  |  |  |  |  |
| PC model |  | IPC100U-1037 series |  |  |  |  |  |  |  |  |
| PC size |  | Dimensions with handle: 430mm(L)*301mm(W)*44mm(H) Dimensions with handle: 480mm(L)*301mm(W)*44mm(H) |  |  |  |  |  |  |  |  |
| Cooling system |  | With ventilation holes and silent fan cooling |  |  |  |  |  |  |  |  |
| color |  | black |  |  |  |  |  |  |  |  |
| Compatible motherboard models |  | 1007F, 1037F, 2117F |  |  |  |  |  |  |  |  |
| motherboard |  | CPU |  | Supports Intel® Ivy Bridge Celeron series CPUs; Intel Celeron 1007U Dual-Core 1.5GHz; Intel Celeron 1037U Dual-Core 1.8GHz; Intel Pentium 2117U Dual-Core 1.8GHz; |  |  |  |  |  |  |
|  |  | graphics card |  | Intel® HD Graphics 4000 graphics controller, supporting single display, dual display duplication, and dual display expansion. |  |  |  |  |  |  |
|  |  | Memory |  | 1*NB-DDR3 memory, supports 1.35V/1.5V, supports 1333/1600MHz memory, maximum support 8GB |  |  |  |  |  |  |
| Front panel interface |  | POWER |  | 1* Switch button |  |  |  |  |  |  |
|  |  | RESET |  | 1* Restart Switch |  |  |  |  |  |  |
|  |  | led |  | 1 power indicator, 1 hard drive indicator |  |  |  |  |  |  |
|  |  | USB |  | 2 USB 2.0 ports |  |  |  |  |  |  |
| Rear panel interface |  | Power interface |  | 1 power interface, built-in DC power adapter, supports DC 12V, recommended 12V 5A or higher power supply. |  |  |  |  |  |  |
|  |  | Display Interface |  | 1*HDMI® 1.4 port (1920*1080@60Hz) 1 VGA interface (1920*1080@60Hz) |  |  |  |  |  |  |
|  |  | LAN |  | Single gigabit network card; default support for Realtek 8111H; Dual Gigabit Ethernet adapters; supports Realtek 8111H + Realtek 8111H; Supports Wake-on-LAN and PXE functionality. |  |  |  |  |  |  |
|  |  | USB |  | 2 USB 3.0 ports 4 USB 2.0 ports (2 USB 2.0 ports for dual-network PC) |  |  |  |  |  |  |
|  |  | COM |  | 6 COM interfaces, default RS232; COM2 can optionally support RS232/RS422/RS485. |  |  |  |  |  |  |
|  |  | audio |  | 1*LINE_OUT interface, audio output 1*MIC_IN interface, microphone input |  |  |  |  |  |  |
|  |  | WiFi |  | 2 WiFi antenna interfaces |  |  |  |  |  |  |
| storage |  | mSATA |  | One mSATA slot supports mSATA drives; the SATA signal can be converted to the PCIe signal, which, after conversion, supports WiFi modules. |  |  |  |  |  |  |
|  |  | SATA |  | 2 SATA 3.0 hard drive interfaces, supporting 2.5-inch SSDs/HDDs or 3.5-inch HDDs. |  |  |  |  |  |  |
|  |  | Hard drive bays in the chassis |  | Two hard drive bays (one 2.5-inch SSD/HDD bay and one 3.5-inch HDD bay; this only indicates the number of hard drives the case can accommodate). |  |  |  |  |  |  |
| Extend |  | Mini-PCIe |  | 1 Mini-PCIe slot, supports PCIe 2.0 x1 & USB signal, supports WiFi, Bluetooth/4G modules. |  |  |  |  |  |  |
| environment |  | Operating temperature |  | -10℃ ~ 60℃ |  |  |  |  |  |  |
|  |  | Storage temperature |  | -20℃ ~ 70℃ |  |  |  |  |  |  |
|  |  | relative humidity |  | 5%-95% relative humidity, non-condensing |  |  |  |  |  |  |
|  |  | Working vibration |  | 0.5grms/5-500HZ/random/operating |  |  |  |  |  |  |
| BIOS |  | AMI BIOS supports power-on, scheduled power-on, remote power-on, and intelligent device identification. |  |  |  |  |  |  |  |  |
| Watch Dog |  | Watchdog programming with support for hardware reset function (256 levels, 0~255 seconds). |  |  |  |  |  |  |  |  |
| operating system |  | Windows 10, Windows 8, Windows 7, Windows XP, Linux, etc. |  |  |  |  |  |  |  |  |

※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※

※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※

| HDMI®​ | VGA |  | LAN | USB 2.0 | USB 3.0 | COM | SSD | Memory | Power |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2K 60Hz | 2K 60Hz |  | 1 | 6 | 2 | 2/6 | 1*MSATA 1/2*SATA | 1*NB-DDR3 MAX 8G | 12V |
|  |  |  | 2 | 4 |  |  |  |  |  |
| PC model |  | IPC100U-2nd Generation Series |  |  |  |  |  |  |  |
| PC size |  | Dimensions with handle: 430mm(L)*301mm(W)*44mm(H) Dimensions with handle: 480mm(L)*301mm(W)*44mm(H) |  |  |  |  |  |  |  |
| Cooling system |  | With ventilation holes and silent fan cooling |  |  |  |  |  |  |  |
| color |  | black |  |  |  |  |  |  |  |
| Compatible motherboard models |  | QM6600, QM6637, QM6616, QM2360 |  |  |  |  |  |  |  |
| motherboard |  | CPU | Intel Core i3-2310M Dual-core 2.1GHz; Intel Core i3-2330M Dual-core 2.2GHz; Intel Core i3-2350M Dual-core 2.3GHz; Intel Core i3-2370M Dual-core 2.4GHz; Intel Core i5-2410M dual-core 2.3GHz, turbo boost 2.9GHz; Core i5-2430M dual-core 2.4GHz, turbo boost 3.0GHz; Intel Core i5-2520M dual-core 2.5GHz, turbo boost 3.2GHz; Core i5-2540M dual-core 2.6GHz, turbo boost 3.3GHz; Intel Core i7-2620M dual-core 2.7GHz, turbo boost 3.4GHz; Core i7-2640M dual-core 2.8GHz, turbo boost 3.5GHz; |  |  |  |  |  |  |
|  |  | graphics card | Intel® HD Graphics 3000 graphics controller, supporting single display, dual display duplication, and dual display expansion. |  |  |  |  |  |  |
|  |  | Memory | 1*NB-DDR3 memory, supports 1.35V/1.5V, supports 1333/1600MHz memory, maximum support 8GB |  |  |  |  |  |  |
| Front panel interface |  | POWER | 1* Switch button |  |  |  |  |  |  |
|  |  | RESET | 1* Restart Switch |  |  |  |  |  |  |
|  |  | led | 1 power indicator, 1 hard drive indicator |  |  |  |  |  |  |
|  |  | USB | 2 USB 2.0 ports |  |  |  |  |  |  |
| Rear panel interface |  | Power interface | 1 power interface, built-in DC power adapter, supports DC 12V, 12V 7A or higher power supply recommended. |  |  |  |  |  |  |
|  |  | Display Interface | 1*HDMI® 1.4 port (1920*1080@60Hz) 1 VGA interface (1920*1080@60Hz) |  |  |  |  |  |  |
|  |  | LAN | Single gigabit network card; default support for Realtek 8111H; Dual Gigabit Ethernet adapters; supports Realtek 8111H + Realtek 8111H; Supports Wake-on-LAN and PXE functionality. |  |  |  |  |  |  |
|  |  | USB | 2 USB 3.0 ports 4 USB 2.0 ports (2 USB 2.0 ports for dual-network PC) |  |  |  |  |  |  |
|  |  | COM | 6 COM interfaces, default RS232; COM2 can optionally support RS232/RS422/RS485. |  |  |  |  |  |  |
|  |  | audio | 1*LINE_OUT interface, audio output 1*MIC_IN interface, microphone input |  |  |  |  |  |  |
|  |  | WiFi | 2 WiFi antenna interfaces |  |  |  |  |  |  |
| storage |  | mSATA | 1 mSATA slot, supports mSATA drives |  |  |  |  |  |  |
|  |  | SATA | 1/2 SATA 3.0 hard drive interfaces, supporting 2.5-inch SSDs/HDDs or 3.5-inch HDDs. |  |  |  |  |  |  |
|  |  | Hard drive bays in the chassis | Two hard drive bays (one 2.5-inch SSD/HDD bay and one 3.5-inch HDD bay; this only indicates the number of hard drives the case can accommodate). |  |  |  |  |  |  |
| Extend |  | Mini-PCIe | 1 Mini-PCIe slot, supports PCIe 2.0 x1 & USB signal, supports WiFi, Bluetooth/4G modules. |  |  |  |  |  |  |
| environment |  | Operating temperature | -10℃ ~ 60℃ |  |  |  |  |  |  |
|  |  | Storage temperature | -20℃ ~ 70℃ |  |  |  |  |  |  |
|  |  | relative humidity | 5%-95% relative humidity, non-condensing |  |  |  |  |  |  |
|  |  | Working vibration | 0.5grms/5-500HZ/random/operating |  |  |  |  |  |  |
| BIOS |  | AMI BIOS supports power-on, scheduled power-on, remote power-on, and intelligent device identification. |  |  |  |  |  |  |  |
| Watch Dog |  | Watchdog programming with support for hardware reset function (256 levels, 0~255 seconds). |  |  |  |  |  |  |  |
| operating system |  | Windows 10, Windows 7, Windows XP, Linux, etc. |  |  |  |  |  |  |  |

※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※

| HDMI®​ | VGA |  | LAN |  | USB 2.0 | USB 3.0 | COM | SSD | Memory | Power |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2K 60Hz | 2K 60Hz |  | 1 |  | 6 | 2 | 2/6 | 1*MSATA 1/2*SATA | 1*NB-DDR3 MAX 8G | 12V |
|  |  |  | 2 |  | 4 |  |  |  |  |  |
| PC model |  | IPC100U-3rd Generation Series |  |  |  |  |  |  |  |  |
| PC size |  | Dimensions with handle: 430mm(L)*301mm(W)*44mm(H) Dimensions with handle: 480mm(L)*301mm(W)*44mm(H) |  |  |  |  |  |  |  |  |
| Cooling system |  | With ventilation holes and silent fan cooling |  |  |  |  |  |  |  |  |
| color |  | black |  |  |  |  |  |  |  |  |
| Compatible motherboard models |  | i3HGP, i5HGP, i7HGP, QM6600, QM6637, QM6616, QM2360 |  |  |  |  |  |  |  |  |
| motherboard |  | CPU |  | Intel® Ivy Bridge U and M series Core i3-3217U, i3-3110M, i5-3317U, i5-3210M, i7-3517U, i7-3520M (See the next page for CPU list details) |  |  |  |  |  |  |
|  |  | graphics card |  | Intel® HD Graphics 4000 graphics controller, supporting single display, dual display duplication, and dual display expansion. |  |  |  |  |  |  |
|  |  | Memory |  | 1*NB-DDR3 memory, supports 1.35V/1.5V, supports 1333/1600MHz memory, maximum support 8GB |  |  |  |  |  |  |
| Front panel interface |  | POWER |  | 1* Switch button |  |  |  |  |  |  |
|  |  | RESET |  | 1* Restart Switch |  |  |  |  |  |  |
|  |  | led |  | 1 power indicator, 1 hard drive indicator |  |  |  |  |  |  |
|  |  | USB |  | 2 USB 2.0 ports |  |  |  |  |  |  |
| Rear panel interface |  | Power interface |  | 1 power interface, built-in DC power adapter U-series power supplies are recommended to use 12V 5A or higher power supply; For the M series, a 12V 7A or higher power supply is recommended. |  |  |  |  |  |  |
|  |  | Display Interface |  | 1*HDMI® 1.4 port (1920*1080@60Hz) 1 VGA interface (1920*1080@60Hz) |  |  |  |  |  |  |
|  |  | LAN |  | Single gigabit network card; default support for Realtek 8111H; Dual Gigabit Ethernet adapters; supports Realtek 8111H + Realtek 8111H; Supports Wake-on-LAN and PXE functionality. |  |  |  |  |  |  |
|  |  | USB |  | 2 USB 3.0 ports 4 USB 2.0 ports (2 USB 2.0 ports for dual-network PC) |  |  |  |  |  |  |
|  |  | COM |  | 6 COM interfaces, default RS232; COM2 can optionally support RS232/RS422/RS485. |  |  |  |  |  |  |
|  |  | audio |  | 1*LINE_OUT interface, audio output 1*MIC_IN interface, microphone input |  |  |  |  |  |  |
|  |  | WiFi |  | 2 WiFi antenna interfaces |  |  |  |  |  |  |
| storage |  | mSATA |  | 1 mSATA slot, supports mSATA drives |  |  |  |  |  |  |
|  |  | SATA |  | 1/2 SATA 3.0 hard drive interfaces, supporting 2.5-inch SSDs/HDDs or 3.5-inch HDDs. |  |  |  |  |  |  |
|  |  | Hard drive bays in the chassis |  | Two hard drive bays (one 2.5-inch SSD/HDD bay and one 3.5-inch HDD bay; this only indicates the number of hard drives the case can accommodate). |  |  |  |  |  |  |
| Extend |  | Mini-PCIe |  | 1 Mini-PCIe slot, supports PCIe 2.0 x1 & USB signal, supports WiFi, Bluetooth/4G modules. |  |  |  |  |  |  |
| environment |  | Operating temperature |  | -10℃ ~ 60℃ |  |  |  |  |  |  |
|  |  | Storage temperature |  | -20℃ ~ 70℃ |  |  |  |  |  |  |
|  |  | relative humidity |  | 5%-95% relative humidity, non-condensing |  |  |  |  |  |  |
|  |  | Working vibration |  | 0.5grms/5-500HZ/random/operating |  |  |  |  |  |  |
| BIOS |  | AMI BIOS supports power-on, scheduled power-on, remote power-on, and intelligent device identification. |  |  |  |  |  |  |  |  |
| Watch Dog |  | Watchdog programming with support for hardware reset function (256 levels, 0~255 seconds). |  |  |  |  |  |  |  |  |
| operating system |  | Windows 10, Windows 7, Windows XP, Linux, etc. |  |  |  |  |  |  |  |  |

※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※

※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※

The CPU information below is for reference only. Please consult our sales team for specific requirements!

| CPU Specifications | CPU Brand | CPU model | clock speed | Turbo Boost | CPU power consumption (W) | Full load power consumption is approximately (W) |
| --- | --- | --- | --- | --- | --- | --- |
| 3rd generation i3 | Core | 3217U | Dual-core 1.8G |  | 17 | 40 |
|  |  | 3227U | Dual-core 1.9G |  | 17 | 40 |
| 3rd generation i5 | Core | 3317U | Dual-core 1.7GHz | 2.6G | 17 | 40 |
|  |  | 3337U | Dual-core 1.8G | 2.7G | 17 | 40 |
|  |  | 3427U | Dual-core 1.8G | 2.8G | 17 | 40 |
|  |  | 3437U | Dual-core 1.9G | 2.9G | 17 | 40 |
| 3rd generation i7 | Core | 3517U | Dual-core 1.9G | 3.0G | 17 | 40 |
|  |  | 3537U | Dual-core 2.0G | 3.1G | 17 | 40 |
|  |  | 3667U | Dual-core 2.0G | 3.2G | 17 | 40 |
|  |  | 3687U | Dual-core 2.1G | 3.3G | 17 | 40 |
| 3rd generation i3 | Core | 3110M | Dual-core 2.4G |  | 35 | 60 |
|  |  | 3120M | Dual-core 2.5G |  | 35 | 60 |
|  |  | 3130M | Dual-core 2.6G |  | 35 | 60 |
| 3rd generation i5 | Core | 3210M | Dual-core 2.5G | 3.1G | 35 | 60 |
|  |  | 3230M | Dual-core 2.6G | 3.2G | 35 | 60 |
|  |  | 3320M | Dual-core 2.6G | 3.3G | 35 | 60 |
|  |  | 3340M | Dual-core 2.7G | 3.4G | 35 | 60 |
|  |  | 3360M | Dual-core 2.8G | 3.5G | 35 | 60 |
|  |  | 3380M | Dual-core 2.9G | 3.6G | 35 | 60 |
| 3rd generation i7 | Core | 3520M | Dual-core 2.9G | 3.6G | 35 | 60 |
|  |  | 3540M | Dual-core 3.0G | 3.7G | 35 | 60 |

※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※

※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※

| HDMI® | VGA |  | LAN |  | USB2.0 | USB3.0 | COM | SSD | Memory | Power |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 4K 30Hz | 2K 60Hz |  | 1 |  | 6 | 2 | 2/6 | 1*MSATA 1/2*SATA | 1*NB-DDR3 MAX 8G | 12V |
|  |  |  | 2 |  | 4 |  |  |  |  |  |
| PC model |  | IPC100U-4th and 5th Generation Series |  |  |  |  |  |  |  |  |
| PC size |  | Dimensions with handle: 430mm(L)*301mm(W)*44mm(H) Dimensions with handle: 480mm(L)*301mm(W)*44mm(H) |  |  |  |  |  |  |  |  |
| Cooling system |  | With ventilation holes and silent fan cooling |  |  |  |  |  |  |  |  |
| color |  | black |  |  |  |  |  |  |  |  |
| Compatible motherboard models |  | M series: QM8680, QM8680L U series: QM9400/QM9400H/QM9400L/QM9450, QM9500/QM9500H/QM9500L/QM9550 |  |  |  |  |  |  |  |  |
| motherboard |  | CPU |  | 4th generation: Intel Haswell U and M series Core i3-4005U, i3-4000M, i5-4200U, i5-4200M, i7-4500U, i7-4600M, etc. 5th generation: Intel Broadwell Core i3-5005U, i5-5200U, i7-5500U, etc.; (See the next page for CPU list details) |  |  |  |  |  |  |
|  |  | graphics card |  | 4th generation M series: Intel® HD Graphics 4600 graphics controller, supporting single display, dual display duplication, and dual display expansion; 4th generation U series: Intel® HD Graphics 4400 graphics controller, supporting single display, dual display duplication, and dual display expansion; 5th generation: Intel® HD Graphics 5500 graphics controller, supporting single display, dual display duplication, and dual display expansion; |  |  |  |  |  |  |
|  |  | Memory |  | 1*NB-DDR3 memory, supports 1.35V, supports 1333/1600MHz memory, maximum support 8GB |  |  |  |  |  |  |
| Front panel interface |  | POWER |  | 1* Switch button |  |  |  |  |  |  |
|  |  | RESET |  | 1* Restart Switch |  |  |  |  |  |  |
|  |  | led |  | 1 power indicator, 1 hard drive indicator |  |  |  |  |  |  |
|  |  | USB |  | 2 USB 2.0 ports |  |  |  |  |  |  |
| Rear panel interface |  | Power interface |  | 1 power interface, built-in DC power adapter, supports DC 12V, recommended 12V 5A or higher power supply. |  |  |  |  |  |  |
|  |  | Display Interface |  | 1*HDMI® 1.4 port (4096/3840*2160@30Hz) 1 VGA interface (1920*1080@60Hz) |  |  |  |  |  |  |
|  |  | LAN |  | Single gigabit network card; default support for Realtek 8111H gigabit network card or Realtek 8111F gigabit network card; Dual Gigabit Ethernet adapters; supports dual Realtek 8111H Gigabit Ethernet adapters or dual Realtek 8111F Gigabit Ethernet adapters. Supports Wake-on-LAN and PXE functionality. |  |  |  |  |  |  |
|  |  | USB |  | 2 USB 3.0 ports 4 USB 2.0 ports (2 USB 2.0 ports for dual-network PC) |  |  |  |  |  |  |
|  |  | COM |  | 6 COM interfaces, default RS232; optional support for 1 RS232/RS422/RS485 |  |  |  |  |  |  |
|  |  | audio |  | 1*LINE_OUT interface, audio output 1*MIC_IN interface, microphone input |  |  |  |  |  |  |
|  |  | WiFi |  | 2 WiFi antenna interfaces |  |  |  |  |  |  |
| storage |  | mSATA |  | 1 mSATA slot, supports mSATA drives |  |  |  |  |  |  |
|  |  | SATA |  | It has 2 SATA 3.0 hard drive interfaces or 1 SATA 3.0 and 1 SATA 2.0 hard drive interface, supporting 2.5-inch SSDs/HDDs or 3.5-inch HDDs. |  |  |  |  |  |  |
|  |  | Hard drive bays in the chassis |  | Two hard drive bays (one 2.5-inch SSD/HDD bay and one 3.5-inch HDD bay; this only indicates the number of hard drives the case can accommodate). |  |  |  |  |  |  |
| Extend |  | Mini-PCIe |  | 1 Mini-PCIe slot, supports PCIe 2.0 x1 & USB signal, supports WiFi, Bluetooth/4G modules. |  |  |  |  |  |  |
| environment |  | Operating temperature |  | -10℃ ~ 60℃ |  |  |  |  |  |  |
|  |  | Storage temperature |  | -20℃ ~ 70℃ |  |  |  |  |  |  |
|  |  | relative humidity |  | 5%-95% relative humidity, non-condensing |  |  |  |  |  |  |
|  |  | Working vibration |  | 0.5grms/5-500HZ/random/operating |  |  |  |  |  |  |
| BIOS |  | AMI BIOS supports power-on, scheduled power-on, remote power-on, and intelligent device identification. |  |  |  |  |  |  |  |  |
| Watch Dog |  | Watchdog programming with support for hardware reset function (256 levels, 0~255 seconds). |  |  |  |  |  |  |  |  |
| operating system |  | Windows 10, Windows 7, Linux, etc. |  |  |  |  |  |  |  |  |

The CPU information below is for reference only. Please consult our sales team for specific requirements!

| CPU Specifications | CPU Brand | CPU model | clock speed | Turbo Boost | TDP power consumption (W) | Full load power consumption is approximately (W) |
| --- | --- | --- | --- | --- | --- | --- |
| 4th generation i3 | Core | 4000M | Dual-core 2.4G |  | 37 | 60 |
|  |  | 4100M | Dual-core 2.5G |  | 37 | 60 |
|  |  | 4110M | Dual-core 2.6G |  | 37 | 60 |
| 4th generation i5 | Core | 4200M | Dual-core 2.5G | 3.1G | 37 | 60 |
|  |  | 4210M | Dual-core 2.6G | 3.2G | 37 | 60 |
|  |  | 4300M | Dual-core 2.6G | 3.3G | 37 | 60 |
|  |  | 4310M | Dual-core 2.7G | 3.4G | 37 | 60 |
|  |  | 4330M | Dual-core 2.8G | 3.5G | 37 | 60 |
|  |  | 4340M | Dual-core 2.9G | 3.6G | 37 | 60 |
| 4th generation i7 | Core | 4600M | Dual-core 2.9G | 3.6G | 37 | 60 |
|  |  | 4610M | Dual-core 3.0G | 3.7G | 37 | 60 |
|  | Intel Pentium | 3825U | Dual-core 1.9G |  | 10 | 30 |
| 4th generation i3 | Intel Core | 4005U | Dual-core 1.7GHz |  | 15 | 40 |
|  |  | 4010U | Dual-core 1.7GHz |  | 15 | 40 |
|  |  | 4025U | Dual-core 1.9G |  | 15 | 40 |
|  |  | 4030U | Dual-core 1.9G |  | 15 | 40 |
|  |  | 4100U | Dual-core 1.8G |  | 15 | 40 |
| 4th generation i5 | Intel Core | 4200U | Dual-core 1.6G | 2.6G | 15 | 40 |
|  |  | 4210U | Dual-core 1.7GHz | 2.7G | 15 | 40 |
|  |  | 4250U | Dual-core 1.3G | 2.6G | 15 | 40 |
|  |  | 4278U | Dual-core 2.6G | 3.1G | 28 | 50 |
|  |  | 4300U | Dual-core 1.9G | 2.9G | 15 | 40 |
|  |  | 4308U | Dual-core 2.8G | 3.3G | 28 | 50 |
|  |  | 4310U | Dual-core 2.0G | 3.0G | 15 | 40 |
| 4th generation i7 | Intel Core | 4500U | Dual-core 1.8G | 3.0G | 15 | 40 |
|  |  | 4510U | Dual-core 2.0G | 3.1G | 15 | 40 |
|  |  | 4550U | Dual-core 1.5G | 3.0G | 15 | 40 |
|  |  | 4558U | Dual-core 2.8G | 3.3G | 15 | 40 |
|  |  | 4578U | Dual-core 3.0G | 3.5G | 28 | 50 |
|  |  | 4600U | Dual-core 2.1G | 3.3G | 15 | 35 |
|  |  | 4650U | Dual-core 1.7GHz | 3.0G | 15 | 40 |
| 5th generation i3 | Intel Core | 5005U | Dual-core 2.0G |  | 15 | 40 |
|  |  | 5010U | Dual-core 2.1G |  | 15 | 40 |
|  |  | 5015U | Dual-core 2.1G |  | 15 | 40 |
|  |  | 5020U | Dual-core 2.2GHz |  | 15 | 40 |
| 5th generation i5 | Intel Core | 5200U | Dual-core 2.2GHz | 2.7G | 15 | 40 |
|  |  | 5250U | Dual-core 1.6G | 2.7G | 15 | 40 |
|  |  | 5257U | Dual-core 2.7G | 3.1G | 15 | 40 |
|  |  | 5287U | Dual-core 2.9G | 3.3G | 28 | 50 |
|  |  | 5300U | Dual-core 2.3G | 2.9G | 15 | 40 |
|  |  | 5350U | Dual-core 1.8G | 2.9G | 15 | 40 |
| 5th generation i7 | Intel Core | 5500U | Dual-core 2.4G | 3.0G | 15 | 40 |
|  |  | 5550U | Dual-core 2.0G | 3.0G | 15 | 40 |
|  |  | 5557U | Dual-core 3.1GHz | 3.4G | 28 | 50 |
|  |  | 5600U | Dual-core 2.6G | 3.2G | 15 | 40 |
|  |  | 5650U | Dual-core 2.2GHz | 3.1G | 15 | 40 |

※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※

| HDMI®​ | VGA |  | LAN |  | USB 2.0 | USB 3.0 | COM | SSD | Memory | Power |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 4K 30Hz | 2K 60Hz |  | 1 |  | 6 | 2 | 2/6 | 1*MSATA 2*SATA | 2*NB-DDR3 MAX 32G | 12V/19V |
|  |  |  | 2 |  | 4 |  |  |  |  |  |
| PC model |  | IPC100U-6th and 7th generation series |  |  |  |  |  |  |  |  |
| PC size |  | Dimensions with handle: 430mm(L)*301mm(W)*44mm(H) Dimensions with handle: 480mm(L)*301mm(W)*44mm(H) |  |  |  |  |  |  |  |  |
| Cooling system |  | With ventilation holes and silent fan cooling |  |  |  |  |  |  |  |  |
| color |  | black |  |  |  |  |  |  |  |  |
| Compatible motherboard models |  | QM9600, QM9700, QM9600H, QM9700H |  |  |  |  |  |  |  |  |
| motherboard |  | CPU |  | 6th generation: Intel® Celeron 3855U, 3955U and Sky Lake Core i3-6100U, i5-6200U, i7-6500U, etc.; 7th generation: Intel® Celeron 3865U, 3965U and Kaby Lake Core i3-7100U, i5-7200U, i7-7500U, etc.; (See the next page for CPU list details) |  |  |  |  |  |  |
|  |  | graphics card |  | 6th generation: Intel® HD Graphics 510/520 graphics controller, supporting single display, dual display duplication, and dual display expansion. 7th generation: Intel® HD Graphics 610/620 graphics controller, supporting single display, dual display duplication, and dual display expansion. |  |  |  |  |  |  |
|  |  | Memory |  | 2*NB-DDR3 memory modules, supporting 1.35V, 1600MHz memory, maximum support 32GB. |  |  |  |  |  |  |
| Front panel interface |  | POWER |  | 1* Switch button |  |  |  |  |  |  |
|  |  | RESET |  | 1* Restart Switch |  |  |  |  |  |  |
|  |  | led |  | 1 power indicator, 1 hard drive indicator |  |  |  |  |  |  |
|  |  | USB |  | 2 USB 2.0 ports |  |  |  |  |  |  |
| Rear panel interface |  | Power interface |  | 1 power interface, built-in DC power adapter, supports DC 12V, recommended power supply is 12V 5A or 19V 3A or higher. |  |  |  |  |  |  |
|  |  | Display Interface |  | 1*HDMI® 1.4 port (4096/3840*2160@30HZ) 1 VGA interface (1920*1080@60Hz) |  |  |  |  |  |  |
|  |  | LAN |  | Single Gigabit Ethernet adapter: Supports Realtek 8111F, Realtek 8111H, or Yutaiwei YT6801 Gigabit Ethernet adapter; Dual Gigabit Ethernet adapters: Supports Realtek 8111F + Realtek 8111F or optionally supports Realtek 8111H + Intel I219-LM Alternatively, it may support Yutai Micro YT6801+ Intel I219-LM; Supports Wake-on-LAN and PXE network functionality; |  |  |  |  |  |  |
|  |  | USB |  | 2 USB 3.0 ports 4 USB 2.0 ports (2 USB 2.0 ports for dual-network PC) |  |  |  |  |  |  |
|  |  | COM |  | 6 COM interfaces, default RS232; COM2 can optionally support RS232/RS422/RS485. |  |  |  |  |  |  |
|  |  | audio |  | 1*LINE_OUT interface, audio output 1*MIC_IN interface, microphone input |  |  |  |  |  |  |
|  |  | WiFi |  | 2 WiFi antenna interfaces |  |  |  |  |  |  |
| storage |  | mSATA |  | 1 mSATA slot, supports mSATA drives |  |  |  |  |  |  |
|  |  | SATA |  | 2 SATA 3.0 hard drive interfaces, supporting 2.5-inch SSDs/HDDs or 3.5-inch HDDs. |  |  |  |  |  |  |
|  |  | Hard drive bays in the chassis |  | Two hard drive bays (one 2.5-inch SSD/HDD bay and one 3.5-inch HDD bay; this only indicates the number of hard drives the case can accommodate). |  |  |  |  |  |  |
| Extend |  | Mini-PCIe |  | One Mini-PCIe slot, supporting PCIe 3.0 x1 signal and WiFi module; optional support for WiFi, Bluetooth/4G modules. |  |  |  |  |  |  |
| Operating environment |  | Operating temperature |  | -10℃ ~ 60℃ |  |  |  |  |  |  |
|  |  | Storage temperature |  | -20℃ ~ 70℃ |  |  |  |  |  |  |
|  |  | relative humidity |  | 5%-95% relative humidity, non-condensing |  |  |  |  |  |  |
|  |  | Working vibration |  | 0.5grms/5-500HZ/random/operating |  |  |  |  |  |  |
| BIOS |  | Supports power-on upon startup, scheduled power-on, and remote power-on/off. |  |  |  |  |  |  |  |  |
| Watch Dog |  | Watchdog programming with support for hardware reset function (256 levels, 0~255 seconds). |  |  |  |  |  |  |  |  |
| operating system |  | Windows 10, Windows 7, Linux, etc. |  |  |  |  |  |  |  |  |

※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※

※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※

The CPU information below is for reference only. Please consult our sales team for specific requirements!

| CPU Specifications | CPU Brand | CPU model | clock speed | Turbo Boost | TDP power consumption (W) | Full load power consumption is approximately (W) |
| --- | --- | --- | --- | --- | --- | --- |
| 6th generation | Cy Young | 3855U | Dual-core 1.6G |  | 15 | 40 |
|  |  | 3955U | Dual-core 2.0G |  | 15 | 40 |
| 6th generation i3 | Core | 6006U | Dual-core 2.0G |  | 15 | 40 |
|  |  | 6100U | Dual-core 2.3G |  | 15 | 40 |
| 6th generation i5 | Core | 6200U | Dual-core 2.3G | 2.8G | 15 | 40 |
|  |  | 6300U | Dual-core 2.4G | 3.0G | 15 | 40 |
| 6th generation i7 | Core | 6500U | Dual-core 2.5G | 3.1G | 15 | 40 |
|  |  | 6600U | Dual-core 2.6G | 3.4G | 15 | 40 |
|  |  | 6650U | Dual-core 2.2GHz | 3.4G | 15 | 40 |
| 7th generation | Cy Young | 3865U | Dual-core 1.8G |  | 15 | 40 |
|  |  | 3965U | Dual-core 2.2GHz |  | 15 | 40 |
| 7th generation i3 | Core | 7020U | Dual-core 2.3G |  | 15 | 40 |
|  |  | 7100U | Dual-core 2.4G |  | 15 | 40 |
|  |  | 7130U | Dual-core 2.7GHz |  | 15 | 40 |
| 7th Gen i5 | Core | 7200U | Dual-core 2.5G | 3.1G | 15 | 40 |
|  |  | 7300U | Dual-core 2.6G | 3.5G | 15 | 40 |
| 7th generation i7 | Core | 7500U | Dual-core 2.7G | 3.5G | 15 | 40 |
|  |  | 7600U | Dual-core 2.8G | 3.9G | 15 | 40 |

※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※

※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※

| HDMI®​ | VGA/DP |  | LAN | USB 2.0 | USB 3.0 | COM | SSD | Memory | Power |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 4K 30Hz | VGA 2K 60Hz Optional DP 4K 60Hz |  | 1 | 6 | 2 | 2/6 | 1*M.2 1*SATA Optional 1*MSATA | 2*NB-DDR4 MAX 32G | 12V/19V |
|  |  |  | 2 | 4 |  |  |  |  |  |
| PC model |  | IPC100U-QM9850 series |  |  |  |  |  |  |  |
| PC size |  | Dimensions with handle: 430mm(L)*301mm(W)*44mm(H) Dimensions with handle: 480mm(L)*301mm(W)*44mm(H) |  |  |  |  |  |  |  |
| Cooling system |  | With ventilation holes and silent fan cooling |  |  |  |  |  |  |  |
| color |  | black |  |  |  |  |  |  |  |
| Compatible motherboard models |  | QM9650, QM9750, QM9850 |  |  |  |  |  |  |  |
| motherboard |  | CPU | 6th generation: Intel® Celeron 3855U, 3955U and Sky Lake Core i3-6100U, i5-6200U, i7-6500U, etc.; 7th generation: Intel® Celeron 3865U, 3965U and Kaby Lake Core i3-7100U, i5-7200U, i7-7500U, etc.; 8th generation: Intel® Kaby Lake R Core i3-8130U, i5-8250U, i7-8550U, etc.; (See the next page for CPU list details) |  |  |  |  |  |  |
|  |  | graphics card | 6th generation: Intel® HD Graphics 510/520 graphics controller, supporting single display, dual display duplication, and dual display expansion; 7th generation: Intel® HD Graphics 610/620 graphics controller, supporting single display, dual display duplication, and dual display expansion; 8th generation: Intel® UHD Graphics 620 graphics controller, supporting single display, dual display duplication, and dual display expansion; |  |  |  |  |  |  |
|  |  | Memory | 2*NB-DDR4 memory supports 2133/2400MHz, with a maximum capacity of 32GB. |  |  |  |  |  |  |
| Front panel interface |  | POWER | 1* Switch button |  |  |  |  |  |  |
|  |  | RESET | 1* Restart Switch |  |  |  |  |  |  |
|  |  | led | 1 power indicator, 1 hard drive indicator |  |  |  |  |  |  |
|  |  | USB | 2 USB 2.0 ports |  |  |  |  |  |  |
| Rear panel interface |  | Power interface | 1 power interface, built-in DC power adapter, supports DC 12V, recommended power supply is 12V 7A or 19V 4A or higher. |  |  |  |  |  |  |
|  |  | Display Interface | 1*HDMI® 1.4 port (4096/3840*2160@30Hz) 1 VGA interface (1920*1080@60Hz) Optional 1*DP 1.4 interface (4096/3840*2160@60Hz) |  |  |  |  |  |  |
|  |  | LAN | Single gigabit network card: Realtek 8111H; Dual Gigabit Ethernet adapters: Realtek 8111H + Realtek 8111H; Supports Wake-on-LAN and PXE network functionality; |  |  |  |  |  |  |
|  |  | USB | 2 USB 3.0 ports 4 USB 2.0 ports (2 USB 2.0 ports for dual-network PC) |  |  |  |  |  |  |
|  |  | COM | 6 COM interfaces, supporting RS232 by default; COM1 can optionally support RS232/RS422/RS485. |  |  |  |  |  |  |
|  |  | audio | 1*LINE_OUT interface, audio output 1*MIC_IN interface, microphone input |  |  |  |  |  |  |
|  |  | WiFi | 2 WiFi antenna interfaces |  |  |  |  |  |  |
| storage |  | M.2 SSD | One M.2 Key-M slot, supporting M.2 2280 NVMe PCIe 3.0 x4 / SATA adaptive standard SSDs; optional M.2 2242 hard drive. |  |  |  |  |  |  |
|  |  | mSATA | Optional 1 mSATA slot, defaults to WiFi module, can be modified to support mSATA drives. |  |  |  |  |  |  |
|  |  | SATA | One SATA 3.0 hard drive interface, supporting 2.5-inch SSDs/HDDs or 3.5-inch HDDs. |  |  |  |  |  |  |
|  |  | Hard drive bays in the chassis | Two hard drive bays (one 2.5-inch SSD/HDD bay and one 3.5-inch HDD bay; this only indicates the number of hard drives the case can accommodate). |  |  |  |  |  |  |
| Extend |  | Mini-PCIe | One Mini-PCIe slot, supporting PCIe 3.0 x1 signal and WiFi module; optional support for WiFi, Bluetooth/4G modules. |  |  |  |  |  |  |
| Operating environment |  | Operating temperature | -10℃ ~ 60℃ |  |  |  |  |  |  |
|  |  | Storage temperature | -20℃ ~ 70℃ |  |  |  |  |  |  |
|  |  | relative humidity | 5%-95% relative humidity, non-condensing |  |  |  |  |  |  |
|  |  | Working vibration | 0.5grms/5-500HZ/random/operating |  |  |  |  |  |  |
| BIOS |  | Supports power-on upon startup, scheduled power-on, and remote power-on/off. |  |  |  |  |  |  |  |
| Watch Dog |  | Watchdog programming with support for hardware reset function (256 levels, 0~255 seconds). |  |  |  |  |  |  |  |
| operating system |  | 6th and 7th generation: Windows 10, Windows 7, Linux, etc. 8th generation: Windows 10, Linux, etc. |  |  |  |  |  |  |  |

※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※

The CPU information below is for reference only. Please consult our sales team for specific requirements!

| CPU Specifications | CPU Brand | CPU model | clock speed | Turbo Boost | TDP power consumption (W) | Full load power consumption is approximately (W) |
| --- | --- | --- | --- | --- | --- | --- |
| 6th generation | Cy Young | 3855U | Dual-core 1.6G |  | 15 | 40 |
|  |  | 3955U | Dual-core 2.0G |  | 15 | 40 |
| 6th generation i3 | Core | 6006U | Dual-core 2.0G |  | 15 | 40 |
|  |  | 6100U | Dual-core 2.3G |  | 15 | 40 |
| 6th generation i5 | Core | 6200U | Dual-core 2.3G | 2.8G | 15 | 40 |
|  |  | 6300U | Dual-core 2.4G | 3.0G | 15 | 40 |
| 6th generation i7 | Core | 6500U | Dual-core 2.5G | 3.1G | 15 | 40 |
|  |  | 6600U | Dual-core 2.6G | 3.4G | 15 | 40 |
|  |  | 6650U | Dual-core 2.2GHz | 3.4G | 15 | 40 |
| 7th generation i3 | Core | 7020U | Dual-core 2.3G |  | 15 | 40 |
|  |  | 7100U | Dual-core 2.4G |  | 15 | 40 |
|  |  | 7130U | Dual-core 2.7GHz |  | 15 | 40 |
| 7th Gen i5 | Core | 7200U | Dual-core 2.5G | 3.1G | 15 | 40 |
|  |  | 7300U | Dual-core 2.6G | 3.5G | 15 | 40 |
| 7th generation i7 | Core | 7500U | Dual-core 2.7GHz | 3.5G | 15 | 40 |
|  |  | 7600U | Dual-core 2.8G | 3.9G | 15 | 40 |
| 8th generation i3 | Core | 8130U | Dual-core 2.2GHz | 3.4G | 15 | 60 |
| 8th Gen i5 | Core | 8250U | Quad-core 1.6GHz | 3.4G | 15 | 60 |
|  |  | 8350U | Quad-core 1.7GHz | 3.6G | 15 | 60 |
| 8th Gen i7 | Core | 8550U | Quad-core 1.8GHz | 4.0G | 15 | 60 |
|  |  | 8650U | Quad-core 1.9GHz | 4.2G | 15 | 60 |

※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※

※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※

| HDMI®​ | VGA/DP |  | LAN | USB 2.0 | USB 3.2 | COM | SSD | Memory | Power |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 4K 30Hz | VGA 2K 60Hz Optional DP 4K 60Hz |  | 1 | 6 | 2 | 2/6 | 1*M.2 1/2*SATA Optional 1*MSATA | 2*NB-DDR4 MAX 64G | 12V/19V |
|  |  |  | 2 | 4 |  |  |  |  |  |
| PC model |  | IPC100U-8th and 10th Generation Series |  |  |  |  |  |  |  |
| PC size |  | Dimensions with handle: 430mm(L)*301mm(W)*44mm(H) Dimensions with handle: 480mm(L)*301mm(W)*44mm(H) |  |  |  |  |  |  |  |
| Cooling system |  | With ventilation holes and silent fan cooling |  |  |  |  |  |  |  |
| color |  | black |  |  |  |  |  |  |  |
| Compatible motherboard models |  | QM8H, QM9H, QM10H, QM8U, QM10U |  |  |  |  |  |  |  |
| motherboard |  | CPU | QM8H/QM8U: Intel® Whiskey Lake 8th generation Core i3-8145U, i5-8265U, i7-8565U, etc.; QM9H: Intel® Coffee Lake 8th generation Core i3-8109U, i5-8259U, i7-8559U, etc.; QM10H/QM10U: Intel® Comet Lake 10th generation Core i3-10110U, i5-10210U, i7-10510U, etc.; (See the next page for CPU list details) |  |  |  |  |  |  |
|  |  | graphics card | Intel® UHD Graphics graphics controller, supporting single display, dual display duplication, and dual display expansion; |  |  |  |  |  |  |
|  |  | Memory | 2*NB-DDR4 memory supports 2133/2400/2666MHz, with a maximum capacity of 64GB. |  |  |  |  |  |  |
| Front panel interface |  | POWER | 1* Switch button |  |  |  |  |  |  |
|  |  | RESET | 1* Restart Switch |  |  |  |  |  |  |
|  |  | led | 1 power indicator, 1 hard drive indicator |  |  |  |  |  |  |
|  |  | USB | 2 USB 2.0 ports |  |  |  |  |  |  |
| Rear panel interface |  | Power interface | 1 power interface, built-in DC power adapter, supports DC 12V, recommended power supply is 12V 7A or 19V 5A or higher. |  |  |  |  |  |  |
|  |  | Display Interface | 1*HDMI® 1.4 port (4096/3840*2160@30Hz) 1 VGA interface (1920*1080@60Hz) Optional 1*DP 1.4 interface (4096/3840*2160@60Hz) |  |  |  |  |  |  |
|  |  | LAN | Single Gigabit Ethernet adapter: Realtek 8111H Gigabit Ethernet adapter; optional Yutai Micro YT6801 Gigabit Ethernet adapter; Dual Gigabit Ethernet Adapters: Realtek 8111H + Realtek 8111H Gigabit Ethernet Adapter; Optional: YT6801 + YT6801 Gigabit Ethernet Adapter Supports Wake-on-LAN and PXE network functionality; |  |  |  |  |  |  |
|  |  | USB | 2 USB 3.2 Gen1 ports 4 USB 2.0 ports (2 USB 2.0 ports for dual-network PC) |  |  |  |  |  |  |
|  |  | COM | 6 COM interfaces, supporting RS232 by default; COM1 can optionally support RS232/RS422/RS485. |  |  |  |  |  |  |
|  |  | audio | 1*LINE_OUT interface, audio output 1*MIC_IN interface, microphone input |  |  |  |  |  |  |
|  |  | WiFi | 2 WiFi antenna interfaces |  |  |  |  |  |  |
| storage |  | M.2 SSD | One M.2 Key-M slot, supporting M.2 2280 NVMe PCIe 3.0 x4 / SATA adaptive standard SSDs; optional M.2 2242 hard drive. |  |  |  |  |  |  |
|  |  | mSATA | Optional 1 mSATA slot, defaults to WiFi module, can be modified to support mSATA drives. |  |  |  |  |  |  |
|  |  | SATA | 1/2 SATA 3.0 hard drive interfaces, supporting 2.5-inch SSDs/HDDs or 3.5-inch HDDs. |  |  |  |  |  |  |
|  |  | Hard drive bays in the chassis | Two hard drive bays (one 2.5-inch SSD/HDD bay and one 3.5-inch HDD bay; this only indicates the number of hard drives the case can accommodate). |  |  |  |  |  |  |
| Extend |  | Mini-PCIe | One Mini-PCIe slot, supporting PCIe 3.0 x1 signal and WiFi module; optional support for WiFi, Bluetooth/4G modules. |  |  |  |  |  |  |
|  |  | M.2_5G | One optional M.2 Key-B slot, supporting M.2 5G (3052/3042) modules. |  |  |  |  |  |  |
| Operating environment |  | Operating temperature | -10℃ ~ 60℃ |  |  |  |  |  |  |
|  |  | Storage temperature | -20℃ ~ 70℃ |  |  |  |  |  |  |
|  |  | relative humidity | 5%-95% relative humidity, non-condensing |  |  |  |  |  |  |
|  |  | Working vibration | 0.5grms/5-500HZ/random/operating |  |  |  |  |  |  |
| BIOS |  | Supports power-on upon startup, scheduled power-on, and remote power-on/off. |  |  |  |  |  |  |  |
| Watch Dog |  | Watchdog programming with support for hardware reset function (256 levels, 0~255 seconds). |  |  |  |  |  |  |  |
| operating system |  | Windows11, Windows 10, Linux, etc. |  |  |  |  |  |  |  |

※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※

The CPU information below is for reference only. Please consult our sales team for specific requirements!

| CPU Specifications | CPU Brand | CPU Model | clock speed | Turbo Boost | TDP power consumption | The full load power consumption of the entire board is approximately |
| --- | --- | --- | --- | --- | --- | --- |
|  | Cy Young | 4305U | Dual-core 2.2GHz | / | 15W | 60W |
| 8th generation i3 | Core | 8145U | Dual-core 2.1G | 3.9G | 15W | 60W |
| 8th Gen i5 | Core | 8365U | Quad-core 1.6GHz | 4.1G | 15W | 60W |
|  |  | 8265U | Quad-core 1.6GHz | 3.9G | 15W | 60W |
| 8th Gen i7 | Core | 8665U | Quad-core 1.9GHz | 4.8G | 15W | 60W |
|  |  | 8565U | Quad-core 1.8GHz | 4.6G | 15W | 60W |
| 8th generation i3 (QM9H) | Core | 8109U | Dual-core 3.0G | 3.6G | 28W | 70W |
|  |  | 8140U | Dual-core 2.1G | 3.9G | 15W | 60W |
| 8th generation i5 (QM9H) | Core | 8259U | Quad-core 2.3GHz | 3.8G | 28W | 70W |
|  |  | 8269U | Quad-core 2.6GHz | 4.2G | 28W | 70W |
|  |  | 8279U | Quad-core 2.4GHz | 4.1G | 28W | 70W |
|  |  | 8257U | Quad-core 1.4GHz | 3.9G | 15W | 60W |
|  |  | 8260U | Quad-core 1.6GHz | 3.9G | 15W | 60W |
| 8th Gen i7 (QM9H) | Core | 8559U | Quad-core 2.7GHz | 4.50 | 28W | 70W |
|  |  | 8569U | Quad-core 2.8GHz | 4.7G | 28W | 70W |
|  |  | 8557U | Quad-core 1.7GHz | 4.5G | 15W | 60W |
| 10th Gen i3 | Core | 10110U | Dual-core 2.1G | 4.10G | 15W | 60W |
| 10th Gen i5 | Core | 10310U | Quad-core 1.7GHz | 4.40G | 15W | 60W |
|  |  | 10210U | Quad-core 1.6GHz | 4.20G | 15W | 60W |
| 10th Gen i7 | Core | 10510U | Quad-core 1.8GHz | 4.90G | 15W | 60W |
|  |  | 10610U | Quad-core 1.8GHz | 4.90G | 15W | 60W |
|  |  | 10810U | Six-core 1.1GHz | 4.90G | 15W | 60W |
|  |  | 10710U | Six-core 1.1GHz | 4.70G | 15W | 60W |

※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※

※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※

| HDMI®​ | VGA/DP |  | LAN | USB 2.0 | USB 3.2 | COM | SSD | Memory | Power |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 4K 60Hz | VGA 2K 60Hz Optional DP 4K 60Hz |  | 1 | 6 | 2 | 2/6 | 1*M.2 1*SATA Optional 1*MSATA | 2*NB-DDR4 MAX 64G | 12V/19V |
|  |  |  | 2 | 4 |  |  |  |  |  |
| PC model |  | IPC100U-11th Generation Series |  |  |  |  |  |  |  |
| PC size |  | Dimensions with handle: 430mm(L)*301mm(W)*44mm(H) Dimensions with handle: 480mm(L)*301mm(W)*44mm(H) |  |  |  |  |  |  |  |
| Cooling system |  | With ventilation holes and silent fan cooling |  |  |  |  |  |  |  |
| color |  | black |  |  |  |  |  |  |  |
| Compatible motherboard models |  | QM11U, QM11H |  |  |  |  |  |  |  |
| motherboard |  | CPU | Supports Intel® Tiger Lake 11th Gen CPUs: i3-1115G4E, i5-1145G7E, i7-1185GRE, etc. (See the next page for a detailed list of CPU information) |  |  |  |  |  |  |
|  |  | graphics card | Intel® UHD Graphics / Intel Iris Xe Graphics controller, supporting single display, dual display duplication, and dual display expansion; |  |  |  |  |  |  |
|  |  | Memory | 2*NB-DDR4 memory supports 2133/2400/2666MHz, with a maximum capacity of 64GB. |  |  |  |  |  |  |
| Front panel interface |  | POWER | 1* Switch button |  |  |  |  |  |  |
|  |  | RESET | 1* Restart Switch |  |  |  |  |  |  |
|  |  | led | 1 power indicator, 1 hard drive indicator |  |  |  |  |  |  |
|  |  | USB | 2 USB 2.0 ports |  |  |  |  |  |  |
| Rear panel interface |  | Power interface | 1 power interface, built-in DC power adapter, supports DC 12V, recommended power supply is 12V 7A or 19V 5A or higher. |  |  |  |  |  |  |
|  |  | Display Interface | 1*HDMI® 2.0 port (4096/3840*2160@60Hz) 1 VGA interface (1920*1080@60Hz) Optional 1*DP 1.4 interface (4096/3840*2160@60Hz) |  |  |  |  |  |  |
|  |  | LAN | Single Gigabit Ethernet adapter: Intel i219V Gigabit Ethernet adapter; Dual Gigabit Ethernet adapters: Supports Intel i219V + Realtek 8111H Gigabit Ethernet adapter; Supports Wake-on-LAN and PXE network functionality; |  |  |  |  |  |  |
|  |  | USB | 2 USB 3.2 Gen1 ports 4 USB 2.0 ports (2 USB 2.0 ports for dual-network PC) |  |  |  |  |  |  |
|  |  | COM | 6 COM interfaces, supporting RS232 by default; COM1/COM2 can optionally support RS232/RS422/RS485. |  |  |  |  |  |  |
|  |  | audio | 1*LINE_OUT interface, audio output 1*MIC_IN interface, microphone input |  |  |  |  |  |  |
|  |  | WiFi | 2 WiFi antenna interfaces |  |  |  |  |  |  |
| storage |  | M.2 SSD | One M.2 Key-M slot, supporting M.2 2280 NVMe PCIe 3.0 x4 / SATA adaptive standard SSDs; optional M.2 2242 hard drive. |  |  |  |  |  |  |
|  |  | mSATA | Optional 1 mSATA slot, defaults to WiFi module, can be modified to support mSATA drives. |  |  |  |  |  |  |
|  |  | SATA | 1/2 SATA 3.0 hard drive interfaces, supporting 2.5-inch SSDs/HDDs or 3.5-inch HDDs. |  |  |  |  |  |  |
|  |  | Hard drive bays in the chassis | Two hard drive bays (one 2.5-inch SSD/HDD bay and one 3.5-inch HDD bay; this only indicates the number of hard drives the case can accommodate). |  |  |  |  |  |  |
| Extend |  | Mini-PCIe | One Mini-PCIe slot, supporting PCIe 3.0 x1 signal and WiFi module; optional support for WiFi, Bluetooth/4G modules. |  |  |  |  |  |  |
| Operating environment |  | Operating temperature | -10℃ ~ 60℃ |  |  |  |  |  |  |
|  |  | Storage temperature | -20℃ ~ 70℃ |  |  |  |  |  |  |
|  |  | relative humidity | 5%-95% relative humidity, non-condensing |  |  |  |  |  |  |
|  |  | Working vibration | 0.5grms/5-500HZ/random/operating |  |  |  |  |  |  |
| BIOS |  | Supports power-on upon startup, scheduled power-on, and remote power-on/off. |  |  |  |  |  |  |  |
| Watch Dog |  | Watchdog programming with support for hardware reset function (256 levels, 0~255 seconds). |  |  |  |  |  |  |  |
| operating system |  | Windows11, Windows 10, Linux, etc. |  |  |  |  |  |  |  |

※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※

The CPU information below is for reference only. Please consult our sales team for specific requirements!

| CPU Specifications | CPU Brand | CPU model | clock speed | Turbo Boost | TDP power consumption (W) |
| --- | --- | --- | --- | --- | --- |
|  | Cy Young | 6305E | Dual-core 2.8G | / | 15 |
| 11th Gen i3 | Core | 1115G4E | Dual-core 2.2GHz | 3.9G | 15 |
|  | Core | 1115GRE | Dual-core 2.2GHz | 3.9G | 15 |
|  | Core | 1115G4 | Dual-core 3.0G | 4.1G | 28 |
|  | Core | 1125G4 | Quad-core 2.0GHz | 3.7G | 28 |
| 11th Gen i5 | Core | 1145G7E | Quad-core 1.5GHz | 4.1G | 15 |
|  | Core | 1145GRE | Quad-core 1.5GHz | 4.1G | 15 |
|  | Core | 1145G7 | Quad-core 2.6GHz | 4.4G | 28 |
|  | Core | 1135G7 | Quad-core 2.4GHz | 4.2G | 28 |
|  | Core | 1155G7 | Quad-core 2.5GHz | 4.4G | 28 |
|  | Core | 11300H | Quad-core 3.1GHz | 4.4G | 35 |
|  | Core | 11320H | Quad-core 3.2GHz | 4.5G | 35 |
| 11th Gen i7 | Core | 1185GRE | Quad-core 1.8GHz | 4.4G | 15 |
|  | Core | 1185G7E | Quad-core 1.8GHz | 4.4G | 15 |
|  | Core | 1185G7 | Quad-core 3.0GHz | 4.8G | 28 |
|  | Core | 1165G7 | Quad-core 2.8GHz | 4.7G | 28 |
|  | Core | 1195G7 | Quad-core 2.9GHz | 5.0G | 28 |
|  | Core | 11370H | Quad-core 3.3GHz | 4.8G | 35 |
|  | Core | 11375H | Quad-core 3.3GHz | 5.0G | 35 |
|  | Core | 11390H | Quad-core 3.4GHz | 5.0G | 35 |

※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※

| HDMI®​ | VGA/DP |  | LAN | USB 2.0 | USB 3.2 | COM | SSD | Memory | Power |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 4K 60Hz | VGA 2K 60Hz Optional DP 4K 60Hz |  | 1 | 6 | 2 | 2/6 | 1*M.2 1*SATA | 2*NB-DDR4 MAX 64G | 12V/19V |
|  |  |  | 2 | 4 |  |  |  |  |  |
| PC model |  | IPC100U-12th and 13th generation series |  |  |  |  |  |  |  |
| PC size |  | Dimensions with handle: 430mm(L)*301mm(W)*44mm(H) Dimensions with handle: 480mm(L)*301mm(W)*44mm(H) |  |  |  |  |  |  |  |
| Cooling system |  | With ventilation holes and silent fan cooling |  |  |  |  |  |  |  |
| color |  | black |  |  |  |  |  |  |  |
| Compatible motherboard models |  | QM12U, QM13U, QM12P, QM13P, QM12H, QM13H |  |  |  |  |  |  |  |
| motherboard |  | CPU | Supports Intel® Alder Lake 12th generation U, P, and H series Core i3-1215U, i5-1235U, i7-1260P, i9-12900H, etc. Supports Intel® Raptor Lake 13th generation U, P, and H series Core i3-1305U, i5-1334U, i7-1340P, i9-13900H, etc. (See the next page for a complete CPU list) |  |  |  |  |  |  |
|  |  | graphics card | Intel® UHD Graphics / Intel® Iris Xe Graphics controller, supporting single display, dual display duplication, and dual display expansion. |  |  |  |  |  |  |
|  |  | Memory | 2*NB-DDR4 memory modules support 2133/2400/2666/2933/3200MHz, with a maximum capacity of 64GB. |  |  |  |  |  |  |
| Front panel interface |  | POWER | 1* Switch button |  |  |  |  |  |  |
|  |  | RESET | 1* Restart Switch |  |  |  |  |  |  |
|  |  | led | 1 power indicator, 1 hard drive indicator |  |  |  |  |  |  |
|  |  | USB | 2 USB 2.0 ports |  |  |  |  |  |  |
| Rear panel interface |  | Power interface | 1 power interface, built-in DC power adapter, supports DC 12V/19V. The maximum turbo frequency power consumption of U-series CPUs is 55W, and it is recommended to use a power supply with 12V 7A or 19V 5A or higher. The maximum turbo frequency power consumption of P-series CPUs is 64W, and it is recommended to use a power supply with 12V 7A or 19V 5A or higher. The maximum turbo frequency power consumption of H-series CPUs is 115W, and it is recommended to use a power supply of 12V 10A or 19V 6.3A or higher. |  |  |  |  |  |  |
|  |  | Display Interface | 1*HDMI® 2.0 port (4096/3840*2160@60Hz) 1 VGA interface (1920*1080@60Hz) Optional 1*DP 1.4 interface (4096/3840*2160@60Hz) |  |  |  |  |  |  |
|  |  | LAN | Single Gigabit Ethernet adapter: Intel i219V Gigabit Ethernet adapter; Dual Gigabit Ethernet Adapters: Intel i219V + Realtek 8111H Gigabit Ethernet Adapter Supports Wake-on-LAN and PXE network functionality; |  |  |  |  |  |  |
|  |  | USB | 2 USB 3.2 Gen1 ports 4 USB 2.0 ports (2 USB 2.0 ports for dual-network PC) |  |  |  |  |  |  |
|  |  | COM | 6 COM ports, supporting RS232 by default; COM1 can optionally support RS232/RS422/RS485. |  |  |  |  |  |  |
|  |  | audio | 1*LINE_OUT interface, audio output 1*MIC_IN interface, microphone input |  |  |  |  |  |  |
|  |  | WiFi | 2 WiFi antenna interfaces |  |  |  |  |  |  |
| storage |  | M.2 SSD | 1 M.2 Key-M slot, supporting M.2 2280 NVMe PCIe 4.0 x4 / SATA adaptive standard SSD; optional M.2 2242 hard drive; |  |  |  |  |  |  |
|  |  | SATA | One SATA 3.0 hard drive interface, supporting 2.5-inch SSD/HDD or 3.5-inch HDD. |  |  |  |  |  |  |
|  |  | Hard drive bays in the chassis | Two hard drive bays (one 2.5-inch SSD/HDD bay and one 3.5-inch HDD bay; this only indicates the number of hard drives the case can accommodate). |  |  |  |  |  |  |
| Extend |  | M.2 WiFi | 1 M.2 Key-E slot, supports M.2 2230 PCIe 3.0 x1 & USB, supports WiFi and Bluetooth modules. |  |  |  |  |  |  |
|  |  | M.2_5G | One M.2 Key-B slot, supporting M.2 5G (3052/3042) modules or M.2 4G (3042) modules. |  |  |  |  |  |  |
| Operating environment |  | Operating temperature | -10℃ ~ 60℃ |  |  |  |  |  |  |
|  |  | Storage temperature | -20℃ ~ 70℃ |  |  |  |  |  |  |
|  |  | relative humidity | 5%-95% relative humidity, non-condensing |  |  |  |  |  |  |
|  |  | Working vibration | 0.5grms/5-500HZ/random/operating |  |  |  |  |  |  |
| BIOS |  | Supports power-on upon startup, scheduled power-on, and remote power-on/off. |  |  |  |  |  |  |  |
| Watch Dog |  | Watchdog programming with support for hardware reset function (256 levels, 0~255 seconds). |  |  |  |  |  |  |  |
| operating system |  | Windows11, Windows 10, Linux, etc. |  |  |  |  |  |  |  |

※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※

The CPU information below is for reference only. Please consult our sales team for specific requirements!

| CPU Specifications | CPU Brand | CPU model | kernel/thread | clock speed | Turbo Boost | CPU base power consumption (W) | Maximum turbo power consumption (W) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 12th Gen i3 | Core | 1215U | Six cores and eight threads | 1.2G | 4.4G | 15 | 55 |
| 12th Gen i5 | Core | 1235U | Ten cores and twelve threads | 1.3G | 4.4G | 15 | 55 |
|  |  | 1245U | Ten cores and twelve threads | 1.6G | 4.4G | 15 | 55 |
| 12th Gen i7 | Core | 1265U | Ten cores and twelve threads | 1.8G | 4.8G | 15 | 55 |
|  |  | 1255U | Ten cores and twelve threads | 1.7G | 4.7G | 15 | 55 |
| 13th generation i3 | Core | 1305U | Five cores and six threads | 1.6G | 4.5G | 15 | 55 |
|  |  | 1315U | Six cores and eight threads | 1.2G | 4.5G | 15 | 55 |
| 13th Gen i5 | Core | 1334U | Ten cores and twelve threads | 1.3G | 4.6G | 15 | 55 |
|  |  | 1335U | Ten cores and twelve threads | 1.3G | 4.6G | 15 | 55 |
|  |  | 1345U | Ten cores and twelve threads | 1.6G | 4.7G | 15 | 55 |
| 13th Gen i7 | Core | 1355U | Ten cores and twelve threads | 1.7G | 5.0G | 15 | 55 |
|  |  | 1365U | Ten cores and twelve threads | 1.8G | 5.2G | 15 | 55 |
| 12th Gen i3 | Core | 1220P | Ten cores and twelve threads | 1.5G | 4.4G | 28 | 64 |
| 12th Gen i5 | Core | 1240P | Twelve cores and sixteen threads | 1.7G | 4.4G | 28 | 64 |
|  |  | 1250P | Twelve cores and sixteen threads | 1.7G | 4.4G | 28 | 64 |
| 12th Gen i7 | Core | 1280P | Fourteen cores and twenty threads | 1.8G | 4.8G | 28 | 64 |
|  |  | 1260P | Twelve cores and sixteen threads | 2.1G | 4.7G | 28 | 64 |
|  |  | 1270P | Twelve cores and sixteen threads | 2.2G | 4.8G | 28 | 64 |
| 13th Gen i5 | Core | 1340P | Twelve cores and sixteen threads | 1.8G | 4.6G | 28 | 64 |
|  |  | 1350P | Twelve cores and sixteen threads | 1.9G | 4.7G | 28 | 64 |
| 13th Gen i7 | Core | 1360P | Twelve cores and sixteen threads | 2.2G | 5.0G | 28 | 64 |
|  |  | 1370P | Fourteen cores and twenty threads | 1.9G | 5.2G | 28 | 64 |
| 12th Gen i5 | Core | 12450H | Eight cores and twelve threads | 2.0G | 4.4G | 45 | 95 |
|  |  | 12500H | Twelve cores and sixteen threads | 2.5G | 4.5G | 45 | 95 |
|  |  | 12600H | Twelve cores and sixteen threads | 2.7G | 4.5G | 45 | 95 |
| 12th Gen i7 | Core | 12800H | Fourteen cores and twenty threads | 2.4G | 4.8G | 45 | 115 |
|  |  | 12700H | Fourteen cores and twenty threads | 2.7G | 4.7G | 45 | 115 |
|  |  | 12650H | Ten cores and sixteen threads | 2.3G | 4.7G | 45 | 115 |
| 12th Gen i9 | Core | 12900H | Fourteen cores and twenty threads | 2.5G | 5.0G | 45 | 115 |
| 13th Gen i5 | Core | 13420H | Eight cores and twelve threads | 2.1G | 4.60G | 45 | 95 |
|  |  | 13500H | Twelve cores and sixteen threads | 2.6G | 4.70G | 45 | 95 |
|  |  | 13600H | Twelve cores and sixteen threads | 2.8G | 4.80G | 45 | 95 |
| 13th Gen i7 | Core | 13620H | Ten cores and sixteen threads | 2.4G | 4.90G | 45 | 115 |
|  |  | 13700H | Fourteen cores and twenty threads | 2.4G | 5.00G | 45 | 115 |
|  |  | 13800H | Fourteen cores and twenty threads | 2.5G | 5.20G | 45 | 115 |
| 13th generation i9 | Core | 13900H | Fourteen cores and twenty threads | 2.6G | 5.40G | 45 | 115 |

| CPU brand | CPU Specification | CPU model | kernel/thread | Performance core (P core) |  | Energy efficiency core (E core) |  | CPU base power consumption (W) | CPU maximum turbo frequency power consumption (W) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Basic frequency | Turbo Boost | Basic frequency | Turbo Boost |  |  |
| Intel | Core 3 | 100U | Six cores and eight threads | 1.2G | 4.7G | 900M | 3.3G | 15 | 55 |
| Intel | Core 5 | 120U | Ten cores and twelve threads | 1.4G | 5.0G | 900M | 3.8G | 15 | 55 |
|  |  | 220U | Ten cores and twelve threads | 1.4G | 5.0G | 900M | 3.8G | 15 | 55 |
|  |  | 210H | Eight cores and twelve threads | 2.2G | / | 1.6G | 3.6G | 45 | 115 |
|  |  | 220H | Twelve cores and sixteen threads | 2.7G | / | 2.0G | 3.7G | 45 | 115 |
| Intel | Core 7 | 150U | Ten cores and twelve threads | 1.8G | 5.4G | 1.2G | 4.0G | 15 | 55 |
|  |  | 250U | Ten cores and twelve threads | 1.8G | 5.4G | 1.2G | 4.0G | 15 | 55 |
|  |  | 240H | Ten cores and sixteen threads | 2.5G | 5.2G | 1.8G | 4.0G | 45 | 115 |
|  |  | 250H | Fourteen cores and twenty threads | 2.5G | 5.4G | 1.8G | 4.0G | 45 | 115 |
| Intel | Core 9 | 270H | Fourteen cores and twenty threads | 2.7G | 5.8G | 2.0G | 4.1G | 45 | 115 |

※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※

※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※

| HDMI®​ | VGA/DP |  | LAN | USB 2.0 | USB 3.2 | COM | SSD | Memory | Power |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 4K 60Hz | VGA 2K 60Hz Optional DP 8K 60Hz |  | 1 | 6 | 2 | 2/6 | 1*M.2 1*SATA | Onboard LPDDR4 16G MAX 64G | 12V/19V |
|  |  |  | 2 | 4 |  |  |  |  |  |
| PC model |  | IPC100U -QM12M |  |  |  |  |  |  |  |
| PC size |  | Dimensions with handle: 430mm(L)*301mm(W)*44mm(H) Dimensions with handle: 480mm(L)*301mm(W)*44mm(H) |  |  |  |  |  |  |  |
| Cooling system |  | With ventilation holes and silent fan cooling |  |  |  |  |  |  |  |
| color |  | black |  |  |  |  |  |  |  |
| Compatible motherboard models |  | QM12M |  |  |  |  |  |  |  |
| motherboard |  | CPU | Supports Intel® Alder Lake-M 12th generation: Intel Pentium 8200U: Five cores, 1.0GHz, turbo boost up to 4.4GHz; Intel Core i3-1210U, six cores, 1.0GHz, turbo boost up to 4.4GHz; Intel Core i5-1230U: 10 cores, 1.0GHz, turbo boost up to 4.4GHz; Intel Core i5-1240U: 10 cores, 1.1GHz, turbo boost up to 4.4GHz; Intel Core i7-1250U: 10 cores, 1.1GHz, turbo boost up to 4.7GHz; Intel Core i7-1260U: 10 cores, 1.1GHz, turbo boost up to 4.7GHz; |  |  |  |  |  |  |
|  |  | graphics card | Intel® UHD Graphics / Intel Iris Xe Graphics integrated graphics, supporting single display, dual display duplication, and dual display expansion. |  |  |  |  |  |  |
|  |  | Memory | It supports 16GB of onboard LPDDR4 4000MHz by default, with an optional maximum support of 64GB 4267MHz. |  |  |  |  |  |  |
| Front panel interface |  | POWER | 1* Switch button |  |  |  |  |  |  |
|  |  | RESET | 1* Restart Switch |  |  |  |  |  |  |
|  |  | led | 1 power indicator, 1 hard drive indicator |  |  |  |  |  |  |
|  |  | USB | 2 USB 2.0 ports |  |  |  |  |  |  |
| Rear panel interface |  | Power interface | 1 power interface, built-in DC power adapter, supports DC 12V/19V, recommended to use a 12V 5A or 19V 3A or higher power supply. |  |  |  |  |  |  |
|  |  | Display Interface | 1*HDMI® 2.0 port (4096/3840*2160@60Hz) 1 VGA interface (1920*1080@60Hz) Optional 1*DP interface (7680 x 2160 @ 60Hz) |  |  |  |  |  |  |
|  |  | LAN | Single Gigabit Ethernet adapter: Realtek 8111H Gigabit Ethernet adapter, with optional support for Yutai Micro YT6801 Gigabit Ethernet adapter; Dual Gigabit Ethernet adapters: Realtek 8111H + Realtek 8111H Gigabit Ethernet adapter; optional support for YT6801 + YT6801 Gigabit Ethernet adapter; Supports Wake-on-LAN and PXE network functionality; |  |  |  |  |  |  |
|  |  | USB | 2 USB 3.2 Gen1 ports 4 USB 2.0 ports (2 USB 2.0 ports for dual-network PC) |  |  |  |  |  |  |
|  |  | COM | 6 COM ports, supporting RS232 by default; COM1 can optionally support RS232/RS422/RS485. |  |  |  |  |  |  |
|  |  | audio | 1*LINE_OUT interface, audio output 1*MIC_IN interface, microphone input |  |  |  |  |  |  |
|  |  | WiFi | 2 WiFi antenna interfaces |  |  |  |  |  |  |
| storage |  | M.2 SSD | 1 M.2 Key-M slot, supporting M.2 2280 NVMe PCIe 4.0 x4 / SATA adaptive standard SSD; optional M.2 2242 hard drive; |  |  |  |  |  |  |
|  |  | SATA | One SATA 3.0 hard drive interface, supporting 2.5-inch SSD/HDD or 3.5-inch HDD. |  |  |  |  |  |  |
|  |  | Hard drive bays in the chassis | Two hard drive bays (one 2.5-inch SSD/HDD bay and one 3.5-inch HDD bay; this only indicates the number of hard drives the case can accommodate). |  |  |  |  |  |  |
| Extend |  | M.2 WiFi | 1 M.2 Key-E slot, supports M.2 2230 PCIe 3.0 x1 & USB, supports WiFi and Bluetooth modules. |  |  |  |  |  |  |
|  |  | M.2_5G | One M.2 Key-B slot, supporting M.2 5G (3052/3042) modules or M.2 4G (3042) modules. |  |  |  |  |  |  |
| Operating environment |  | Operating temperature | -10℃ ~ 60℃ |  |  |  |  |  |  |
|  |  | Storage temperature | -20℃ ~ 70℃ |  |  |  |  |  |  |
|  |  | relative humidity | 5%-95% relative humidity, non-condensing |  |  |  |  |  |  |
|  |  | Working vibration | 0.5grms/5-500HZ/random/operating |  |  |  |  |  |  |
| BIOS |  | Supports power-on upon startup, scheduled power-on, and remote power-on/off. |  |  |  |  |  |  |  |
| Watch Dog |  | Watchdog programming with support for hardware reset function (256 levels, 0~255 seconds). |  |  |  |  |  |  |  |
| operating system |  | Windows11, Windows 10, Linux, etc. |  |  |  |  |  |  |  |

※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※

| HDMI®​ | VGA/DP |  |  | LAN | USB 2.0 | USB 3.2 | COM | SSD | Memory | Power |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 4K 60Hz | VGA 2K 60Hz Optional DP 8K 120Hz |  |  | 1 | 6 | 2 | 2/6 | 1*M.2 1*SATA | 2*NB-DDR4 MAX 96G | 12V/19V |
|  |  |  |  | 2 | 4 |  |  |  |  |  |
| PC model |  | IPC100U-14th Generation Series |  |  |  |  |  |  |  |  |
| PC size |  | Dimensions with handle: 430mm(L)*301mm(W)*44mm(H) Dimensions with handle: 480mm(L)*301mm(W)*44mm(H) |  |  |  |  |  |  |  |  |
| Cooling system |  | With ventilation holes and silent fan cooling |  |  |  |  |  |  |  |  |
| color |  | black |  |  |  |  |  |  |  |  |
| Compatible motherboard models |  | QM14U, QM14H, QM15U, 1M15H |  |  |  |  |  |  |  |  |
| motherboard |  | CPU | Supports Intel® Meteor Lake 14th generation U/H series Core processors: Ultra 5-125U, Ultra 7-155U, Ultra 7-155H, Ultra 9-185H, etc. Supports Intel® Arrow Lake 15th generation U/H series Core processors such as Ultra 5-225U, Ultra 7-255U, and Ultra 9-285H. (See the next page for a complete CPU list) |  |  |  |  |  |  |  |
|  |  | graphics card | Integrated Intel® Graphics /Intel Arc Graphics hyper-graphics card, supporting single display, dual display duplication, and dual display expansion. |  |  |  |  |  |  |  |
|  |  | Memory | 2*NB-DDR5 memory modules support 4800/5600MT/s, with a maximum capacity of 96GB. |  |  |  |  |  |  |  |
| Front panel interface |  | POWER | 1* Switch button |  |  |  |  |  |  |  |
|  |  | RESET | 1* Restart Switch |  |  |  |  |  |  |  |
|  |  | led | 1 power indicator, 1 hard drive indicator |  |  |  |  |  |  |  |
|  |  | USB | 2 USB 2.0 ports |  |  |  |  |  |  |  |
| Rear panel interface |  | Power interface | 1 power interface, built-in DC power adapter, supports DC 12V/19V. The maximum turbo frequency power consumption of U-series CPUs is 57W, and it is recommended to use a power supply with 12V 7A or 19V 5A or higher. The maximum turbo frequency power consumption of H-series CPUs is 115W, and it is recommended to use a power supply of 12V 10A or 19V 6.3A or higher. |  |  |  |  |  |  |  |
|  |  | Display Interface | 1*HDMI® 2.0 port (4096/3840*2160@60Hz) 1 VGA interface (1920*1080@60Hz) Optional 1*DP interface (7680*2160@120Hz) |  |  |  |  |  |  |  |
|  |  | LAN | Single Gigabit Ethernet adapter: Intel® i219V Gigabit Ethernet adapter; Dual Gigabit Ethernet adapters: Intel® i219V + Realtek 8111H Gigabit Ethernet adapter, with optional support for Intel i219V + Yutai Micro YT6801 Gigabit Ethernet adapter; Supports Wake-on-LAN and PXE network functionality; |  |  |  |  |  |  |  |
|  |  | USB | 2 USB 3.2 Gen1 ports 4 USB 2.0 ports (2 USB 2.0 ports for dual-network PC) |  |  |  |  |  |  |  |
|  |  | COM | 6 COM interfaces, supporting RS232 by default; COM1 can optionally support RS232/RS422/RS485. |  |  |  |  |  |  |  |
|  |  | audio | 1*LINE_OUT interface, audio output 1*MIC_IN interface, microphone input |  |  |  |  |  |  |  |
|  |  | WiFi | 2 WiFi antenna interfaces |  |  |  |  |  |  |  |
| storage |  | M.2 SSD | 1 M.2 Key-M slot, supporting M.2 2280 NVMe PCIe 4.0 x4 / SATA adaptive standard SSD; optional M.2 2242 hard drive; |  |  |  |  |  |  |  |
|  |  | SATA | One SATA 3.0 hard drive interface, supporting 2.5-inch SSD/HDD or 3.5-inch HDD. |  |  |  |  |  |  |  |
|  |  | Hard drive bays in the chassis | Two hard drive bays (one 2.5-inch SSD/HDD bay and one 3.5-inch HDD bay; this only indicates the number of hard drives the case can accommodate). |  |  |  |  |  |  |  |
| Extend |  | M.2 WiFi | 1 M.2 Key-E slot, supports M.2 2230 PCIe 4.0 x1 & USB, supports WiFi and Bluetooth modules. |  |  |  |  |  |  |  |
|  |  | M.2_5G | One M.2 Key-B slot, supporting M.2 5G (3052/3042) modules or M.2 4G (3042) modules. |  |  |  |  |  |  |  |
| Operating environment |  | Operating temperature | -10℃ ~ 60℃ |  |  |  |  |  |  |  |
|  |  | Storage temperature | -20℃ ~ 70℃ |  |  |  |  |  |  |  |
|  |  | relative humidity | 5%-95% relative humidity, non-condensing |  |  |  |  |  |  |  |
|  |  | Working vibration | 0.5grms/5-500HZ/random/operating |  |  |  |  |  |  |  |
| BIOS |  | Supports power-on upon startup, scheduled power-on, and remote power-on/off. |  |  |  |  |  |  |  |  |
| Watch Dog |  | Watchdog programming with support for hardware reset function (256 levels, 0~255 seconds). |  |  |  |  |  |  |  |  |
| operating system |  | Windows11, Windows 10, Linux, etc. |  |  |  |  |  |  |  |  |

※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※

※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※

The CPU information below is for reference only. Please consult our sales team for specific requirements!

| CPU brand | CPU Specification | CPU model | kernel/thread | Performance core (P core) |  | Energy efficiency core (E core) |  | CPU base power consumption (W) | CPU maximum turbo frequency power consumption (W) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Basic frequency | Turbo Boost | Basic frequency | Turbo Boost |  |  |
| Intel Core | 14th generation Ultra 5 | 125U | Twelve cores and fourteen threads | 1.3G | 4.3G | 800M | 3.6G | 15 | 57 |
|  |  | 135U | Twelve cores and fourteen threads | 1.6G | 4.4G | 1.1G | 3.6G | 15 | 57 |
|  |  | 125H | Fourteen cores and eighteen threads | 1.2G | 4.5G | 700M | 3.6G | 28 | 115 |
|  |  | 135H | Fourteen cores and eighteen threads | 1.7G | 4.6G | 1.2G | 3.6G | 28 | 115 |
| Intel Core | 14th generation Ultra 7 | 155U | Twelve cores and fourteen threads | 1.7G | 4.8G | 1.2G | 3.8G | 15 | 57 |
|  |  | 165U | Twelve cores and fourteen threads | 1.7G | 4.9G | 1.2G | 3.8G | 15 | 57 |
|  |  | 155H | Sixteen cores and twenty-two threads | 1.4G | 4.8G | 900M | 3.8G | 28 | 115 |
|  |  | 165H | Sixteen cores and twenty-two threads | 1.4G | 5.0G | 900M | 3.8G | 28 | 115 |
| Intel Core | 14th generation Ultra 9 | 185H | Sixteen cores and twenty-two threads | 2.3G | 5.1G | 1.8G | 3.8G | 45 | 115 |
| Intel Core | 15th generation Ultra 5 | 225U | Twelve cores and fourteen threads | 1.5G | 4.8G | 1.3G | 3.8G | 15 | 57 |
|  |  | 235U | Twelve cores and fourteen threads | 2.0G | 4.9G | 1.6G | 4.1G | 15 | 57 |
|  |  | 225H | Fourteen cores and fourteen threads | 1.7G | 4.9G | 1.3G | 4.3G | 28 | 115 |
|  |  | 235H | Fourteen cores and fourteen threads | 2.4G | 5.0G | 1.8G | 4.4G | 28 | 115 |
| Intel Core | 15th generation Ultra 7 | 255U | Twelve cores and fourteen threads | 2.0G | 5.2G | 1.7G | 4.2G | 15 | 57 |
|  |  | 265U | Twelve cores and fourteen threads | 2.1G | 5.3G | 1.7G | 4.2G | 15 | 57 |
|  |  | 255H | Sixteen cores and sixteen threads | 2.2G | 5.1G | 1.5G | 4.4G | 28 | 115 |
|  |  | 265H | Sixteen cores and sixteen threads | 2.2G | 5.3G | 1.7G | 4.5G | 28 | 115 |
| Intel Core | 15th generation Ultra 9 | 285H | Sixteen cores and sixteen threads | 2.9G | 5.4G | 2.7G | 4.5G | 45 | 115 |

※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※

※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※

| HDMI®​ | VGA |  |  | LAN | USB 2.0 | USB 3.0 | COM | SSD | Memory | Power |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 4K 30Hz | VGA 2K 60Hz |  |  | 1 | 3 | 4 | 2/6 | 1*M.2 2*SATA Optional 1*MSATA | 1*NB-DDR4 MAX 16G | 12V/19V |
|  |  |  |  | 2 | 2 |  |  |  |  |  |
| PC model |  | IPC100U-QM1100 |  |  |  |  |  |  |  |  |
| PC size |  | Dimensions with handle: 430mm(L)*301mm(W)*44mm(H) Dimensions with handle: 480mm(L)*301mm(W)*44mm(H) |  |  |  |  |  |  |  |  |
| Cooling system |  | With ventilation holes and silent fan cooling |  |  |  |  |  |  |  |  |
| color |  | black |  |  |  |  |  |  |  |  |
| Compatible motherboard models |  | QM1100 |  |  |  |  |  |  |  |  |
| motherboard |  | CPU | Supports Intel® SkyLake 6th generation LGA1151 Core i3-6100, i5-6400, i7-6700, etc. Supports Intel® Kaby Lake 7th generation LGA1151 Core i3-7100, i5-7400, i7-7700, etc. (See the next page for a complete CPU list) |  |  |  |  |  |  |  |
|  |  | graphics card | 6th generation: Intel® HD Graphics 530 graphics controller, supporting single display, dual display expansion, and dual display duplication. 7th generation: Intel® HD Graphics 630 graphics controller, supporting single display, dual display expansion, and dual display duplication. |  |  |  |  |  |  |  |
|  |  | Memory | 1*NB-DDR4 supports 2133/2400MHz memory, with a maximum support of 16GB. |  |  |  |  |  |  |  |
| Front panel interface |  | POWER | 1* Switch button |  |  |  |  |  |  |  |
|  |  | RESET | 1* Restart Switch |  |  |  |  |  |  |  |
|  |  | led | 1 power indicator, 1 hard drive indicator |  |  |  |  |  |  |  |
|  |  | USB | 2 USB 3.0 ports |  |  |  |  |  |  |  |
| Rear panel interface |  | Power interface | 1 power interface, built-in DC power adapter, supports DC 12V/19V, recommended power supply is 12V 10A or 19V 6A or higher. |  |  |  |  |  |  |  |
|  |  | Display Interface | 1*HDMI® 1.4 port (4096/3840*2160@30Hz) 1 VGA interface (1920*1080@60Hz) |  |  |  |  |  |  |  |
|  |  | LAN | Single Gigabit Ethernet adapter: Realtek 8111F Gigabit Ethernet adapter; Dual Gigabit Ethernet adapters: Realtek 8111F + Realtek 8111F Gigabit Ethernet adapter; Supports Wake-on-LAN and PXE network functionality; |  |  |  |  |  |  |  |
|  |  | USB | 2 USB 3.0 ports 3 USB 2.0 ports (dual-network motherboards support 2 USB 2.0 ports) |  |  |  |  |  |  |  |
|  |  | COM | 6 COM ports, supporting RS232 by default; COM1 can optionally support RS232/RS422/RS485. |  |  |  |  |  |  |  |
|  |  | audio | 1*LINE_OUT interface, audio output 1*MIC_IN interface, microphone input |  |  |  |  |  |  |  |
|  |  | WiFi | 2 WiFi antenna interfaces |  |  |  |  |  |  |  |
| storage |  | M.2 SSD | 1 M.2 Key-M slot, supporting M.2 2280 SATA protocol, with optional M.2 NVMe PCIe 3.0 x4 protocol; optional M.2 2242 hard drive; |  |  |  |  |  |  |  |
|  |  | mSATA | Optional 1 mSATA slot, defaults to WiFi module, can be modified to support mSATA drives. |  |  |  |  |  |  |  |
|  |  | SATA | 2 SATA 3.0 hard drive interfaces, supporting 2.5-inch SSDs/HDDs or 3.5-inch HDDs. |  |  |  |  |  |  |  |
|  |  | Hard drive bays in the chassis | Two hard drive bays (one 2.5-inch SSD/HDD bay and one 3.5-inch HDD bay; this only indicates the number of hard drives the case can accommodate). |  |  |  |  |  |  |  |
| Extend |  | Mini-PCIe | 1 Mini-PCIe slot, supports PCIe 3.0 x1 & USB signal, supports WiFi, Bluetooth/4G modules. |  |  |  |  |  |  |  |
| Operating environment |  | Operating temperature | -10℃ ~ 60℃ |  |  |  |  |  |  |  |
|  |  | Storage temperature | -20℃ ~ 70℃ |  |  |  |  |  |  |  |
|  |  | relative humidity | 5%-95% relative humidity, non-condensing |  |  |  |  |  |  |  |
|  |  | Working vibration | 0.5grms/5-500HZ/random/operating |  |  |  |  |  |  |  |
| BIOS |  | Supports power-on upon startup, scheduled power-on, and remote power-on/off. |  |  |  |  |  |  |  |  |
| Watch Dog |  | Watchdog programming with support for hardware reset function (256 levels, 0~255 seconds). |  |  |  |  |  |  |  |  |
| operating system |  | Supports Windows 10, Windows 7, Linux, etc. |  |  |  |  |  |  |  |  |

※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※

The CPU information below is for reference only. Please consult our sales team for specific requirements!

| CPU Specifications | CPU Brand | CPU model | clock speed | Turbo Boost | CPU power consumption is approximately (W) | CPU full load power consumption (W) |
| --- | --- | --- | --- | --- | --- | --- |
| 6th generation i3 | Intel Core | 6100 | Dual-core 3.7G |  | 65 | 90 |
|  |  | 6300 | Dual-core 3.8G |  | 65 | 90 |
|  |  | 6320 | Dual-core 3.9G |  | 65 | 90 |
| 6th generation i5 | Intel Core | 6400 | Quad-core 2.7GHz | 3.3G | 65 | 90 |
|  |  | 6500 | Quad-core 3.2GHz | 3.6G | 65 | 90 |
|  |  | 6600 | Quad-core 3.3GHz | 3.9G | 65 | 90 |
| 6th generation i7 | Intel Core | 6700 | Quad-core 3.4GHz | 4.0G | 65 | 90 |
| 7th generation i3 | Intel Core | 7100 | Dual-core 3.9G |  | 65 | 90 |
|  |  | 7300 | Dual-core 4.0G |  | 65 | 90 |
|  |  | 7320 | Dual-core 4.1G |  | 65 | 90 |
| 7th Gen i5 | Intel Core | 7400 | Quad-core 3.0GHz | 3.5G | 65 | 90 |
|  |  | 7500 | Quad-core 3.4GHz | 3.8G | 65 | 90 |
|  |  | 7600 | Quad-core 3.5GHz | 4.1G | 65 | 90 |
| 7th generation i7 | Intel Core | 7700 | Quad-core 3.6GHz | 4.2G | 65 | 90 |

※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※

※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※

| HDMI® | VGA/DP |  |  | LAN | USB2.0 | USB3.0 | COM | SSD | Memory | Power |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 4K 30Hz | VGA 2K 60Hz Optional DP 4K 60Hz |  |  | 1 | 6 | 2 | 2/6 | 1*MSATA 1*SATA | 2*NB-DDR4 MAX 64G | 12V/19V |
|  |  |  |  | 2 | 4 |  |  |  |  |  |
| PC model |  | IPC100U-H310 series |  |  |  |  |  |  |  |  |
| PC size |  | Dimensions with handle: 430mm(L)*301mm(W)*44mm(H) Dimensions with handle: 480mm(L)*301mm(W)*44mm(H) |  |  |  |  |  |  |  |  |
| Cooling system |  | With ventilation holes and silent fan cooling |  |  |  |  |  |  |  |  |
| color |  | black |  |  |  |  |  |  |  |  |
| Compatible motherboard models |  | QM3100, QM3600 |  |  |  |  |  |  |  |  |
| motherboard |  | CPU | Supports Intel® Skylate 6th generation LGA1151 Core i3-6100, i5-6400, i7-6700, etc. Supports Intel® Kaby Lake 7th generation LGA1151 Core i3-7100, i5-7400, i7-7700, etc. Supports Intel® Coffee Lake 8th generation LGA1151 Core i3-8100, i5-8400, i7-8700, etc. Supports Intel® Coffee Lake 9th generation LGA1151 Core i3-9100, i5-9400, i7-9700, i9-9900, etc. (See the next page for a complete list of CPUs) |  |  |  |  |  |  |  |
|  |  | graphics card | 6th generation: Intel® HD Graphics 530 graphics controller, supporting single display, dual display expansion, and dual display duplication. 7th generation: Intel® HD Graphics 630 graphics controller, supporting single display, dual display expansion, and dual display duplication. 8th and 9th generation: Intel® UHD Graphics 630 graphics controller, supporting single display, dual display expansion, and dual display duplication. |  |  |  |  |  |  |  |
|  |  | Memory | 2*NB-DDR4 memory modules support 2133/2400/2666MHz, with a maximum capacity of 64GB. |  |  |  |  |  |  |  |
| Front panel interface |  | POWER | 1* Switch button |  |  |  |  |  |  |  |
|  |  | RESET | 1* Restart Switch |  |  |  |  |  |  |  |
|  |  | led | 1 power indicator, 1 hard drive indicator |  |  |  |  |  |  |  |
|  |  | USB | 2 USB 2.0 ports |  |  |  |  |  |  |  |
| Rear panel interface |  | Power interface | 1 power interface, built-in DC power adapter, supports DC 12V/19V, recommended power supply is 12V 10A or 19V 6A or higher. |  |  |  |  |  |  |  |
|  |  | Display Interface | 1*HDMI® 1.4 port (4096/3840*2160@30Hz) 1 VGA interface (1920*1080@60Hz) Optional 1*DP 1.4 interface (4096/3840*2160@60Hz) |  |  |  |  |  |  |  |
|  |  | LAN | Single Gigabit Ethernet adapter: Realtek 8111H Gigabit Ethernet adapter, optional Yutai Micro YT6801 Gigabit Ethernet adapter; Dual Gigabit Ethernet adapters: Realtek 8111H + Realtek 8111H Gigabit Ethernet adapter, optional Yutai Micro YT6801 + Yutai Micro YT6801 Gigabit Ethernet adapter; Supports Wake-on-LAN and PXE network functionality; |  |  |  |  |  |  |  |
|  |  | USB | 2 USB 3.0 ports 4 USB 2.0 ports (2 USB 2.0 ports for dual-network PC) |  |  |  |  |  |  |  |
|  |  | COM | 6 COM ports, supporting RS232 by default; COM1 can optionally support RS232/RS422/RS485. |  |  |  |  |  |  |  |
|  |  | audio | 1*LINE_OUT interface, audio output 1*MIC_IN interface, microphone input |  |  |  |  |  |  |  |
|  |  | WiFi | 2 WiFi antenna interfaces |  |  |  |  |  |  |  |
| storage |  | mSATA | 1 mSATA slot, supports mSATA drives |  |  |  |  |  |  |  |
|  |  | SATA | One SATA 3.0 hard drive interface, supporting 2.5-inch SSDs/HDDs or 3.5-inch HDDs. |  |  |  |  |  |  |  |
|  |  | Hard drive bays in the chassis | Two hard drive bays (one 2.5-inch SSD/HDD bay and one 3.5-inch HDD bay; this only indicates the number of hard drives the case can accommodate). |  |  |  |  |  |  |  |
| Extend |  | Mini-PCIe | 1 Mini-PCIe slot, supports PCIe 3.0 x1 & USB signal, supports WiFi, Bluetooth/4G modules. |  |  |  |  |  |  |  |
| Operating environment |  | Operating temperature | -10℃ ~ 60℃ |  |  |  |  |  |  |  |
|  |  | Storage temperature | -20℃ ~ 70℃ |  |  |  |  |  |  |  |
|  |  | relative humidity | 5%-95% relative humidity, non-condensing |  |  |  |  |  |  |  |
|  |  | Working vibration | 0.5grms/5-500HZ/random/operating |  |  |  |  |  |  |  |
| BIOS |  | Supports power-on upon startup, scheduled power-on, and remote power-on/off. |  |  |  |  |  |  |  |  |
| Watch Dog |  | Watchdog programming with support for hardware reset function (256 levels, 0~255 seconds). |  |  |  |  |  |  |  |  |
| operating system |  | 6th and 7th generation motherboards support: Windows 11, Windows 10, Windows 8, Windows 7, Linux, etc. 8th and 9th generation motherboards support: Windows 11, Windows 10, Linux, etc. |  |  |  |  |  |  |  |  |

※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※

The CPU information below is for reference only. Please consult our sales team for specific requirements!

| CPU Specifications | CPU Brand | CPU model | clock speed | Turbo Boost | CPU power consumption is approximately (W) | CPU full load power consumption (W) |
| --- | --- | --- | --- | --- | --- | --- |
| 6th generation i3 | Intel Core | 6100 | Dual-core 3.7G |  | 65 | 90 |
|  |  | 6300 | Dual-core 3.8G |  | 65 | 90 |
|  |  | 6320 | Dual-core 3.9G |  | 65 | 90 |
| 6th generation i5 | Intel Core | 6400 | Quad-core 2.7GHz | 3.3G | 65 | 90 |
|  |  | 6500 | Quad-core 3.2GHz | 3.6G | 65 | 90 |
|  |  | 6600 | Quad-core 3.3GHz | 3.9G | 65 | 90 |
| 6th generation i7 | Intel Core | 6700 | Quad-core 3.4GHz | 4.0G | 65 | 90 |
| 7th generation i3 | Intel Core | 7100 | Dual-core 3.9G |  | 65 | 90 |
|  |  | 7300 | Dual-core 4.0G |  | 65 | 90 |
|  |  | 7320 | Dual-core 4.1G |  | 65 | 90 |
| 7th Gen i5 | Intel Core | 7400 | Quad-core 3.0GHz | 3.5G | 65 | 90 |
|  |  | 7500 | Quad-core 3.4GHz | 3.8G | 65 | 90 |
|  |  | 7600 | Quad-core 3.5GHz | 4.1G | 65 | 90 |
| 7th generation i7 | Intel Core | 7700 | Quad-core 3.6GHz | 4.2G | 65 | 90 |
| 8th generation i3 | Intel Core | 8100 | Quad-core 3.6GHz |  | 65 | 90 |
|  |  | 8300 | Quad-core 3.7GHz |  | 65 | 90 |
| 8th Gen i5 | Intel Core | 8400 | Six-core 2.8GHz | 4.0G | 65 | 90 |
|  |  | 8500 | Six-core 3.0GHz | 4.1G | 65 | 90 |
|  |  | 8600 | Six-core 3.2GHz | 4.3G | 65 | 90 |
| 8th Gen i7 | Intel Core | 8700 | Six-core 3.2GHz | 4.6G | 65 | 90 |
| 9th generation i3 | Intel Core | 9100 | Quad-core 3.6GHz | 4.2G | 65 | 90 |
|  |  | 9300 | Quad-core 3.7GHz | 4.2G | 65 | 90 |
| 9th Gen i5 | Intel Core | 9400 | Six-core 2.9GHz | 4.1G | 65 | 90 |
|  |  | 9500 | Six-core 3.0GHz | 4.4G | 65 | 90 |
|  |  | 9600 | Six-core 3.1GHz | 4.6G | 65 | 90 |
| 9th Gen i7 | Intel Core | 9700 | Octa-core 3.0G | 4.7G | 65 | 90 |
| 9th generation I9 | Intel Core | 9900 | Octa-core 3.1GHz | 5.0G | 65 | 90 |

※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※

※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※

| HDMI®​ | VGA/DP |  |  | LAN | USB 2.0 | USB 3.2 | COM | SSD | Memory | Power |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 4K 30Hz Optional 4K 60Hz | VGA 2K 60Hz Optional DP 4K 60Hz |  |  | 1 | 6 | 2 | 2/6 | 1*MSATA 2*SATA | 2*NB-DDR4 MAX 64G | 12V/19V |
|  |  |  |  | 2 | 4 |  |  |  |  |  |
| PC model |  | IPC100U-QM5100 |  |  |  |  |  |  |  |  |
| PC size |  | Dimensions with handle: 430mm(L)*301mm(W)*44mm(H) Dimensions with handle: 480mm(L)*301mm(W)*44mm(H) |  |  |  |  |  |  |  |  |
| Cooling system |  | With ventilation holes and silent fan cooling |  |  |  |  |  |  |  |  |
| color |  | black |  |  |  |  |  |  |  |  |
| Compatible motherboard models |  | QM5100 |  |  |  |  |  |  |  |  |
| motherboard |  | CPU | Supports Intel® Comet late 10th generation LGA1200 Core i3-10100, i5-10400, i7-10700, i9-10900, etc. Supports Intel® Rocket Lake 11th Gen LGA1200 Core i5-11400, i7-11700, i9-11900, etc. (See the next page for a complete list of CPUs) |  |  |  |  |  |  |  |
|  |  | graphics card | 10th Gen: Intel® HD Graphics 630 integrated graphics, supporting single display, dual display duplication, and dual display expansion; 11th Gen: Intel® HD Graphics 730/750 integrated graphics, supporting single display, dual display duplication, and dual display expansion; |  |  |  |  |  |  |  |
|  |  | Memory | 2*NB-DDR4 memory modules support 2133/2400/2666/2933MHz, with a maximum capacity of 64GB. |  |  |  |  |  |  |  |
| Front panel interface |  | POWER | 1* Switch button |  |  |  |  |  |  |  |
|  |  | RESET | 1* Restart Switch |  |  |  |  |  |  |  |
|  |  | led | 1 power indicator, 1 hard drive indicator |  |  |  |  |  |  |  |
|  |  | USB | 2 USB 2.0 ports |  |  |  |  |  |  |  |
| Rear panel interface |  | Power interface | 1 power interface, built-in DC power adapter, supports DC 12V/19V, recommended power supply is 12V 10A or 19V 6A or higher. |  |  |  |  |  |  |  |
|  |  | Display Interface | 1 HDMI® port When the CPU is an 11th generation i5/i7/i9, the motherboard's HDMI® 2.0 supports 4K@60Hz; When the CPU is a 10th generation i3/i5/i7/i9, the motherboard's HDMI® 1.4 supports 4K@30Hz. 1 VGA interface (1920*1080@60Hz) Optional 1*DP 1.4 interface (4096/3840*2160@60Hz) |  |  |  |  |  |  |  |
|  |  | LAN | Single Gigabit Ethernet adapter: Realtek 8111H Gigabit Ethernet adapter, with optional support for Yutai Micro YT6801 Gigabit Ethernet adapter; Dual Gigabit Ethernet adapters: Realtek 8111H + Realtek 8111H Gigabit Ethernet adapter, with optional support for Yutai Micro YT6801 + Yutai Micro YT6801 Gigabit Ethernet adapter; Supports Wake-on-LAN and PXE network functionality; |  |  |  |  |  |  |  |
|  |  | USB | 2 USB 3.2 Gen1 ports 4 USB 2.0 ports (2 USB 2.0 ports for dual-network PC) |  |  |  |  |  |  |  |
|  |  | COM | 6 COM interfaces, supporting RS232 by default; COM1 can optionally support RS232/RS422/RS485. |  |  |  |  |  |  |  |
|  |  | audio | 1*LINE_OUT interface, audio output 1*MIC_IN interface, microphone input |  |  |  |  |  |  |  |
|  |  | WiFi | 2 WiFi antenna interfaces |  |  |  |  |  |  |  |
| storage |  | mSATA | 1 mSATA slot, supports mSATA drives |  |  |  |  |  |  |  |
|  |  | SATA | 2 SATA 3.0 hard drive interfaces, supporting 2.5-inch SSDs/HDDs or 3.5-inch HDDs. |  |  |  |  |  |  |  |
|  |  | Hard drive bays in the chassis | Two hard drive bays (one 2.5-inch SSD/HDD bay and one 3.5-inch HDD bay; this only indicates the number of hard drives the case can accommodate). |  |  |  |  |  |  |  |
| Extend |  | Mini-PCIe | 1 Mini-PCIe slot, supports PCIe 3.0 x1 & USB signal, supports WiFi, Bluetooth/4G modules. |  |  |  |  |  |  |  |
| Operating environment |  | Operating temperature | -10℃ ~ 60℃ |  |  |  |  |  |  |  |
|  |  | Storage temperature | -20℃ ~ 70℃ |  |  |  |  |  |  |  |
|  |  | relative humidity | 5%-95% relative humidity, non-condensing |  |  |  |  |  |  |  |
|  |  | Working vibration | 0.5grms/5-500HZ/random/operating |  |  |  |  |  |  |  |
| BIOS |  | Supports power-on upon startup, scheduled power-on, and remote power-on/off. |  |  |  |  |  |  |  |  |
| Watch Dog |  | Watchdog programming with support for hardware reset function (256 levels, 0~255 seconds). |  |  |  |  |  |  |  |  |
| operating system |  | Supports Windows 11, Windows 10, Linux, etc. |  |  |  |  |  |  |  |  |

※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※

※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※

The CPU information below is for reference only. Please consult our sales team for specific requirements!

| CPU Specifications | CPU Brand | CPU model | clock speed | Turbo Boost | CPU power (W) | Full load power consumption (W) |
| --- | --- | --- | --- | --- | --- | --- |
| 10th Gen i3 | Intel Core | 10100 | Quad-core 3.6GHz | 4.3G | 65 | 110 |
|  |  | 10105 | Quad-core 3.7GHz | 4.4G | 65 | 110 |
|  |  | 10300 | Quad-core 3.7GHz | 4.4G | 65 | 110 |
|  |  | 10305 | Quad-core 3.8GHz | 4.5G | 65 | 110 |
|  |  | 10320 | Quad-core 3.8GHz | 4.6G | 65 | 110 |
|  |  | 10325 | Quad-core 3.9GHz | 4.7G | 65 | 110 |
| 10th Gen i5 | Intel Core | 10400 | Six-core 2.9GHz | 4.3G | 65 | 110 |
|  |  | 10505 | Six-core 3.2GHz | 4.6G | 65 | 110 |
|  |  | 10500 | Six-core 3.1GHz | 4.5G | 65 | 110 |
|  |  | 10600 | Six-core 3.3GHz | 4.8G | 65 | 110 |
| 10th Gen i7 | Intel Core | 10700 | Octa- core 2.9GHz | 4.8G | 65 | 110 |
| 10th Gen i9 | Intel Core | 10900 | Deca core 2.8G | 5.2G | 65 | 110 |
| 11th Gen i5 | Intel Core | 11400 | Six-core 2.6GHz | 4.4G | 65 | 110 |
|  |  | 11500 | Six-core 2.7GHz | 4.6G | 65 | 110 |
|  |  | 11600 | Six-core 2.8GHz | 4.8G | 65 | 110 |
| 11th Gen i7 | Intel Core | 11700 | Octa-core 2.5GHz | 4.9G | 65 | 110 |
| 11th Gen i9 | Intel Core | 11900 | Octa-core 2.5GHz | 5.2G | 65 | 110 |

※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※

※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※

| HDMI® | VGA/DP |  |  | LAN | USB2.0 | USB3.2 | COM | SSD | Memory | Power |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 4K 60Hz | VGA 2K 60Hz Optional DP 4K 60Hz |  |  | 1 | 6 | 2 | 2/6 | 1*M.2 1*SATA | 2*NB-DDR4 MAX 64G | 12V/19V |
|  |  |  |  | 2 | 4 |  |  |  |  |  |
| PC model |  | IPC100U-QM6100 |  |  |  |  |  |  |  |  |
| PC size |  | Dimensions with handle: 430mm(L)*301mm(W)*44mm(H) Dimensions with handle: 480mm(L)*301mm(W)*44mm(H) |  |  |  |  |  |  |  |  |
| Cooling system |  | With ventilation holes and silent fan cooling |  |  |  |  |  |  |  |  |
| color |  | black |  |  |  |  |  |  |  |  |
| Compatible motherboard models |  | QM6100 |  |  |  |  |  |  |  |  |
| motherboard |  | CPU | Supports Intel® Alder Lake 12th generation LGA 1700 Core i3-12100, i5-12400, i7-12700, i9-12900, etc. Supports Intel® Raptor Lake 13th generation LGA 1700 Core i3-13100, i5-13400, i7-13400, i9-13900, etc. Supports Intel® Raptor Lake 14th generation LGA 1700 Core i3-14100, i5-14400, i7-14700, i9-14900, etc. (See the next page for a complete CPU list) |  |  |  |  |  |  |  |
|  |  | graphics card | Intel® UHD Graphics 730/770 graphics controller, supporting single display, dual display duplication, and dual display expansion. |  |  |  |  |  |  |  |
|  |  | Memory | 2*NB-DDR4 memory modules support 2133/2400/2666/2933/3200MHz, with a maximum capacity of 64GB. |  |  |  |  |  |  |  |
| Front panel interface |  | POWER | 1* Switch button |  |  |  |  |  |  |  |
|  |  | RESET | 1* Restart Switch |  |  |  |  |  |  |  |
|  |  | led | 1 power indicator, 1 hard drive indicator |  |  |  |  |  |  |  |
|  |  | USB | 2 USB 2.0 ports |  |  |  |  |  |  |  |
| Rear panel interface |  | Power interface | 1 power interface, built-in DC power adapter, supports DC 12V/19V, recommended power supply is 12V 10A or 19V 6A or higher. |  |  |  |  |  |  |  |
|  |  | Display Interface | 1*HDMI® 2.0 port (4096/3840*2160@60HZ) 1 VGA interface (1920*1080@60Hz) Optional 1*DP 1.4 interface (4096/3840*2160@60HZ) |  |  |  |  |  |  |  |
|  |  | LAN | Single Gigabit Ethernet adapter: Intel® I219V Gigabit Ethernet adapter; Dual Gigabit Ethernet adapters: Intel® I219V + Realtek 8111H Gigabit Ethernet adapter; Supports Wake-on-LAN and PXE network functionality; |  |  |  |  |  |  |  |
|  |  | USB | 2 USB 3.2 Gen1 ports 4 USB 2.0 ports (2 USB 2.0 ports for dual-network PC) |  |  |  |  |  |  |  |
|  |  | COM | 6 COM ports, supporting RS232 by default; COM1 can optionally support RS232/RS422/RS485. |  |  |  |  |  |  |  |
|  |  | audio | 1*LINE_OUT interface, audio output 1*MIC_IN interface, microphone input |  |  |  |  |  |  |  |
|  |  | WiFi | 2 WiFi antenna interfaces |  |  |  |  |  |  |  |
| storage |  | M.2 | 1 M.2 Key-M slot, supporting M.2 2280 NVME PCIe 3.0 x4 protocol, can be modified to support SATA protocol; optional M.2 2242 hard drive; |  |  |  |  |  |  |  |
|  |  | SATA | One SATA 3.0 hard drive interface, supporting 2.5-inch SSDs/HDDs or 3.5-inch HDDs. |  |  |  |  |  |  |  |
|  |  | Hard drive bays in the chassis | Two hard drive bays (one 2.5-inch SSD/HDD bay and one 3.5-inch HDD bay; this only indicates the number of hard drives the case can accommodate). |  |  |  |  |  |  |  |
| Extend |  | Mini-PCIe | 1 Mini-PCIE slot, supporting PCIE 3.0 x1 & USB signal, and supporting WiFi, Bluetooth/4G modules; |  |  |  |  |  |  |  |
| Operating environment |  | Operating temperature | -10℃ ~ 60℃ |  |  |  |  |  |  |  |
|  |  | Storage temperature | -20℃ ~ 70℃ |  |  |  |  |  |  |  |
|  |  | relative humidity | 5%-95% relative humidity, non-condensing |  |  |  |  |  |  |  |
|  |  | Working vibration | 0.5grms/5-500HZ/random/operating |  |  |  |  |  |  |  |
| BIOS |  | Supports power-on upon startup, scheduled power-on, and remote power-on/off. |  |  |  |  |  |  |  |  |
| Watch Dog |  | Watchdog programming with support for hardware reset function (256 levels, 0~255 seconds). |  |  |  |  |  |  |  |  |
| operating system |  | Supports Windows 11, Windows 10, Linux, etc. |  |  |  |  |  |  |  |  |

※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※

The CPU information below is for reference only. Please consult our sales team for specific requirements!

| CPU Specifications | CPU Brand | CPU model | clock speed | Turbo Boost | CPU power consumption (W) | Maximum turbo power consumption (W) |
| --- | --- | --- | --- | --- | --- | --- |
| 12th Gen i3 | Intel Core | 1 21 00 | Quad- core 3.3 GHz | 4.3 G | 60 | 89 |
|  | Intel Core | 12300 | Quad- core 3.5 GHz | 4.4 G | 60 | 89 |
| 12th Gen i5 | Intel Core | 1 2 400 | Six-core 2.5GHz | 4.4G | 65 | 117 |
|  |  | 1 2 500 | Six-core 3.0GHz | 4.6G | 65 | 117 |
|  |  | 1 2 600 | Six-core 3.3GHz | 4.8G | 65 | 117 |
| 12th Gen i7 | Intel Core | 12700 | 12 cores 2.1GHz | 4.9G | 65 | 180 |
| 12th Gen i9 | Intel Core | 1 2 900 | Sixteen cores , 2.4 GHz | 5.1 G​ | 65 | 202 |
| 13th generation i3 | Intel Core | 13100 | Quad-core 3.4GHz | 4.5G | 60 | 89 |
| 13th Gen i5 | Intel Core | 13400 | deca-core 2.5GHz | 4.6G | 65 | 154 |
| 13th Gen i5 | Intel Core | 13500 | Fourteen cores 2.5G | 4.8G | 65 | 154 |
| 13th Gen i5 | Intel Core | 13600 | Fourteen cores 2.7GHz | 5.3G | 65 | 154 |
| 13th Gen i7 | Intel Core | 13700 | 16 cores 2.1GHz | 5.2G | 65 | 219 |
| 13th generation i9 | Intel Core | 13900 | 24 cores 2.3GHz | 5.6G | 65 | 219 |
| 14th Gen i3 | Intel Core | 14100 | Quad-core 3.5GHz | 4.7G | 60 | 154 |
| 14th Gen i5 | Intel Core | 14400 | Deca core 2.5GHz | 4.7G | 65 | 219 |
| 14th Gen i5 | Intel Core | 14500 | Fourteen cores 2.6GHz | 5.0G | 65 | 219 |
| 14th Gen i5 | Intel Core | 14600 | Fourteen cores, 2.7GHz | 5.2G | 65 | 219 |
| 14th Gen i7 | Intel Core | 14700 | Twenty cores 2.1GHz | 5.4G | 65 | 219 |
| 14th Gen i9 | Intel Core | 14900 | 24 cores 2.0GHz | 5.8G | 65 | 219 |

※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※

※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※

| HDMI® | VGA/DP |  |  | LAN | USB2.0 | USB3.2 | COM | SSD | Memory | Power |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 4K 60Hz | VGA 2K 60Hz Optional DP 4K 60Hz |  |  | 1 | 4 | 4 | 2/6 | 1*M.2 1*SATA | 2*NB-DDR4 MAX 64G | 12V/19V |
| PC model |  | IPC100U-QM6300 |  |  |  |  |  |  |  |  |
| PC size |  | Dimensions with handle: 430mm(L)*301mm(W)*44mm(H) Dimensions with handle: 480mm(L)*301mm(W)*44mm(H) |  |  |  |  |  |  |  |  |
| Cooling system |  | With ventilation holes and silent fan cooling |  |  |  |  |  |  |  |  |
| color |  | black |  |  |  |  |  |  |  |  |
| Compatible motherboard models |  | QM6300 |  |  |  |  |  |  |  |  |
| motherboard |  | CPU | Supports Intel® Alder Lake 12th generation LGA 1700 Core i3-12100, i5-12400, i7-12700, i9-12900, etc. Supports Intel® Raptor Lake 13th generation LGA 1700 Core i3-13100, i5-13400, i7-13400, i9-13900, etc. Supports Intel® Raptor Lake 14th generation LGA 1700 Core i3-14100, i5-14400, i7-14700, i9-14900, etc. (See the next page for a complete list of CPUs) |  |  |  |  |  |  |  |
|  |  | graphics card | Intel® UHD Graphics 730/770 graphics controller, supporting single display, dual display duplication, and dual display expansion. |  |  |  |  |  |  |  |
|  |  | Memory | 2*NB-DDR4 memory modules support 2133/2400/2666/2933/3200MHz, with a maximum capacity of 64GB. |  |  |  |  |  |  |  |
| Front panel interface |  | POWER | 1* Switch button |  |  |  |  |  |  |  |
|  |  | RESET | 1* Restart Switch |  |  |  |  |  |  |  |
|  |  | led | 1 power indicator, 1 hard drive indicator |  |  |  |  |  |  |  |
|  |  | USB | 2 USB 2.0 ports |  |  |  |  |  |  |  |
| Rear panel interface |  | Power interface | 1 power interface, built-in DC power adapter, supports DC 12V/19V, recommended power supply is 12V 10A or 19V 6A or higher. |  |  |  |  |  |  |  |
|  |  | Display Interface | 1*HDMI® 2.0 port (4096/3840*2160@60HZ) 1 VGA interface (1920*1080@60Hz) Optional 1*DP 1.4 interface (4096/3840*2160@60HZ) |  |  |  |  |  |  |  |
|  |  | LAN | Single Gigabit Ethernet adapter: Realtek 8111H Gigabit Ethernet adapter; Supports Wake-on-LAN and PXE network functionality; |  |  |  |  |  |  |  |
|  |  | USB | 4 USB 3.2 Gen1 ports 2 USB 2.0 ports |  |  |  |  |  |  |  |
|  |  | COM | 6 COM interfaces, supporting RS232 by default; COM1 can optionally support RS232/RS422/RS485. |  |  |  |  |  |  |  |
|  |  | audio | 1*LINE_OUT interface, audio output 1*MIC_IN interface, microphone input |  |  |  |  |  |  |  |
|  |  | WiFi | 2 WiFi antenna interfaces |  |  |  |  |  |  |  |
| storage |  | M.2 | 1 M.2 Key-M slot, supporting M.2 2280 NVME PCIe 3.0 x4 / SATA adaptive standard SSD; |  |  |  |  |  |  |  |
|  |  | SATA | One SATA 3.0 hard drive interface, supporting 2.5-inch SSDs/HDDs or 3.5-inch HDDs. |  |  |  |  |  |  |  |
|  |  | Hard drive bays in the chassis | Two hard drive bays (one 2.5-inch SSD/HDD bay and one 3.5-inch HDD bay; this only indicates the number of hard drives the case can accommodate). |  |  |  |  |  |  |  |
| Extend |  | M.2 WiFi | 1*M.2 Key-E slot, supports M.2 2230 PCIE3.0 x1 & USB signals, supports WiFi and Bluetooth modules; |  |  |  |  |  |  |  |
| Operating environment |  | Operating temperature | -10℃ ~ 60℃ |  |  |  |  |  |  |  |
|  |  | Storage temperature | -20℃ ~ 70℃ |  |  |  |  |  |  |  |
|  |  | relative humidity | 5%-95% relative humidity, non-condensing |  |  |  |  |  |  |  |
|  |  | Working vibration | 0.5grms/5-500HZ/random/operating |  |  |  |  |  |  |  |
| BIOS |  | Supports power-on upon startup, scheduled power-on, and remote power-on/off. |  |  |  |  |  |  |  |  |
| Watch Dog |  | Watchdog programming with support for hardware reset function (256 levels, 0~255 seconds). |  |  |  |  |  |  |  |  |
| operating system |  | Supports Windows 11, Windows 10, Linux, etc. |  |  |  |  |  |  |  |  |

※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※

※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※

The CPU information below is for reference only. Please consult our sales team for specific requirements!

| CPU Specifications | CPU Brand | CPU model | clock speed | Turbo Boost | CPU power consumption (W) | Maximum turbo power consumption (W) |
| --- | --- | --- | --- | --- | --- | --- |
| 12th Gen i3 | Intel Core | 1 21 00 | Quad- core 3.3 GHz | 4.3 G | 60 | 89 |
|  | Intel Core | 12300 | Quad- core 3.5 GHz | 4.4 G | 60 | 89 |
| 12th Gen i5 | Intel Core | 1 2 400 | Six-core 2.5GHz | 4.4G | 65 | 117 |
|  |  | 1 2 500 | Six-core 3.0GHz | 4.6G | 65 | 117 |
|  |  | 1 2 600 | Six-core 3.3GHz | 4.8G | 65 | 117 |
| 12th Gen i7 | Intel Core | 12700 | 12 cores 2.1GHz | 4.9G | 65 | 180 |
| 12th Gen i9 | Intel Core | 1 2 900 | Sixteen cores , 2.4 GHz | 5.1 G​ | 65 | 202 |
| 13th generation i3 | Intel Core | 13100 | Quad-core 3.4GHz | 4.5G | 60 | 89 |
| 13th Gen i5 | Intel Core | 13400 | deca-core 2.5GHz | 4.6G | 65 | 154 |
| 13th Gen i5 | Intel Core | 13500 | Fourteen cores 2.5G | 4.8G | 65 | 154 |
| 13th Gen i5 | Intel Core | 13600 | Fourteen cores 2.7GHz | 5.3G | 65 | 154 |
| 13th Gen i7 | Intel Core | 13700 | 16 cores 2.1GHz | 5.2G | 65 | 219 |
| 13th generation i9 | Intel Core | 13900 | 24 cores 2.3GHz | 5.6G | 65 | 219 |
| 14th Gen i3 | Intel Core | 14100 | Quad-core 3.5GHz | 4.7G | 60 | 154 |
| 14th Gen i5 | Intel Core | 14400 | Deca core 2.5GHz | 4.7G | 65 | 219 |
| 14th Gen i5 | Intel Core | 14500 | Fourteen cores 2.6GHz | 5.0G | 65 | 219 |
| 14th Gen i5 | Intel Core | 14600 | Fourteen cores, 2.7GHz | 5.2G | 65 | 219 |
| 14th Gen i7 | Intel Core | 14700 | Twenty cores 2.1GHz | 5.4G | 65 | 219 |
| 14th Gen i9 | Intel Core | 14900 | 24 cores 2.0GHz | 5.8G | 65 | 219 |

※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※※

Illustrate

Except for the accessories listed that come with the product, the contents of this manual do not represent a commitment by the company, and the company reserves the right to make changes to this manual without prior notice.

We are not responsible for any direct, indirect, intentional or unintentional damage or potential hazards caused by improper installation or use.

Before ordering a product, please consult with the distributor to understand in detail whether the product performance meets your needs.

The contents of this manual are protected by copyright. All rights reserved. No part of this manual may be reproduced in any mechanical, electronic or other manner without permission.

The final interpretation right of this instruction manual belongs to SHENZHEN ELSKY TECHNOLOGY CO.,LTD.

The above ordering information is for reference only. Please contact our sales department for details. Tel: 0755-82260150

Motherboard driver download: https://pan.baidu.com/s/1ie7qi5NutS53ipPecRSYtw

Access password: cbn5