ELSKY IPC7000A PC Specification

PC diagram:

Dual network pc

The above pictures are for reference only,please refer to the actual product!

Product model ordering optional:

| PC model | LAN | USB2.0 | USB3.0 | SSD | Memory | Power |
| --- | --- | --- | --- | --- | --- | --- |
| 3th gen series | 1 | 6 | 2 | MSATA | 1*NB-DDR3 8G（MAX） | 12V |
|  | 2 | 4 | 2 |  |  |  |
| J1900 series | 1 | 7 | 1 | MSATA | 1*NB-DDR3 8G（MAX） | 12V |
|  | 2 | 5 | 1 |  |  |  |
| 6th,7th gen series | 1 | 6 | 2 | MSATA | 2*NB-DDR3 16G（MAX） | 12V/19V |
|  | 2 | 4 | 2 |  |  |  |
| J3355 series | 1 | 6 | 2 | MSATA/M.2 | 1*NB-DDR3 8G（MAX） | 12V |
|  | 2 | 4 | 2 |  |  |  |
| N3160 series | 1 | 6 | 2 | MSATA | 1*NB-DDR3 8G（MAX） | 12V |
|  | 2 | 4 | 2 |  |  |  |
| QM9850 | 1 | 6 | 2 | MSATA/M.2 | 2*NB-DDR4 32G（MAX） | 12V/19V |
|  | 2 | 4 | 2 |  |  |  |

Note: The PC does not support 2.5-inch hard drives. To support it, customization is required!

IPC7000A PC Specification:

| PC model | IPC7000A-3th gen series |  |
| --- | --- | --- |
| Motherboard model | 1007F, 1037F, 2117F, i3HGP, i5HGP, i7HGP |  |
| PC size | 200mm*200mm*41mm |  |
| Power supply | 1*DC_IN | The inner diameter of the DC head is 2.5mm and supports 12V. It is recommended to use a power supply of 12V5A or higher. |
| 3th gen CPU | Intel Celeron 1007U dual core 1.5GHz; Intel Celeron 1037U dual core 1.8GHz; Intel Pentium 2117U dual core 1.8GHz; Intel Core i3-3217U dual core 1.8GHz; Intel Core i3-3227U Dual Core 1.9GHz; Intel Core i5-3317U dual core 1.7GHz, turbo 2.6GHz; Intel Core i5-3337U dual core 1.8GHz, turbo 2.7GHz; Intel Core i5-3427U dual core 1.8GHz, turbo 2.8GHz; Intel Core i5-3437U dual core 1.9GHz, turbo 2.9GHz; Intel Core i7-3517U dual core 1.9GHz, turbo 3.0GHz; Intel Core i7-3537U dual core 2.0GHz, turbo 3.1GHz; Intel Core i7-3667U dual core 2.0GHz, turbo 3.2GHz; Intel Core i7-3687U dual core 2.1GHz, turbo 3.3GHz; |  |
| Chipset | Intel 6 series chipset: HM65, HM67, QM67 Intel 7 series chipset: NM70, HM70, HM75, HM76, HM77, QM77 |  |
| Memory | 1*NB-DDR3 memory ,support 1.35V/1.5V ,support 1333/1600MHz memory ,maximum support 8G |  |
| Network function | Integrated Realtek 8111E Gigabit Ethernet card, supporting Wake on LAN and network PXE function |  |
|  | Supports WIFI/3G/4G and 2*WIFI antenna poles |  |
| Display function | Intel HD Graphics 4000 graphics controller, supporting single display, dual display replication, and dual display expansion |  |
|  | 1*VGA | Supports the highest resolution 1920*1200@60Hz |
|  | 1*HDMI | Supports the highest resolution 1920*1080@60Hz |
| USB function | 2*USB3.0 | The blue group USB is USB 3.0 speed (USB 2.0 speed when using Intel 6 series chipset) |
|  | 6*USB2.0 | Black group USB with USB 2.0 speed |
| Sound function | Integrated Realtek ALC662 HD digital audio decoder, 6-channel high fidelity audio controller |  |
|  | 1*SPK | Support audio output |
|  | 1*MIC | Support microphone input |
|  | HDMI interface supports HDMI audio output |  |
| Hard disk function | MSATA disk (8G/16G/32G/64G/120G/128G/240G/256G, etc.) |  |
| Switch function | POWER | Switch button |
| Operating environment | Working temperature: -20 ℃~60 ℃; Working humidity: 5%~95% relative humidity, no condensation |  |
| BIOS | AMI BIOS,support power on, timed power on, and remote power on |  |
| Watch Dog | Watchdog programming, supports hardware reset function (256 levels, 0-255 seconds) |  |
| Operating system | Windows 10, Windows 8, Windows 7, Windows XP, Linux, etc |  |

Instructions:

☆ When the main chipset is an Intel 6 series chipset, the blue USB interface is USB 2.0 and there is no USB 3.0;

☆ When the main chipset is Intel 7 series chipset, the blue USB interface is USB 3.0;

☆ The PC only supports MSATA disks;

☆ Single Gigabit Ethernet port/Dual Gigabit Ethernet port optional for the PC; Dual network ports only affect the number of USB 2.0 ports;

☆ The PC is equipped with wall mounted holes;

IPC7000A PC Specification:

| PC model | IPC7000A-J1900 series |  |
| --- | --- | --- |
| Motherboard model | M218F, M219F, M218FC, M219FC |  |
| PC size | 200mm*200mm*41mm |  |
| Power supply | 1*DC_IN | The inner diameter of the DC head is 2.5mm and supports 12V. It is recommended to use a power supply of 12V5A or higher |
| CPU | Intel Celeron J1800 dual core 2.41GHz, turbo 2.58GHz; Intel Celeron J1900 quad core 1.99GHz, turbo 2.4GHz; Intel Pentium J2900 quad core 2.41GHz, turbo 2.66GHz; |  |
| Chipset | Intel Bay trail SOC chipset |  |
| Memory | 1*NB-DDR3 memory,support 1.35V,support 1600MHz memory,maximum support 8G |  |
| Network function | Integrated Realtek 8111E Gigabit Ethernet card, supporting Wake on LAN and network PXE function |  |
|  | Supports WIFI/3G/4G and 2 * WIFI antenna poles |  |
| Display function | Intel HD Graphics (Sandy Bridge) graphics controller, supporting single display, dual display replication, and dual display expansion |  |
|  | 1*VGA | Supports the high resolution 1920*1200@60Hz |
|  | 1*HDMI | Supports the high resolution 1920*1080@60Hz |
| USB function | 1*USB3.0 | The blue group USB is USB 3.0 speed (blue group, the one above the chassis direction is USB 3.0) |
|  | 7*USB2.0 | The black group USB is USB 2.0 speed (the blue group, below the chassis direction is USB 2.0) |
| Sound function | Integrated Realtek ALC662 HD digital audio decoder, 6-channel high fidelity audio controller |  |
|  | 1*SPK | Support audio output |
|  | 1*MIC | Support microphone input |
|  | HDMI interface supports HDMI audio output |  |
| Hard disk function | MSATA disk（8G/16G/32G/64G/120G/128G/240G/256G,etc） |  |
| Switch function | POWER | Swutch button |
| Operating environment | Working temperature: -20 ℃~60 ℃; Working humidity: 5%~95% relative humidity, no condensation |  |
| BIOS | AMI BIOS,support power on, timed power on, and remote power on |  |
| Watch Dog | Watchdog programming, supports hardware reset function (256 levels, 0-255 seconds) |  |
| Operating system | Windows 10, Windows 8, Windows 7, Linux, etc |  |

Instructions:

☆ The blue USB interface of the PC is USB 3.0 on the top of the chassis direction and USB 2.0 on the bottom of the chassis direction;

☆ The PC only supports MSATA disks;

☆ Single Gigabit Ethernet port/Dual Gigabit Ethernet port optional for the PC; Dual network ports only affect the number of USB 2.0 ports;

☆ The PC is equipped with wall mounted holes;

IPC7000A PC Specification:

| PC model | IPC7000A-6th,7th gen series |  |
| --- | --- | --- |
| Motherboard model | QM9600, QM9700, QM9600H, QM9700H |  |
| PC size | 200mm*200mm*41mm |  |
| Power supply | 1*DC_IN | The inner diameter of the DC head is 2.5mm, supporting 12V/19V. It is recommended to use a power supply of 12V5A or 19V5A or higher |
| 6th gen CPU | Intel Celeron 3855U dual core 1.6GHz; Celeron 3955U dual core 2.0GHz; Intel Core i3-6006U dual core 2.0GHz; Core i3-6100U dual core 2.3GHz; Intel Core i3-6157U dual core 2.4GHz; Core i3-6167U dual core 2.7GHz; Intel Core i5-6200U dual core 2.3GHz, turbo 2.8GHz; Core i5-6260U dual core 1.8GHz, turbo 2.9GHz; Intel Core i5-6267U dual core 2.9GHz, turbo 3.3GHz; Core i5-6287U dual core 3.1GHz, turbo 3.5GHz; Intel Core i5-6300U dual core 2.4GHz, turbo 3.0GHz; Core i5-6360U dual core 2.0GHz, turbo 3.1GHz; Intel Core i7-6500U dual core 2.5GHz, turbo 3.1GHz; Core i7-6560U dual core 2.2GHz, turbo 3.2GHz; Intel Core i7-6567U dual core 3.3GHz, turbo 3.6GHz; Core i7-6600U dual core 2.6GHz, turbo frequency 3.4GHz; Intel Core i7-6650U dual core 2.2GHz, turbo 3.4GHz; Core i7-6660U dual core 2.4GHz, turbo 3.4GHz; |  |
| 7th gen CPU | Intel Celeron 3865U dual core 1.8GHz; Celeron 3965U dual core 2.2GHz; Intel Core i3-7020U dual core 2.3GHz; Core i3-7100U dual core 2.4GHz; Intel Core i3-7130U dual core 2.7GHz; Intel Core i5-7200U dual core 2.5GHz, turbo 3.1GHz; Core i5-7260U dual core 2.2GHz, turbo 3.4GHz; Intel Core i5-7267U dual core 3.1GHz, turbo 3.5GHz; Core i5-7287U dual core 3.3GHz, turbo 3.7GHz; Intel Core i5-7300U dual core 2.6GHz, turbo 3.5GHz; Core i5-7360U dual core 2.3GHz, turbo 3.6GHz; Intel Core i7-7500U dual core 2.7GHz, turbo 3.5GHz; Core i7-7560U dual core 2.4GHz, turbo 3.8GHz; Intel Core i7-7567U dual core 3.5GHz, turbo 4.0GHz; Core i7-7600U dual core 2.8GHz, turbo 3.9GHz; Intel Core i7-7660U dual core 2.5GHz, turbo 4.0GHz; |  |
| Chipset | Single chip integrated into CPU |  |
| Memory | 2*NB-DDR3 memory, supports 1.35V, 1600MHz memory, maximum support 16GB |  |
| Network function | Integrated Realtek 8111F Gigabit Ethernet card, supporting Wake on LAN and network PXE function |  |
|  | Supports WIFI/3G/4G and 2*WIFI antenna poles |  |
| Display function | 6th generation integrated Intel HD Graphics 510/520 graphics controller, supporting single display, dual display replication, and dual display expansion; 7th generation integrated Intel HD Graphics 610/620 graphics controller, supporting single display, dual display replication, and dual display expansion; |  |
|  | 1*VGA | Supports the highest resolution 1920*1200@60Hz |
|  | 1*HDMI | Supports 4K resolution 4096/3840*2160@24/30Hz |
| USB function | 2*USB3.0 | The blue group USB is USB 3.0 speed |
|  | 6*USB2.0 | The black group USB is USB 2.0 speed |
| Sound function | Integrated Realtek ALC662 HD digital audio decoder, 6-channel high fidelity audio controller |  |
|  | 1*SPK | Support audio output |
|  | 1*MIC | Support microphone input |
|  | HDMI interface supports HDMI audio output |  |
| Hard disk function | MSATA disks（8G/16G/32G/64G/120G/128G/240G/256G,etc） |  |
| Switch function | POWER | Switch button |
| Operating environment | Working temperature: -20 ℃~60 ℃; Working humidity: 5%~95% relative humidity, no condensation |  |
| BIOS | AMI BIOS,support power on, timed power on, and remote power on |  |
| Watch Dog | Watchdog programming, supports hardware reset function (256 levels, 0-255 seconds) |  |
| Operating system | Windows 10, Windows 8, Windows 7, Linux, etc |  |

Instructions:

☆ The PC only supports MSATA disks;

☆ Single Gigabit Ethernet port/Dual Gigabit Ethernet port optional for the PC; Dual network ports only affect the number of USB 2.0 ports;

☆ HDMI supports 4K output;

☆ The PC is equipped with wall mounted holes;

IPC7000A PC Specification:

| PC model | IPC7000A-J3355 series |  |
| --- | --- | --- |
| Motherboard model | M335F |  |
| PC size | 200mm*200mm*41mm |  |
| Power supply | 1*DC_IN | The inner diameter of the DC head is 2.5mm and supports 12V. It is recommended to use a power supply of 12V5A or higher |
| CPU | Intel Celeron J3355 dual core 2.0GHz, turbo 2.5GHz; Intel Celeron J3455 quad core 1.5GHz, Turbo 2.3GHz; Intel Pentium N4200 quad core 1.1GHz, turbo 2.5GHz; |  |
| Chipset | Single chip integrated into CPU |  |
| Memory | 1*NB-DDR3 memory, supports 1.35V, 1600MHz memory, maximum support 8GB |  |
| Network function | Integrated Realtek 8111E Gigabit Ethernet card, supporting Wake on LAN and network PXE function |  |
|  | Supports WIFI/3G/4G and 2*WIFI antenna poles |  |
| Display function | Intel HD Graphics 500/505 graphics controller, supporti single display, dual display replication, and dual display expansion |  |
|  | 1*VGA | Supports the highest resolution 1920*1200@60Hz |
|  | 1*HDMI | Supports 4K resolution 4096/3840*2160@24/30Hz |
| USB function | 2*USB3.0 | The blue group USB is USB 3.0 speed |
|  | 6*USB2.0 | The black group USB is USB 2.0 speed |
| Sound function | Integrated Realtek ALC662 HD digital audio decoder, 6-channel high fidelity audio controller |  |
|  | 1*SPK | Support audio output |
|  | 1*MIC | Support microphone input |
|  | HDMI interface supports HDMI audio output |  |
| Hard disk function | MSATA disks（8G/16G/32G/64G/120G/128G/240G/256G,etc） |  |
|  | M.2 2242 hard drives（64G/120G/128G/240G/256G/512G,etc） |  |
| Switch function | POWER | Switch button |
| Operating environment | Working temperature: -20 ℃~60 ℃; Working humidity: 5%~95% relative humidity, no condensation |  |
| BIOS | AMI BIOS， Support power on, timed power on, and remote power on |  |
| Watch Dog | Watchdog programming, supports hardware reset function (256 levels, 0-255 seconds) |  |
| Operating system | Windows 10 64 bit, Linux ,etc |  |

☆ The PC supports dual hard drives of MSATA and M.2 2242;

☆ Single Gigabit Ethernet port/Dual Gigabit Ethernet port optional for the PC; Dual network ports only affect the number of USB 2.0 ports;

☆ HDMI supports 4K output;

☆ The operating system only supports Windows 10 64 bit systems and Linux series systems;

☆ The PC is equipped with wall mounted holes;

IPC7000A PC Specification

| PC model | IPC7000A-N3160 series |  |
| --- | --- | --- |
| Motherboard model | M316F |  |
| PC size | 200mm*200mm*41mm |  |
| Power supply | 1*DC_IN | The inner diameter of the DC head is 2.5mm and supports 12V. It is recommended to use a power supply of 12V5A or higher |
| CPU | Intel Celeron N3150 quad core 1.6GHz, Turbo 2.08GHz; Intel Celeron N3160 quad core 1.6GHz, Turbo 2.24GHz; Intel Pentium N3700 quad core 1.6GHz, turbo 2.4GHz; |  |
| Chipset | Single chip integrated into CPU |  |
| Memory | 1*NB-DDR3 memory, supports 1.35V, 1600MHz memory, maximum support 8GB |  |
| Network function | Integrated Realtek 8111H Gigabit Ethernet card, support Wake on LAN and network PXE function |  |
|  | Supports WIFI/3G/4G and 2*WIFI antenna poles |  |
| Display function | Intel Celeron processor N3000 series core graphics card, support single display, dual display replication, and dual display expansion |  |
|  | 1*VGA | Supports the highest resolution 1920*1200@60Hz |
|  | 1*HDMI | Supports 4K resolution 4096/3840*2160@24/30Hz |
| USB function | 2*USB3.0 | The blue group USB is USB 3.0 speed |
|  | 6*USB2.0 | The black group USB is USB 2.0 speed |
| Sound function | Integrated Realtek ALC662 HD digital audio decoder, 6-channel high fidelity audio controller |  |
|  | 1*SPK | Support audio output |
|  | 1*MIC | Support microphone input |
|  | HDMI interface supports HDMI audio output |  |
| Hard disk function | MSATA disks（8G/16G/32G/64G/120G/128G/240G/256G,etc） |  |
| Switch function | POWER | Switch button |
| Operating environment | Working temperature: -20 ℃~60 ℃; Working humidity: 5%~95% relative humidity, no condensation |  |
| BIOS | AMI BIOS,support power on, timed power on, and remote power on |  |
| Watch Dog | Watchdog programming, supports hardware reset function (256 levels, 0-255 seconds) |  |
| Operating system | Windows 10, Windows8.1, Windows7 |  |

☆ he PC only supports MSATA disks;

☆ Single Gigabit Ethernet port/Dual Gigabit Ethernet port optional for the PC; Dual network ports only affect the number of USB 2.0 ports;

☆ HDMI supports 4K output;

☆ The operating system does not support Linux system;

☆ The PC is equipped with wall mounted holes;

IPC7000A PC Specification:

| PC model | IPC7000A-QM9850 |  |
| --- | --- | --- |
| Motherboard model | QM9850 |  |
| PC size | 200mm*200mm*41mm |  |
| Power supply | 1*DC_IN | The inner diameter of the DC head is 2.5mm, supporting 12V/19V. It is recommended to use a power supply of 12V7A or 19V5A or higher |
| 7th gen CPU | Intel Celeron 3865U dual core 1.8GHz; Celeron 3965U dual core 2.2GHz; Intel Core i3-7020U dual core 2.3GHz; Core i3-7100U dual core 2.4GHz; Intel Core i3-7130U dual core 2.7GHz; Intel Core i5-7200U dual core 2.5GHz, turbo 3.1GHz; Core i5-7260U dual core 2.2GHz, turbo 3.4GHz; Intel Core i5-7267U dual core 3.1GHz, turbo 3.5GHz; Core i5-7287U dual core 3.3GHz, turbo 3.7GHz; Intel Core i5-7300U dual core 2.6GHz, turbo 3.5GHz; Core i5-7360U dual core 2.3GHz, turbo 3.6GHz; Intel Core i7-7500U dual core 2.7GHz, turbo 3.5GHz; Core i7-7560U dual core 2.4GHz, turbo 3.8GHz; Intel Core i7-7567U dual core 3.5GHz, turbo 4.0GHz; Core i7-7600U dual core 2.8GHz, turbo 3.9GHz; Intel Core i7-7660U dual core 2.5GHz, turbo 4.0GHz; |  |
| 8th gen CPU | Intel Core i3-8130U dual core 2.2GHz, turbo 3.4GHz; Intel Core i5-8250U dual core 1.6GHz, turbo 3.4GHz; Intel Core i5-8350U dual core 1.7GHz, turbo 3.6GHz; Intel Core i7-8550U dual core 1.8GHz, turbo 4.0GHz; Intel Core i7-8650U dual core 1.9GHz, turbo 4.2GHz; |  |
| Chipset | Single chip integrated into CPU |  |
| Memory | 2*NB-DDR4 memory, supports 2133/2400MHz memory, maximum support 32GB |  |
| Network function | Integrated Realtek 8111H Gigabit Ethernet card, supporting Wake on LAN and network PXE function |  |
|  | Supports WIFI/3G/4G and 2*WIFI antenna poles |  |
| Display function | Integrated Intel HD Graphics 610/620 graphics controller, support single display, dual display replication, and dual display expansion |  |
|  | 1*VGA | Supports the highest resolution 1920*1200@60Hz |
|  | 1*HDMI | Supports 4K resolution 4096/3840*2160@24/30Hz |
| USB function | 2*USB3.0 | The blue group USB is USB 3.0 speed |
|  | 6*USB2.0 | The black group USB is USB 2.0 speed |
| Sound function | Integrated Realtek ALC662 HD digital audio decoder, 6-channel high fidelity audio controller |  |
|  | 1*SPK | Support audio output |
|  | 1*MIC | Support microphone input |
|  | HDMI interface supports HDMI audio output |  |
| Hard disk function | M.2 2280/2242（64G/128/512G/1TB,etc） Optional support MSATA disks（8G/16G/32G/64G/120G/128G/240G/256G,etc） |  |
| Switch function | POWER | Switch button |
| Operating environment | Working temperature: -20 ℃~60 ℃; Working humidity: 5%~95% relative humidity, no condensation |  |
| BIOS | AMI BIOS,support power on, timed power on, and remote power on |  |
| Watch Dog | Watchdog programming, supports hardware reset function (256 levels, 0-255 seconds) |  |
| Operating system | Windows 10, Windows 8, Linux, etc |  |

☆ The PC only supports M.2 disks, optional support for MSATA disks;

☆ Single Gigabit Ethernet port/Dual Gigabit Ethernet port optional for the PC; Dual network ports only affect the number of USB 2.0 ports;

☆ HDMI supports 4K output;

☆ The PC is equipped with wall mounted holes;