ELSKY NANO19 Specification

――――――――――――――――――――――――――――――――――――――――――――――――――――――

PC size

――――――――――――――――――――――――――――――――――――――――――――――――――――――

Single network interface diagram

――――――――――――――――――――――――――――――――――――――――――――――――――――――

――――――――――――――――――――――――――――――――――――――――――――――――――――――

Dual network interface diagram

――――――――――――――――――――――――――――――――――――――――――――――――――――――

――――――――――――――――――――――――――――――――――――――――――――――――――――――

――――――――――――――――――――――――――――――――――――――――――――――――――――――

The pictures are for reference only, please refer to the actual product!

Product model ordering options:

| PC model | CPU | 4K | LAN | COM | USB2.0 | USB3.0 | SSD | Memory | Power |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NANO19 | J1800 J1900 J2900 Optional | / | 1 | 2 | 3 | 1 | 1*MSATA 1*SATA2.0 | 1*NB-DDR3 MAX 8GB | +12V |
|  |  |  | 2 | 2 | 1 | 1 |  |  |  |

Important Tips:

- The PC power supply only supports 12V, and the full load is about 30W. It is recommended to use a 12V4A or above power supply;

- PC VGA, HDMI support single display, dual display copy, dual display expansion, single display under DOS;

- The blue USB port on the PC has USB 3.0 at the bottom and USB 2.0 at the top . The black USB port is USB 2.0.

- The PC MSATA interface defaults to coexisting SATA signals and USB signals, and supports MSATA hard disks or WiFi modules with USB signals by default;

- SATA signal can be changed to PCIE signal, and after the change, it can support PCIE signal WiFi module;

- The PC Mini-PCIE interface defaults to coexisting PCIE signals and USB signals, and supports WiFi/3G/4G/Bluetooth WiFi modules by default;

- All USB ports on the PC are S-type (no power after shutdown) and cannot be changed to A-type (power after shutdown);

- The mainboard COM port supports RS232 by default; COM2 can select RS232/RS422/RS485, which is controlled by JP9/JP10/JP11 jumper caps;

- The ninth pin of the motherboard COM1/2 can be set to 5V/12V through JCOM1/2, the default is 0V;

- The PC supports 100M and 1000M network cards; single and dual network ports are optional; POE power supply cannot be changed;

Product Specifications

| PC model | NANO19 |  |
| --- | --- | --- |
| PC size | 136 mm ( L ) *1 27 mm ( W ) * 47 mm ( H ) |  |
| CPU Processor | Intel® Processor Celeron J1800, Celeron J1900, Pentium J2900 |  |
| Memory support | 1*DDR3 SODIMM memory slot, supports 1600MHz memory, low voltage 1.35V, maximum support 8GB |  |
| Power supply | 1*DC_IN | Supports 12V voltage, the inner diameter of the DC head is 2.5MM The motherboard consumes about 30W when fully loaded. It is recommended to use a 12V4A or higher power supply |
| Display functions | Intel HD Graphics graphics controller , supports single display, dual display copy, dual display expansion, single display under DOS |  |
|  | 1*HDMI 1.4 interface | Support resolution 1920*1080@60HZ |
|  | 1*VGA DB15 interface | Support resolution 1920*1080@60HZ |
| Network Features | 1/2*RJ45 LAN interface | Realtek 8105E 100M and 8111 F 1000M network cards are optional, supporting network wake-up and PXE diskless boot functions |
|  | 1*Mini-PCIE interface | Support WIFI/3G/4G/Bluetooth WiFi module |
| Hard disk function | 1*MSATA interface | Support MSATA disk, support SATA 2.0 |
|  | 1*SATA1 interface | Standard SATA2.0 hard disk interface |
|  | 1*SATA_PWR1 pin | Hard disk power supply pin, can take 5V, 12V power. (4Pin, 1*4Pin, 2.54mm) |
| Sound Features | 1*LIN_OUT interface | Support audio output (green) |
|  | 1*MIC_IN interface | Support microphone input (red) |
| USB Function | 1*USB3.0 interface | Rear standard USB3.0 interface |
|  | 3*USB2.0 interface | Rear standard USB2.0 interface |
| Switch function | 1* POWER button | Switch button |
| Other I/O | 1*SIM_CARD slot | Support SIM card, SIM card is required when using 3G/4G |
| Operating Environment | Working temperature: -20℃~60℃; Working humidity: 5%~95% relative humidity, no condensation |  |
| BIOS | AMI BIOS, supports power-on, scheduled power-on, remote power on and off, and intelligent device identification |  |
| Watch Dog | Watchdog programming, support hardware reset function (256 levels, 0~255 seconds) |  |
| Operating system | Support Windows 10, Windows 8, Windows7, Linux, etc. |  |

CPU Information

| CPU Specifications | CPU Brand | CPU Model | Frequency | Turbo | CPU power consumption (W) |
| --- | --- | --- | --- | --- | --- |
|  | Intel Celeron | J1800 | Dual core 2.41G | 2.58G | 10W |
|  | Intel Celeron | J1900 | Quad-core 1.99G | 2.41G | 10W |
|  | Intel Pentium | J2900 | Quad-core 2.41G | 2.66G | 10W |