ELSKY QM3600 V1.3 Specification

―――――――――――――――――――――――――――――――――――――――――――

6C motherboard picture↓

―――――――――――――――――――――――――――――――――――――――――――

HDMI+VGA+Single LAN port picture↓（default）

―――――――――――――――――――――――――――――――――――――――――――

HDMI+VGA dual LAN port picture↓

―――――――――――――――――――――――――――――――――――――――――――

DP+HDMI dual LAN port picture↓

―――――――――――――――――――――――――――――――――――――――――――

DP interface+115X Schematic picture of double-pipe radiator↓

――――――――――――――――――――――――――――――――――――――――――

2C Without PCIE slot + embedded heat sink picture↓

―――――――――――――――――――――――――――――――――――――――――――

The back sight picture of the Motherboard↓

―――――――――――――――――――――――――――――――――――――――――――

The above picture is for reference only, please refer to the actual object！

Product Model Ordering Options：

| Motherboard module | CPU | 4K | COM | LAN | USB2.0 | USB3.0 | SSD | Memory | Power |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| QM3600 | Support Intel 6th/7th/8th/9th generation LGA1151 Core i3, i5, i7, i9 independent CPU | support | 2/6 | 1 | 6 | 2 | 1*MSATA 1*SATA | 2*NB-DDR4 MAX 64G | +12V/19V |
| QM3600-2L |  |  | 2/6 | 2 | 4 | 2 |  |  |  |
| The motherboard can be equipped with a PCI-E X16 interface, which can be connected to an external graphics card |  |  |  |  |  |  |  |  |  |

Important remind：

- Power supply interface：

- The motherboard DC interface specification is 5.5*2.5, supporting a 4Pin ATX power interface;

- The motherboard power supply supports 12V or 19V; the full load power consumption is about 90W, and it is recommended to use a power supply of 12V/10A or 19V/6A or above;

- CPU:

- The motherboard supports Intel 6th/7th/8th/9th generation LGA1151 Core i3, i5, i7, i9 independent CPU;

- Display interface：

- 1. The motherboard supports HDMI, VGA/DP, LVDS/EDP pins to support single display, dual display copy, dual display expansion, and single display under DOS;

- 2. The motherboard supports VGA and DP, VGA is supported by default, and DP can be changed to support DP;

- 3. The motherboard supports LVDS and EDP pins, and the pins are shared. LVDS is supported by default, and EDP can be changed to support EDP. EDP uses EDP-B 20Pin backlight to separate independent screen lines;

- 4. The motherboard supports 1 PCIE X16 interface. After connecting to an independent graphics card, the motherboard display interface (VGA/DP/HDMI/LVDS/EDP) has no signal output;

- 5. The motherboard HDMI supports 4K resolution 4096/3840*2160@30Hz;

- 6. The motherboard DP supports 4K resolution 4096/3840*2160@60Hz;

- 7. The motherboard LVDS and EDP support resolution 1920*1080@60Hz;

- Mini-PCIE：

- The motherboard MSATA port defaults to coexisting SATA signals and USB signals, and supports MSATA disks by default. Its USB signal and USB sound card signal are selected from one to the other; when the motherboard is a USB sound card, MSATA has no USB signal; SATA signal cannot be changed to PCIE signal;

- The motherboard Mini-PCIE port defaults to coexisting PCIE signals and USB signals, and supports WiFi/4G/Bluetooth modules by default. The USB signal is an independent signal;

- USB interface：

- 1. The motherboard supports 8 USB by default (2*USB3.0, 6*USB2.0);

- 2. All USB interfaces and pins on the motherboard default to S power (no power when turned off), and the built-in USB pins can be changed to A power (powered when turned off);

- COM port：

- The motherboard supports 2/6*COM ports, of which COM1 supports RS232/RS422/RS485. Through BIOS settings, other COM ports only support RS232.

- The motherboard COM1/2 can be set to output 0V/5V/12V on the ninth pin through JCOM1/2/jumper caps, and the default is 0V.

- All serial ports can be changed to support TTL hardware.

- Network Card：

- Single network: supports Realtek 8111H Gigabit network card by default; optional support for domestic Yutaiwei YT6801 Gigabit network card;

- Dual network: supports Realtek 8111H+Realtek 8111H Gigabit network card or domestic Yutaiwei YT6801+Yutaiwei YT6801 Gigabit network card;

- PoE power supply cannot be changed;

- Sound card：

The motherboard defaults to Realtek ALC897 sound card, and the F_AUDIO built-in pin is available. It can be changed to support Cmedia HS_100B USB sound card. After the change, the F_AUDIO built-in pin is unavailable, and there is no USB signal on the MSATA interface after the change

Illustrate

Except for the accessories listed with the product, the content contained in this manual does not represent the company's commitment. The company reserves the right to change this manual without prior notice.

We are not responsible for any direct, indirect, intentional or unintentional damage and hidden dangers caused by improper installation and use.

Before ordering a product, please ask the dealer for details about whether the product performance meets your needs.

The contents of this manual are protected by copyright and may not be reproduced mechanically, electronically or by any other means without permission.

The final interpretation right of this manual belongs to SHENZHEN ELSKY TECHNOLOGY CO.,LTD.

The above ordering information is for reference only. Please contact us for details.

Motherboard driver download https://pan.baidu.com/s/1ie7qi5NutS53ipPecRSYtw

Access password cbn5

BIOS Function key

| BIOS Function key | Function description |
| --- | --- |
| Power on Delete | The BIOS function screen is displayed |
| Power on F11 | Select the boot option or access the BIOS function screen |
| ← → | Move the left and right arrows to select an entry |
| ↑ ↓ | Move the up and down arrows to select the entry |
| Enter | Select the item and go to the sub-menu |
| +/- | Change the selection, or add or subtract values |
| F1 | The help content is displayed |
| F9 | factory data reset |
| F10 | Save the changes and exit |
| Esc | quit |

The following CPU information is for reference only. Please contact sales for specific requirements！

| CPU | CPU brand | CPU module | Dominant Frequency | Turbo | CPU power consumption (W) | CPU full load power consumption (W) |
| --- | --- | --- | --- | --- | --- | --- |
| 6th gen I3 | Intel Core | 6100 | Dual Core 3.7G |  | 65 | 90 |
|  |  | 6300 | Dual Core 3.8G |  | 65 | 90 |
|  |  | 6320 | Dual Core 3.9G |  | 65 | 90 |
| 6th gen I5 | Intel Core | 6400 | Quad Core 2.7G | 3.3G | 65 | 90 |
|  |  | 6500 | Quad Core 3.2G | 3.6G | 65 | 90 |
|  |  | 6600 | Quad Core 3.3G | 3.9G | 65 | 90 |
| 6th gen I7 | Intel Core | 6700 | Quad Core 3.4G | 4.0G | 65 | 90 |
| 7th gen I3 | Intel Core | 7100 | Dual Core 3.9G |  | 65 | 90 |
|  |  | 7300 | Dual Core 4.0G |  | 65 | 90 |
|  |  | 7320 | Dual Core 4.1G |  | 65 | 90 |
| 7th gen I5 | Intel Core | 7400 | Quad Core 3.0G | 3.5G | 65 | 90 |
|  |  | 7500 | Quad Core 3.4G | 3.8G | 65 | 90 |
|  |  | 7600 | Quad Core 3.5G | 4.,1G | 65 | 90 |
| 7th gen I7 | Intel Core | 7700 | Quad Core 3.6G | 4.2G | 65 | 90 |
| 8th gen I3 | Intel Core | 8100 | Quad Core 3.6G |  | 65 | 90 |
|  |  | 8300 | Quad Core 3.7G |  | 65 | 90 |
| 8th gen I5 | Intel Core | 8400 | Six cores 2.8G | 4.0G | 65 | 90 |
|  |  | 8500 | Six cores 3.0G | 4.1G | 65 | 90 |
|  |  | 8600 | Six cores 3.2G | 4.3G | 65 | 90 |
| 8th gen I7 | Intel Core | 8700 | Six cores 3.2G | 4.6G | 65 | 90 |
| 9th gen I3 | Intel Core | 9100 | Quad Core 3.6G | 4.2G | 65 | 90 |
|  |  | 9300 | Quad Core 3.7G | 4.2G | 65 | 90 |
| 9th gen I5 | Intel Core | 9400 | Six cores 2.9G | 4.1G | 65 | 90 |
|  |  | 9500 | Six cores 3.0G | 4.4G | 65 | 90 |
|  |  | 9600 | Six cores 3.1G | 4.6G | 65 | 90 |
| 9th gen I7 | Intel Core | 9700 | Octa-core 3.0G | 4.7G | 65 | 90 |
| 9th gen I9 | Intel Core | 9900 | Octa-core 3.1G | 5.0G | 65 | 90 |

- Production Specification

| Size | 170mm（L）*170mm（W）*22mm（H） (The thickness of the embedded radiator is 30mm, and the thickness of the 115X double-tube radiator is 42mm) |  |
| --- | --- | --- |
| CPU | Support Intel 6th/7th/8th/9th generation LGA1151 Core i3, i5, i7, i9 independent CPU |  |
| Chipset | Intel H310C Chipset |  |
| Power supply | 1*DC_IN | The inner diameter of the DC head is 2.5MM, The supporting 12V/19V motherboard has a full load power consumption of about 90W. It is recommended to use a 12V/10A or 19V/6A power supply or above |
|  | 1*4Pin DC-ATX |  |
| Memory | 2*SODIMM slots | NB-DDR4 memory, supports 2133/2400/2666MHz memory, maximum support 64G |
| Network function | 1/2*LAN interface | Single network: default support Realtek 8111H Gigabit network port; Optional support Yutaiwei YT6801 Gigabit network card; Dual network: support Realtek 8111H + Realtek 8111H Gigabit network card; Optional support Yutaiwei YT6801 + Yutaiwei YT6801 Gigabit network card; Support network wake-up, PXE function |
|  | 1*MINI_PCIE interface | PCIE3.0 signal, support WIFI/4G/Bluetooth module |
| Display function | Graphics | Intel HD Graphics 530/630 super core graphics card, supports single display, dual display copy, dual display expansion, single display under DOS |
|  | 1*HDMI 1.4 interface | Support 4K resolution 4096/3840*2160@30Hz |
|  | 1*HDMI_CON pin | Support resolution 4096/3840*2160@30Hz （16Pin，2*8Pin，2.0mm） |
|  | 1*VGA DB15 interface | Support resolution 1920*1080@60Hz |
|  | 1*VGA_OUT pin | Support resolution 1920*1080@60Hz （12Pin，1*12Pin，2.0mm） |
|  | 1*DP interface (optional) | Support 4K resolution 4096/3840*2160@60Hz |
|  | VGA and DP are two choices, the default signal is VGA, and the signal can be changed to DP |  |
|  | 1*LVDS pin (EDP optional) | Support resolution 1920*1080@60Hz (30Pin, 2*15Pin, 2.0mm) |
|  | LVDS and EDP share the same pin, the default is LVDS signal, can be changed to EDP signal, use EDP-B screen cable |  |
|  | 1*LVDS_PWR pin | LVDS/EDP voltage control pin (6Pin, 2*3Pin, 2.0mm) |
|  | 1*INVERT pin | LVDS/EDP backlight switch control pin (6Pin, 1*6Pin, 2.0mm) |
| USB function | 2*USB3.0 interface | Rear standard USB3.0 interface |
|  | 2*USB2.0 interface | Rear standard USB2.0 interface |
|  | 1*F_USB2 pin | Front USB2.0 pin, one set has 2*USB2.0 (9Pin, 2*5Pin, 2.54mm) |
|  | 1*F_USB3 pin | Front USB2.0 pin, one set has 2*USB2.0 (9Pin, 2*5Pin, 2.54mm) |
| Sound function | Integrated Realtek ALC897 HD digital audio decoder, 6-channel high-fidelity audio controller; Can be changed to support Cmedia HS_100B USB sound card; |  |
|  | 1*LINE_OUT interface | Support audio output (green) |
|  | 1*MIC_IN interface | Support microphone input (red) |
|  | 1*F_AUDIO pin | Standard audio pin, when the motherboard is a USB sound card, the pin is not available (9Pin, 2*5Pin, 2.54mm) |
|  | 1*INT_SPK pin | The amplifier supports 8 ohm 5 watt speaker dual channel output (4Pin, 1*4Pin, 2.0mm) |
| Hard disk function | 1*MSATA pin | Support MINI_SATA disk, support MSATA 3.0 |
|  | 1*SATA1 interface（optional） | Standard SATA hard disk interface, supports SATA 3.0 (no interface by default, can be changed to an independent signal) |
|  | 1*SATA2interface | Standard SATA hard disk interface, supports SATA 3.0 |
|  | 1*SATA_PWR1 pin | Hard disk power supply pin, can take power 5V, 12V (4Pin, 1*4Pin, 2.54mm) |
|  | 1*SATA_PWR2 pin（optional） | Hard disk power supply pin, can take power 5V, 12V (4Pin, 1*4Pin, 2.54mm) |
| Switch function | 1*F_PANEL pin | Switch, power light, hard disk light, restart pin (9Pin, 2*5Pin, 2.54mm) |
| Other I/O | 1*COM1 pin | Serial port, support RS232/RS422/RS485 optional (9Pin, 2*5Pin, 2.54mm) |
|  | 1*COM2 pin | Serial port, only supports RS232 (9Pin, 2*5Pin, 2.54mm) |
|  | COM3~COM6 pin | Serial port, support standard RS232; (39Pin, 2*20Pin, 2.0mm) |
|  | All serial ports can support TTL hardware |  |
|  | 1*JCOM1 pin | Control COM1 pin 9 voltage 0V/5V/12V optional (6Pin, 2*3Pin, 2.0mm) |
|  | 1*JCOM2 pin | Control COM2 pin 9 voltage 0V/5V/12V optional (6Pin, 2*3Pin, 2.0mm) |
|  | 1*SIM_CARD slot | Support SIM card, SIM card is required when using 4G |
|  | 1*FAN1 pin | CPU fan pin, supports temperature control (3Pin, 1*3Pin, 2.54mm) |
|  | 1*FAN2 pin | System fan pin, full speed does not support temperature control (3Pin, 1*3Pin, 2.54mm) |
|  | 1*CLR_CMOS pin | Motherboard reset and discharge pin (3Pin, 1*3Pin, 2.0mm) |
|  | 1*GPIO_CN pin | GPIO control pin (9Pin, 2*5Pin, 2.0mm) |
| Operating environment | Working temperature: -10℃~60℃; Working humidity: 5%~95% relative humidity, no condensation |  |
| BIOS | AMI BIOS, supports power-on, scheduled power-on, remote power on and off, and intelligent device identification |  |
| Watch Dog | Watchdog programming (256 levels, 0~255 seconds) |  |
| Operating system | Support Windows 11，Windows 10，Windows 7，Linux etc |  |

## 1.1 Motherboard size

―――――――――――――――――――――――――――――――――――――――――――

VGA+single LAN↓

―――――――――――――――――――――――――――――――――――――――――――

VGA+Dual LAN↓

―――――――――――――――――――――――――――――――――――――――――――DP+Single LAN↓

―――――――――――――――――――――――――――――――――――――――――――

DP+Dual LAN↓

―――――――――――――――――――――――――――――――――――――――――――

Motherboard pin definition and description

2.0 The first pin identification method

Method 1：Look at the screen mark next to the pin on the front of the motherboard, which will be represented by  or  or  1 ；

Method 2：Look at the back pad of the motherboard, the square pad  is the first pin ；

- Distinguish the first pin when inserting the device and connecting cable; otherwise, the motherboard and device will be damaged.

## 2.1 VGA Pin definition

| item：VGA_OUT （1*12Pin，2.0mm） |  |  |  |
| --- | --- | --- | --- |
| pin | definition | pin | definition |
| 1 | GND | 2 | VSYNC |
| 3 | HSYNC | 4 | GND |
| 5 | RED | 6 | GND |
| 7 | GRN | 8 | GND |
| 9 | BLUE | 10 | GND |
| 11 | DDC_DATA | 12 | DDC_CLK |
| Pin number diagram |  |  |  |

Note: The VGA_H pin signal is the same as the rear VGA port and only supports dual-display copying.

## 2.2 HDMI Pin definition

| item：HDMI_CON （2*8Pin，2.0mm） |  |  |  | Pin number diagram |
| --- | --- | --- | --- | --- |
| pin | definition | pin | definition |  |
| 1 | HDMI_TXD2P | 2 | HDMI_TXD1P |  |
| 3 | HDMI_TXD2N | 4 | HDMI_TXD1N |  |
| 5 | GND | 6 | GND |  |
| 7 | HDMI_TXD0P | 8 | HDMI_TXC0P |  |
| 9 | HDMI_TXD0N | 10 | HDMI_TXC0N |  |
| 11 | GND | 12 | HDMI_5V |  |
| 13 | HDMI_CLK | 14 | HDMI_5V |  |
| 15 | HDMI_DATA | 16 | HDMI_HPD |  |

Note: When plugging in the HDMI cable, the first pin of the cable must correspond to the first pin of the motherboard. If it is plugged in reverse or in the wrong position, there will be no display.

## 2.3 LVDS Pin definition

| item：LVDS （2*15Pin，2.0mm） |  |  |  | Pin number diagram |
| --- | --- | --- | --- | --- |
| pin | definition | pin | definition |  |
| 1 | VCC | 2 | VCC |  |
| 3 | VCC | 4 | GND |  |
| 5 | GND | 6 | GND |  |
| 7 | ADO0- | 8 | ADO0+ |  |
| 9 | ADO1- | 10 | ADO1+ |  |
| 11 | ADO2- | 12 | ADO2+ |  |
| 13 | GND | 14 | GND |  |
| 15 | ACLK- | 16 | ACLK+ |  |
| 17 | ADO3- | 18 | ADO3+ |  |
| 19 | BDO0- | 20 | BDO0+ |  |
| 21 | BDO1- | 22 | BDO1+ |  |
| 23 | BDO2- | 24 | BDO2+ |  |
| 25 | GND | 26 | GND |  |
| 27 | BCLK- | 28 | BCLK+ |  |
| 29 | BDO3- | 30 | BDO3+ |  |

Note: When plugging in the screen cable, the first pin of the screen cable must correspond to the first pin of the motherboard. If it is plugged in reverse or in the wrong position, there is a risk of burning the screen and motherboard!

## 2.3.1 LVDS backlight power supply definition

| item：INVERT （1*6Pin，2.0mm） |  | Pin number diagram |
| --- | --- | --- |
| pin | definition |  |
| 1 | +12V |  |
| 2 | +12V |  |
| 3 | ON/OFF（Backlight switch） |  |
| 4 | ADJ（Backlight brightness adjustment） |  |
| 5 | GND |  |
| 6 | GND |  |

## 2.3.2 LVDS Working voltage

| item：LVDS_PWR （2*3Pin，2.0mm） |  | Pin number diagram |
| --- | --- | --- |
| pin | definition |  |
| 1-2 Short Circuit | +3.3V |  |
| 3-4 Short Circuit | +5V |  |
| 5-6 Short Circuit | +12V |  |

Note: Screens of different sizes require different operating voltages. The motherboard provides three screen operating voltages: 3.3V, 5V, and 12V. Please set the corresponding value of LVDS_PWR according to the required operating voltage of the screen, otherwise there will be a risk of burning the screen and motherboard!

- 2.4 EDP Pin definition

Note: EDP and LVDS share the same pins, use EDP-B type screen cable, and separate the 20Pin screen cable and backlight;

| item：LVDS/EDP（2*15Pin，2.0mm） |  |  |  | Pin number diagram |
| --- | --- | --- | --- | --- |
| pin | definition | pin | definition |  |
| 1 | VCC | 2 | VCC |  |
| 3 | VCC | 4 | HPD |  |
| 5 | GND | 6 | GND |  |
| 7 | TX0N | 8 | TX0P |  |
| 9 | TX1N | 10 | TX1P |  |
| 11 | NC | 12 | NC |  |
| 13 | GND | 14 | GND |  |
| 15 | AUX- | 16 | AUX+ |  |
| 17 | GND- | 18 | GND |  |
| 19 | NC | 20 | NC |  |
| 21Pin~30Pin signal is empty |  |  |  |  |

Note: When plugging in the screen cable, the first pin of the screen cable must correspond to the first pin of the motherboard. If it is plugged in reverse or in the wrong position, there is a risk of burning the screen and the motherboard！

## 2.5 GPIO_CN Pin definition

| item：GPIO_CN （2*5Pin，2.0mm） |  |  |  | Pin number diagram |
| --- | --- | --- | --- | --- |
| pin | definition | pin | definition |  |
| 1 | GPIO_IN0 | 2 | GPIO_OUT0 |  |
| 3 | GPIO_IN1 | 4 | GPIO_OUT1 |  |
| 5 | GPIO_IN2 | 6 | GPIO_OUT2 |  |
| 7 | GPIO_IN3 | 8 | GPIO_OUT3 |  |
| 9 | +3.3V | 10 | GND |  |

GPIO_IN and GPIO_OUT can be freely changed/interchanged as needed.

## 2.6 USB Pin definition

| item：F_USB1/2 （2*5Pin，2.54mm） |  |  |  | Pin number diagram |
| --- | --- | --- | --- | --- |
| pin | definition | pin | definition |  |
| 1 | VCC+5V | 2 | VCC+5V |  |
| 3 | DATA0- | 4 | DATA1- |  |
| 5 | DATA0+ | 6 | DATA1+ |  |
| 7 | GND | 8 | GND |  |
| 9 | NC | 10 | GND |  |

## 2.7 Fan interface definition

| item：FAN1 （1*3Pin，2.54mm） |  |  | item：FAN2 （1*3Pin，2.54mm） |  | Pin number diagram |
| --- | --- | --- | --- | --- | --- |
| pin | definition | pin |  | definition |  |
| 1 | GND | 1 |  | GND |  |
| 2 | CTL（Temperature control） | 2 |  | +12V |  |
| 3 | TAC（Fan speed detection） | 3 |  | TAC（Fan speed detection） |  |

## 2.8 Serial port (COM) function and pin definition

The motherboard supports standard RS232 6COM motherboard, COM1 can optionally support RS485 and RS422, through BIOS settings;

The 9th pin of COM1/2 can be changed through the JCOM1/JCOM2 jumper settings, and the 9th pin can be selected to output +5V or +12V voltage

| item：JCOM1 （2*3Pin，2.0mm） |  |  | item：JCOM2 （2*3Pin，2.0mm） |  | Pin number diagram |  |
| --- | --- | --- | --- | --- | --- | --- |
| pin | COM1 pin 9 is charged |  | pin | COM2 pin 9 is charged |  |  |  |
| 1-2 Short Circuit | +5V |  | 1-2 Short Circuit | +5V |  |  |  |
| 3-4 Short Circuit | +12V |  | 3-4 Short Circuit | +12V |  |  |  |
| 5-6 Short Circuit | uncharged（default） |  | 5-6 Short Circuit | uncharged（default） |  |  |  |

## 2.8.1 COM1/2 Pin definition

| item：COM1/2（2*5Pin，2.54mm） |  |  |  | Pin number diagram |
| --- | --- | --- | --- | --- |
| pin | definition | pin | definition |  |
| 1 | DCD | 2 | RXD |  |
| 3 | TXD | 4 | DTR |  |
| 5 | GND | 6 | DSR |  |
| 7 | RTS | 8 | CTS |  |
| 9 | RI | 10 | NC |  |

## 2.8.2 COM3~6 Pin definition

| item：COM3~6 （2*20Pin，2.0mm） |  |  |  | Pin number diagram |
| --- | --- | --- | --- | --- |
| pin | definition | pin | definition |  |
| 1 | COM3_DCD | 2 | COM3_RXD |  |
| 3 | COM3_TXD | 4 | COM3_DTR |  |
| 5 | GND | 6 | COM3_DSR |  |
| 7 | COM3_RTS | 8 | COM3_CTS |  |
| 9 | COM3-RI | 10 | NC |  |
| 11 | COM4_DCD | 12 | COM4_RXD |  |
| 13 | COM4_TXD | 14 | COM4_DTR |  |
| 15 | GND | 16 | COM4_DSR |  |
| 17 | COM4_RTS | 18 | COM4_CTS |  |
| 19 | COM4-RI | 20 | NC |  |
| 21 | COM5_DCD | 22 | COM5_RXD |  |
| 23 | COM5_TXD | 24 | COM5_DTR |  |
| 25 | GND | 26 | COM5_DSR |  |
| 27 | COM5_RTS | 28 | COM5_CTS |  |
| 29 | COM5-RI | 30 | NC |  |
| 31 | COM6_DCD | 32 | COM6_RXD |  |
| 33 | COM6_TXD | 34 | COM6_DTR |  |
| 35 | GND | 36 | COM6_DSR |  |
| 37 | COM6_RTS | 38 | COM6_CTS |  |
| 39 | COM6-RI | 40 | NC |  |

## 2.8.3 RS422/RS485 definition of COM1

| item：COM1 （2*5Pin，2.54mm） |  |  |
| --- | --- | --- |
| signal | Pin definition |  |
| RS485 | 1（S485-) | 2（S485+） |
| RS422 | 1（TXD-） | 2（TXD+) |
|  | 3（RXD+） | 4（RXD-） |

## 2.9 Audio interface and pin definition

| item：F_AUDIO （2*5Pin，2.54mm） |  |  |  | Pin number diagram |
| --- | --- | --- | --- | --- |
| pin | definition | pin | definition |  |
| 1 | MIC-L | 2 | GND |  |
| 3 | MIC-R | 4 | NC |  |
| 5 | LINE OUT-R | 6 | MIC_JD |  |
| 7 | FAUDIO_JD | 8 | NC |  |
| 9 | LINE OUT-L | 10 | LINE_JD |  |

When the motherboard is a USB sound card, the F_AUDIO pin is unavailable, but it is available when it is a Realtek sound card.

## 2.10 Speaker (amplifier) ​​pin definition

| item：INT_SPK （1*4Pin，2.0mm） |  | Pin number diagram |
| --- | --- | --- |
| pin | definition |  |
| 1 | L+ |  |
| 2 | L- |  |
| 3 | R- |  |
| 4 | R+ |  |

## 2.11  Hard disk interface and definition

1 SATA3.0 hard disk interface, 1 MSATA hard disk interface

| item：SATA2 |  |  | item：SATA_PWR1 （1*4Pin，2.54mm） |  | Pin number diagram |
| --- | --- | --- | --- | --- | --- |
| pin | definition |  | pin | definition |  |
| 1 | GND |  | 1 | 12V |  |
| 2 | SATA_TXP |  | 2 | GND |  |
| 3 | SATA_TXN |  | 3 | GND |  |
| 4 | GND |  | 4 | 5V |  |
| 5 | SATA_RXN |  |  |  |  |
| 6 | SATA_RXP |  |  |  |  |
| 7 | GND |  |  |  |  |

Note: Pin 1 of the SATA_PWR hard disk power supply interface is 12V output, and pin 4 is 5V output. When using, you must use the power cord provided by our company to avoid burning the hard disk.

## 2.12 Power and switch pin definitions

The motherboard provides a 2.5 standard DC head (DC_IN); a 4-pin ATX power interface, defined as：

| item：ATX （2*2Pin，4.2mm） |  | Pin number diagram |
| --- | --- | --- |
| pin | definition |  |
| 1 | GND |  |
| 2 | GND |  |
| 3 | +12V |  |
| 4 | +12V |  |

Switch pin definition：

| item：F_PANEL （2*5Pin，2.54mm） |  |  |  |  |  | Pin number diagram |
| --- | --- | --- | --- | --- | --- | --- |
| pin | definition |  | pin | definition |  |  |
| 1 | HDLED+ | Hard disk light | 2 | PWRLED+ | Power supply light |  |  |
| 3 | HDLED- |  | 4 | GND |  |  |  |
| 5 | RST | restart | 6 | P_SW IN | switch |  |  |
| 7 | GND |  | 8 | GND |  |  |  |
| 9 | NC |  | 10 |  |  |  |  |

(1) Hard disk indicator light (pins 1 and 3, HDD LED, pin 1 is the positive pole of the LED) When the hard disk is performing read and write operations, the indicator light will flash, indicating that the hard disk is running;

(2) Power indicator light (pins 2 and 4, Power LED, pin 2 is the positive pole of the LED) When the motherboard is powered on, the power indicator light is on; when the motherboard is powered off, the power indicator light is off;

(3) Reset button (pins 5 and 7, Reset Button) When the system fails and cannot continue to work, resetting can make the system start working again;

(4) Power switch control (pins 6 and 8, Power Button) These two pins are connected to the bounce switch on the front panel of the chassis, which can be used to turn on or off the computer

## 2.13 Motherboard discharge reset and battery

CMOS is powered by the button battery on the motherboard. Clearing CMOS will clear the previous BIOS settings and set them to the original factory settings.

The steps are as follows:

(1) Turn off the computer and disconnect the power supply;

(2) Jump the "CLR_CMOS" pin jumper to short the 2-3 pins for 5-6 seconds, and then jump back to the 1-2 pins;

(3) Press the "Delete" key on the keyboard to enter the BIOS interface;

(4) Enter the BIOS interface and press the "F9" key - "Enter" key to reload the optimal default value;

(5) Press F10 to save and exit the settings.

CMOS pin definition:

| item：CLR_CMOS （1*3Pin，2.0mm） |  |  |
| --- | --- | --- |
| pin | definition |  |
| 1-2 Short Circuit | Default normal boot |  |
| 2-3 Short Circuit | Clear CMOS content and restore BIOS to factory settings |  |

Note: Do not clear CMOS when the computer is powered on to avoid damaging the motherboard.

After power failure, re-insert the button battery to reset the motherboard. Button battery specification: 3V CR2032

Note: Please make sure that the positive pole of the battery is facing upwards; Please make sure that the battery voltage is sufficient to be 2.8V~3V; When replacing batteries, please use batteries of the same model or type and recommended by the manufacturer. If the battery is replaced improperly, there is a risk of explosion!